NetPilot Lab 10 — Unified Network Operations Center

The lab continues directly from Lab 9 and turns the WAN-routing environment into a persistent, AI-assisted NOC workflow.

1. Lab Overview

Lab Name

Lab 10 — Unified Network Operations Center

Difficulty

Advanced / Cisco FDE

Primary Objective

Build a unified NetPilot operations workflow that can:

Cisco Network

↓

Live State Collection

↓

Health Monitoring

↓

Incident Detection

↓

Incident Correlation

↓

PostgreSQL Persistence

↓

Ollama AI RCA

↓

Deterministic Remediation Plan

↓

Engineer Approval

↓

Pre-Change Validation

↓

Cisco Change

↓

Post-Change Validation

↓

Incident Resolution

↓

Audit Trail

The important architectural principle is that the LLM reasons over verified evidence, but application code controls network changes. This separation between model reasoning, application execution, validation, logging, and human approval is a core pattern for safe network agents.

2. What You Are Building

Imagine a Cisco customer has a WAN environment with redundant Internet connectivity.

The customer wants:

- centralized visibility

- automatic incident detection

- incident correlation

- persistent incident history

- AI-assisted root-cause analysis

- controlled remediation

- engineer approval

- pre-change validation

- post-change validation

- complete audit history

NetPilot becomes the operational layer connecting these capabilities.

This is no longer simply:

"Write a Python script that checks a router."

It is becoming:

"Design and implement an operational system that can safely assist a customer network engineering team."

That distinction is important for your FDE development.

3. Customer Scenario

Customer

A fictional enterprise operates:

- headquarters

- redundant WAN edges

- two Internet providers

- internal routing

- centralized network operations

The customer experiences intermittent WAN outages.

Their current process is:

Monitoring Alert

↓

NOC Engineer

↓

SSH to Router

↓

Check Interface

↓

Check BGP

↓

Check Routing Table

↓

Determine Impact

↓

Create Ticket

↓

Recommend Fix

↓

Engineer Changes Router

↓

Verify

The problem is that the same investigation is repeated manually.

The customer asks:

Can we automate the repetitive investigation while keeping the engineer in control of changes?

Your answer is NetPilot.

4. Customer Requirements

NetPilot must:

Requirement 1 — Visibility

Provide health information for all five Cisco routers.

Requirement 2 — WAN Awareness

Understand:

- primary WAN

- backup WAN

- BGP state

- default route

- active forwarding path

Requirement 3 — Incident Detection

Detect a primary WAN failure.

Requirement 4 — Correlation

Do not create three unrelated incidents for:

- interface failure

- BGP failure

- traffic moving to backup

Instead, correlate those symptoms into:

PRIMARY_WAN_FAILURE

Requirement 5 — Persistence

Incidents must survive application restarts.

Requirement 6 — AI Assistance

Ollama should analyze verified evidence and provide:

- root cause

- confidence

- risk

- evidence

- symptoms

- customer impact

- recommended action

- validation guidance

Requirement 7 — Human Control

AI must not directly execute configuration.

Requirement 8 — Deterministic Remediation

The application must construct remediation from verified network state.

Requirement 9 — Validation

Validate:

Before change

↓

Change

↓

After change

Requirement 10 — Auditability

Record:

- incident

- evidence

- AI analysis

- approval

- remediation

- validation

- audit events

5. Discovery Questions

Before touching the network, approach this like an FDE.

Ask the customer:

Business

- What is the business impact when the primary WAN fails?

- Is Internet connectivity business critical?

- What applications depend on the WAN?

- Is downtime acceptable?

- What is the expected recovery time?

Technical

- Which router is the primary WAN edge?

- Which router provides backup connectivity?

- What routing protocol controls the WAN?

- How is the preferred path selected?

- How is failover detected?

- What evidence should be collected before making a change?

Change Management

- Can automation make changes automatically?

- Does an engineer need to approve changes?

- What commands are considered safe?

- What happens if the live network changes after the diagnosis?

AI

- What should AI be allowed to do?

- What should AI never be allowed to do?

- What evidence must AI receive?

- How do we prevent hallucinated configuration?

- How do we audit AI recommendations?

These questions force you to think beyond configuration and toward the customer's actual operational problem.

6. EVE-NG Architecture

Use the same five-router topology from Lab 9.

ISP-1

AS 65001

R4

|

10.0.14.0/30

|

R1

HQ-EDGE-1

AS 65000

/       \

/         \

10.0.12.0/30    10.0.13.0/30

/             \

/               \

R2----------------R3

HQ-CORE          HQ-EDGE-2

AS 65000          AS 65000

|

10.0.35.0/30

|

R5

ISP-2

AS 65002

7. EVE-NG Resources

Use:

| Device | Image | vCPU | RAM |
| --- | --- | --- | --- |
| R1 | C8000V | 2 | 6 GB |
| R2 | C8000V | 2 | 6 GB |
| R3 | C8000V | 2 | 6 GB |
| R4 | C8000V | 2 | 6 GB |
| R5 | C8000V | 2 | 6 GB |

Total:

30 GB RAM

This fits comfortably within your available EVE-NG resources while leaving room for NetPilot/Ollama.

8. Management Network

| Router | Management IP |
| --- | --- |
| R1 | 192.168.1.201 |
| R2 | 192.168.1.202 |
| R3 | 192.168.1.203 |
| R4 | 192.168.1.204 |
| R5 | 192.168.1.205 |

NetPilot/Ollama/PostgreSQL:

192.168.1.217

9. Data-Plane Addressing

R1

Loopback0       1.1.1.1/32

Gi1             10.0.14.1/30

Gi2             10.0.12.1/30

Gi3             10.0.13.1/30

Gi4             192.168.1.201/24

R2

Loopback0       2.2.2.2/32

Loopback100     10.10.100.1/24

Gi1             10.0.12.2/30

Gi2             10.0.23.1/30

Gi3             192.168.1.202/24

R3

Loopback0       3.3.3.3/32

Gi1             10.0.23.2/30

Gi2             10.0.13.2/30

Gi3             10.0.35.1/30

Gi4             192.168.1.203/24

R4

Loopback0       4.4.4.4/32

Loopback100     198.51.100.1/24

Gi1             10.0.14.2/30

Gi2             192.168.1.204/24

R5

Loopback0       5.5.5.5/32

Loopback100     198.51.100.2/24

Gi1             10.0.35.2/30

Gi2             192.168.1.205/24

10. Routing Architecture

OSPF provides the internal routing.

BGP provides the WAN/Internet routing relationship.

Autonomous Systems

Customer: AS 65000

ISP-1:    AS 65001

ISP-2:    AS 65002

BGP

R1 ↔ R4

eBGP

65000 ↔ 65001

R3 ↔ R5

eBGP

65000 ↔ 65002

R1 and R3 also form iBGP:

R1 Lo0 1.1.1.1

↕

R3 Lo0 3.3.3.3

with:

update-source Loopback0

next-hop-self

11. WAN Preference

Primary path:

ISP-1

Backup:

ISP-2

Local preference:

ISP-1 = 200

ISP-2 = 100

Therefore:

200 > 100

so the ISP-1 path is preferred.

12. Default Routes

R1 prefers:

10.0.14.2

with OSPF metric:

10

R3 uses:

10.0.35.2

with OSPF metric:

50

The customer network therefore has redundant forwarding paths.

13. Baseline Validation

Before starting NetPilot, verify the network.

On R1:

show ip interface brief

show ip ospf neighbor

show ip bgp summary

show ip route

show ip route 0.0.0.0

On R2:

show ip interface brief

show ip ospf neighbor

show ip route

On R3:

show ip interface brief

show ip ospf neighbor

show ip bgp summary

show ip route

show ip route 0.0.0.0

On R4/R5:

show ip interface brief

show ip bgp summary

You want the baseline to be healthy.

14. NetPilot Project Structure

Your project should now conceptually look like:

NetPilot/

│

├── app/

│   ├── cisco_client.py

│   ├── network_state.py

│   ├── monitoring.py

│   ├── bgp_health.py

│   ├── database.py

│   ├── incident_store.py

│   ├── incident_manager.py

│   ├── ai_incident.py

│   ├── ai_incident_persistence.py

│   ├── incident_orchestrator.py

│   ├── incident_approval.py

│   ├── remediation_planner.py

│   ├── incident_remediation.py

│   └── dashboard.py

│

├── database/

│   └── schema.sql

│

├── agent/

├── telemetry/

├── remediation/

├── api/

├── tests/

└── evidence/

15. Final Script — app/cisco_client.py

This is the final validated Cisco connectivity layer.

from netmiko import ConnectHandler

import time

DEVICES = {

"R1": {"device_type": "cisco_ios", "host": "192.168.1.201", "username": "netpilot", "password": "Cisco123!", "port": 22},

"R2": {"device_type": "cisco_ios", "host": "192.168.1.202", "username": "netpilot", "password": "Cisco123!", "port": 22},

"R3": {"device_type": "cisco_ios", "host": "192.168.1.203", "username": "netpilot", "password": "Cisco123!", "port": 22},

"R4": {"device_type": "cisco_ios", "host": "192.168.1.204", "username": "netpilot", "password": "Cisco123!", "port": 22},

"R5": {"device_type": "cisco_ios", "host": "192.168.1.205", "username": "netpilot", "password": "Cisco123!", "port": 22},

}

def connect_to_router(device_name):

if device_name not in DEVICES:

print("Unknown device: " + device_name)

return None

device = DEVICES[device_name]

print("Connecting to " + device_name + "...")

for attempt in range(1, 4):

try:

connection = ConnectHandler(**device)

print(device_name + " connected!")

return connection

except Exception as error:

print(

device_name

+ " connection attempt "

+ str(attempt)

+ " failed."

)

if attempt < 3:

time.sleep(2)

else:

print(device_name + " could not be reached.")

print(error)

return None

def run_command(device_name, command):

connection = connect_to_router(device_name)

if connection is None:

return None

try:

output = connection.send_command(

command,

read_timeout=30

)

return output

except Exception as error:

print(

"Command failed on "

+ device_name

+ ": "

+ str(error)

)

return None

finally:

connection.disconnect()

def run_commands(device_name, commands):

connection = connect_to_router(device_name)

if connection is None:

return None

results = {}

try:

for command in commands:

print("Running " + command + "...")

try:

results[command] = connection.send_command(

command,

read_timeout=30

)

except Exception as error:

results[command] = (

"COMMAND_FAILED: "

+ str(error)

)

print("Command failed: " + command)

return results

finally:

connection.disconnect()

def run_config(device_name, commands):

connection = connect_to_router(device_name)

if connection is None:

return None

try:

output = connection.send_config_set(commands)

return output

except Exception as error:

print(

"Configuration failed on "

+ device_name

+ ": "

+ str(error)

)

return None

finally:

connection.disconnect()

Security note: In your actual repository, keep credentials out of source code. The version above reflects the validated lab implementation; for a productionized NetPilot, move credentials to a proper secret-management mechanism.

16. Final Script — app/bgp_health.py

from app.cisco_client import run_command

BGP_DEVICES = {

"R1": {

"neighbors": [

"10.0.14.2",

"3.3.3.3"

]

},

"R3": {

"neighbors": [

"10.0.35.2",

"1.1.1.1"

]

},

}

def get_bgp_summary(router):

if router not in BGP_DEVICES:

return {

"router": router,

"status": "UNSUPPORTED",

"output": "",

"neighbors": []

}

output = run_command(

router,

"show ip bgp summary"

)

if output is None:

return {

"router": router,

"status": "OFFLINE",

"output": "",

"neighbors": []

}

neighbors = []

for neighbor in BGP_DEVICES[router]["neighbors"]:

if neighbor in output:

neighbors.append({

"neighbor": neighbor,

"status": "PRESENT"

})

else:

neighbors.append({

"neighbor": neighbor,

"status": "NOT_FOUND"

})

return {

"router": router,

"status": "ONLINE",

"output": output,

"neighbors": neighbors

}

def display_bgp_health(router):

result = get_bgp_summary(router)

print()

print("========================================")

print("NETPILOT BGP HEALTH")

print("========================================")

print()

print("Router: " + result["router"])

print("Status: " + result["status"])

print()

for neighbor in result["neighbors"]:

print(

"Neighbor: "

+ neighbor["neighbor"]

+ " | "

+ neighbor["status"]

)

print()

if __name__ == "__main__":

display_bgp_health("R1")

display_bgp_health("R3")

17. PostgreSQL Architecture

PostgreSQL runs on:

192.168.1.217

Database:

netpilot

Application user:

netpilot_app

Port:

5432

Do not enable remote PostgreSQL access for the postgres superuser.

The application connects using the restricted application account.

18. Environment Variable

Your Python application uses:

NETPILOT_DB_HOST

NETPILOT_DB_PORT

NETPILOT_DB_NAME

NETPILOT_DB_USER

NETPILOT_DB_PASSWORD

The password should remain outside the codebase.

19. Final Script — app/database.py

import os

import psycopg

DATABASE_HOST = os.getenv(

"NETPILOT_DB_HOST",

"192.168.1.217"

)

DATABASE_PORT = os.getenv(

"NETPILOT_DB_PORT",

"5432"

)

DATABASE_NAME = os.getenv(

"NETPILOT_DB_NAME",

"netpilot"

)

DATABASE_USER = os.getenv(

"NETPILOT_DB_USER",

"netpilot_app"

)

DATABASE_PASSWORD = os.getenv(

"NETPILOT_DB_PASSWORD",

""

)

def get_connection():

return psycopg.connect(

host=DATABASE_HOST,

port=DATABASE_PORT,

dbname=DATABASE_NAME,

user=DATABASE_USER,

password=DATABASE_PASSWORD

)

def test_database_connection():

connection = None

try:

connection = get_connection()

with connection.cursor() as cursor:

cursor.execute(

"""

SELECT

current_database(),

current_user,

version();

"""

)

result = cursor.fetchone()

return {

"status": "CONNECTED",

"database": result[0],

"user": result[1],

"version": result[2]

}

except Exception as error:

return {

"status": "FAILED",

"error": str(error)

}

finally:

if connection is not None:

connection.close()

def display_database_status(result):

print()

print("========================================")

print("NETPILOT DATABASE")

print("========================================")

print()

print(

"Status: "

+ result.get("status", "UNKNOWN")

)

if result.get("status") == "CONNECTED":

print(

"Database: "

+ result.get("database", "UNKNOWN")

)

print(

"User: "

+ result.get("user", "UNKNOWN")

)

print(

"PostgreSQL: "

+ result.get("version", "UNKNOWN")

)

else:

print(

"Error: "

+ result.get("error", "UNKNOWN")

)

print()

if __name__ == "__main__":

result = test_database_connection()

display_database_status(result)

20. Test PostgreSQL

Run:

python -m app.database

Expected:

========================================

NETPILOT DATABASE

========================================

Status: CONNECTED

Database: netpilot

User: netpilot_app

PostgreSQL: PostgreSQL ...

21. Final Database Schema

Your authoritative schema is:

CREATE TABLE IF NOT EXISTS incidents (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id VARCHAR(32) NOT NULL UNIQUE,

router VARCHAR(32) NOT NULL,

incident_type VARCHAR(64) NOT NULL,

status VARCHAR(32) NOT NULL,

description TEXT,

impact TEXT,

health VARCHAR(32),

active_path VARCHAR(32),

default_next_hop VARCHAR(64),

created_at TIMESTAMPTZ NOT NULL DEFAULT CURRENT_TIMESTAMP,

updated_at TIMESTAMPTZ NOT NULL DEFAULT CURRENT_TIMESTAMP,

resolved_at TIMESTAMPTZ

);

CREATE TABLE IF NOT EXISTS incident_evidence (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT NOT NULL,

evidence_type VARCHAR(64) NOT NULL,

evidence JSONB NOT NULL,

collected_at TIMESTAMPTZ NOT NULL DEFAULT CURRENT_TIMESTAMP,

CONSTRAINT fk_incident_evidence_incident

FOREIGN KEY (incident_id)

REFERENCES incidents(id)

ON DELETE CASCADE

);

CREATE TABLE IF NOT EXISTS ai_analysis (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT NOT NULL,

status VARCHAR(32) NOT NULL,

root_cause TEXT,

confidence VARCHAR(32),

risk VARCHAR(32),

analysis TEXT,

created_at TIMESTAMPTZ NOT NULL DEFAULT CURRENT_TIMESTAMP,

CONSTRAINT fk_ai_analysis_incident

FOREIGN KEY (incident_id)

REFERENCES incidents(id)

ON DELETE CASCADE

);

CREATE TABLE IF NOT EXISTS approvals (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT NOT NULL,

status VARCHAR(32) NOT NULL,

engineer VARCHAR(128),

decision VARCHAR(32),

notes TEXT,

approved_at TIMESTAMPTZ,

CONSTRAINT fk_approval_incident

FOREIGN KEY (incident_id)

REFERENCES incidents(id)

ON DELETE CASCADE

);

CREATE TABLE IF NOT EXISTS remediation (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT NOT NULL,

status VARCHAR(32) NOT NULL,

plan JSONB,

executed_at TIMESTAMPTZ

);

CREATE TABLE IF NOT EXISTS validation (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT NOT NULL,

status VARCHAR(32) NOT NULL,

result JSONB,

validated_at TIMESTAMPTZ

);

CREATE TABLE IF NOT EXISTS audit_events (

id BIGINT GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

incident_id BIGINT,

event_type VARCHAR(64) NOT NULL,

actor VARCHAR(128),

details JSONB,

created_at TIMESTAMPTZ NOT NULL DEFAULT CURRENT_TIMESTAMP,

CONSTRAINT fk_audit_incident

FOREIGN KEY (incident_id)

REFERENCES incidents(id)

ON DELETE SET NULL

);

CREATE INDEX IF NOT EXISTS idx_incidents_router

ON incidents(router);

CREATE INDEX IF NOT EXISTS idx_incidents_status

ON incidents(status);

CREATE INDEX IF NOT EXISTS idx_incidents_created_at

ON incidents(created_at);

CREATE INDEX IF NOT EXISTS idx_evidence_incident

ON incident_evidence(incident_id);

CREATE INDEX IF NOT EXISTS idx_ai_analysis_incident

ON ai_analysis(incident_id);

CREATE INDEX IF NOT EXISTS idx_approval_incident

ON approvals(incident_id);

CREATE INDEX IF NOT EXISTS idx_remediation_incident

ON remediation(incident_id);

CREATE INDEX IF NOT EXISTS idx_validation_incident

ON validation(incident_id);

CREATE INDEX IF NOT EXISTS idx_audit_incident

ON audit_events(incident_id);

22. Initialize the Database

Execute the schema against:

netpilot

Then verify the tables exist.

You should have:

incidents

incident_evidence

ai_analysis

approvals

remediation

validation

audit_events

23. Network State Collection

NetPilot now needs to understand the network rather than simply execute commands.

The state collector gathers:

- interfaces

- OSPF neighbors

- routes

- default route

- BGP

- WAN path

For WAN edges:

R1 primary = Gi1 → 10.0.14.2

R1 backup  = Gi3 → 10.0.13.2

R3 primary = Gi3 → 10.0.35.2

R3 backup  = Gi2 → 10.0.13.1

An important design decision here is distinguishing:

BGP peer

from:

actual forwarding next hop

Those are not necessarily the same thing.

24. Network Health Model

NetPilot classifies WAN state as:

HEALTHY

Primary path is working.

DEGRADED

Primary path failed but backup is carrying traffic.

CRITICAL

Neither primary nor backup is usable.

This is much more useful than simply saying:

Interface = down

because the customer cares about service impact, not merely interface state.

25. Monitoring

The monitoring layer performs:

Collect

↓

Normalize

↓

Evaluate

↓

Correlate

↓

Report

For R1 with a failed primary WAN, the expected correlated symptoms are:

Primary WAN interface is down.

Primary WAN BGP session is not established.

Traffic is using the backup WAN path.

Backup WAN path is carrying traffic.

The system therefore identifies:

PRIMARY_WAN_FAILURE

instead of four unrelated problems.

26. Why Correlation Matters

Suppose monitoring produces:

Alert 1:

R1 Gi1 down

Alert 2:

R1 BGP peer down

Alert 3:

Default route changed

Alert 4:

Backup WAN active

A poor monitoring system creates:

4 incidents

A better system recognizes:

1 underlying incident

That is the beginning of actual network operations intelligence.

27. Run Baseline Monitoring

Run:

python -m app.incident_manager

Healthy state should show:

Active incidents: 0

This is important.

You have now demonstrated:

Healthy Network

↓

No Incident

28. Create the Controlled Failure

Do not randomly break the topology.

We want a repeatable customer scenario.

On R1:

configure terminal

interface GigabitEthernet1

shutdown

end

This simulates:

Primary WAN connectivity failure.

29. Verify the Failure Manually

On R1:

show ip interface brief

Gi1 should be down.

Then:

show ip bgp summary

The primary BGP relationship should no longer be established.

Then:

show ip route 0.0.0.0

Confirm traffic is using the backup path.

On R2:

show ip route 0.0.0.0

The internal network should select the R3 path after convergence.

30. Run NetPilot Incident Detection

Run:

python -m app.incident_manager

Expected result:

Active incidents: 1

with:

Incident Type:

PRIMARY_WAN_FAILURE

Router:

R1

Status:

ACTIVE

The incident should contain correlated symptoms.

31. Verify Incident Deduplication

Run:

python -m app.incident_manager

again.

You should not receive another incident.

The existing incident should remain active.

This proves:

same condition

↓

same incident

rather than:

same condition

↓

new incident every polling cycle

That is an important NOC capability.

32. PostgreSQL Persistence

The incident should now exist in PostgreSQL.

The database becomes the:

system of record

instead of:

ACTIVE_INCIDENTS = {}

This means restarting NetPilot does not erase the incident.

33. Evidence Persistence

Evidence is stored separately from the incident.

For example:

Incident

↓

Interface evidence

BGP evidence

Routing evidence

WAN evidence

This allows NetPilot to answer:

Why did you believe this incident existed?

That question becomes extremely important when AI is involved.

34. AI RCA Architecture

The AI workflow is:

Incident

↓

Verified Evidence

↓

Prompt

↓

Ollama

↓

Structured JSON

↓

Validation

↓

PostgreSQL

The AI is not given permission to directly change Cisco devices.

This follows the evidence-first principle: model output should match the actual tool evidence rather than inventing unsupported conclusions.

35. Final Script — app/ai_incident_persistence.py

The validated persistent AI analysis implementation is:

import json

import requests

from app.incident_store import (

get_active_incidents,

get_incident_evidence,

get_latest_ai_analysis,

store_ai_analysis,

store_audit_event,

)

OLLAMA_URL = "http://192.168.1.217:11434/api/generate"

OLLAMA_MODEL = "qwen2.5:32b"

OLLAMA_TIMEOUT = 600

def build_ai_prompt(incident, evidence):

evidence_text = json.dumps(

evidence,

indent=2,

default=str,

)

return f"""

You are NetPilot, a senior Cisco network operations assistant.

Your job is to analyze a network incident using ONLY the verified

evidence supplied below.

Do not invent facts.

Do not assume configuration that is not present in the evidence.

Do not generate Cisco configuration commands.

Do not execute changes.

Do not claim that a change was performed.

The application, not the AI, is responsible for determining whether

a configuration change is safe and whether an engineer approves it.

INCIDENT

Incident ID:

{incident["incident_id"]}

Router:

{incident["router"]}

Incident Type:

{incident["incident_type"]}

Status:

{incident["status"]}

Description:

{incident["description"]}

Impact:

{incident["impact"]}

Health:

{incident.get("health")}

Active Path:

{incident.get("active_path")}

Default Next Hop:

{incident.get("default_next_hop")}

VERIFIED EVIDENCE

{evidence_text}

Return ONLY valid JSON using exactly this structure:

{{

"root_cause": "short technical root cause",

"confidence": "HIGH|MEDIUM|LOW",

"evidence": [

"specific evidence item",

"specific evidence item"

],

"symptoms": [

"observed symptom",

"observed symptom"

],

"customer_impact": "concise customer impact",

"recommended_action": "high-level recommended engineering action",

"risk": "LOW|MEDIUM|HIGH",

"validation": "how an engineer should validate the recommended action",

"assumptions": [

"assumption if any"

]

}}

Rules:

1. Every factual conclusion must be supported by the supplied evidence.

2. If evidence is insufficient, say so.

3. Never fabricate an interface, IP address, BGP neighbor, route,

configuration command, or topology detail.

4. Do not provide configuration commands.

5. Do not say that remediation has occurred.

6. Keep the response concise and operational.

"""

def call_ollama(prompt):

payload = {

"model": OLLAMA_MODEL,

"prompt": prompt,

"stream": False,

"options": {

"temperature": 0.1,

"num_predict": 1200,

},

}

response = requests.post(

OLLAMA_URL,

json=payload,

timeout=OLLAMA_TIMEOUT,

)

response.raise_for_status()

data = response.json()

return data.get(

"response",

""

).strip()

def parse_ai_response(raw_response):

if not raw_response:

raise ValueError(

"Ollama returned an empty response."

)

cleaned = raw_response.strip()

if cleaned.startswith("```"):

cleaned = cleaned.replace(

"```json",

"",

1

)

cleaned = cleaned.replace(

"```",

""

)

cleaned = cleaned.strip()

try:

result = json.loads(cleaned)

except json.JSONDecodeError as error:

raise ValueError(

"Ollama did not return valid JSON: "

+ str(error)

)

required_fields = [

"root_cause",

"confidence",

"evidence",

"symptoms",

"customer_impact",

"recommended_action",

"risk",

"validation",

"assumptions",

]

for field in required_fields:

if field not in result:

raise ValueError(

"Ollama response is missing field: "

+ field

)

if result["confidence"] not in [

"HIGH",

"MEDIUM",

"LOW",

]:

raise ValueError(

"Invalid confidence value."

)

if result["risk"] not in [

"LOW",

"MEDIUM",

"HIGH",

]:

raise ValueError(

"Invalid risk value."

)

if not isinstance(

result["evidence"],

list

):

raise ValueError(

"AI evidence must be a list."

)

if not isinstance(

result["symptoms"],

list

):

raise ValueError(

"AI symptoms must be a list."

)

if not isinstance(

result["assumptions"],

list

):

raise ValueError(

"AI assumptions must be a list."

)

return result

def analyze_incident(incident):

existing_analysis = get_latest_ai_analysis(

incident["database_id"]

)

if existing_analysis is not None:

return {

"status": "EXISTS",

"incident_id": incident["incident_id"],

"analysis_id": existing_analysis["id"],

"model": existing_analysis.get("model"),

"analysis": existing_analysis,

}

evidence = get_incident_evidence(

incident["database_id"]

)

if not evidence:

return {

"status": "FAILED",

"incident_id": incident["incident_id"],

"error": "No stored evidence exists for this incident.",

}

prompt = build_ai_prompt(

incident,

evidence,

)

try:

raw_response = call_ollama(prompt)

analysis = parse_ai_response(raw_response)

analysis_id = store_ai_analysis(

incident_database_id=incident["database_id"],

model=OLLAMA_MODEL,

root_cause=analysis["root_cause"],

confidence=analysis["confidence"],

evidence=analysis["evidence"],

symptoms=analysis["symptoms"],

customer_impact=analysis["customer_impact"],

recommended_action=analysis["recommended_action"],

risk=analysis["risk"],

validation=analysis["validation"],

assumptions=analysis["assumptions"],

raw_response=raw_response,

)

store_audit_event(

incident["database_id"],

"AI_ANALYSIS_CREATED",

"netpilot-ai",

{

"analysis_id": analysis_id,

"model": OLLAMA_MODEL,

"incident_id": incident["incident_id"],

},

)

return {

"status": "SUCCESS",

"incident_id": incident["incident_id"],

"analysis_id": analysis_id,

"model": OLLAMA_MODEL,

"analysis": analysis,

}

except Exception as error:

store_audit_event(

incident["database_id"],

"AI_ANALYSIS_FAILED",

"netpilot-ai",

{

"incident_id": incident["incident_id"],

"error": str(error),

},

)

return {

"status": "FAILED",

"incident_id": incident["incident_id"],

"error": str(error),

}

def analyze_active_incidents():

incidents = get_active_incidents()

results = []

for incident in incidents:

print()

print("Analyzing incident:")

print(

"  Incident ID: "

+ incident["incident_id"]

)

print(

"  Router: "

+ incident["router"]

)

print(

"  Type: "

+ incident["incident_type"]

)

result = analyze_incident(incident)

if result["status"] == "EXISTS":

print("  AI analysis already exists.")

print(

"  Analysis ID: "

+ str(result["analysis_id"])

)

elif result["status"] == "SUCCESS":

print("  New AI analysis created.")

print(

"  Analysis ID: "

+ str(result["analysis_id"])

)

else:

print("  AI analysis failed.")

print(

"  Error: "

+ str(

result.get(

"error",

"UNKNOWN"

)

)

)

results.append(result)

return results

def analyze_persisted_incident(incident):

if not incident:

return {

"status": "NO_INCIDENT",

"incident": None,

"analysis": None,

}

result = analyze_incident(incident)

return {

"status": result.get(

"status",

"UNKNOWN"

),

"incident": incident,

"analysis": result.get("analysis"),

"analysis_id": result.get("analysis_id"),

"model": result.get("model"),

"error": result.get("error"),

}

def display_results(results):

print()

print("========================================")

print("NETPILOT AI INCIDENT ANALYSIS")

print("========================================")

print()

if not results:

print(

"No active incidents require AI analysis."

)

print()

return

for result in results:

print(

"Incident ID: "

+ result.get(

"incident_id",

"UNKNOWN"

)

)

print(

"Status: "

+ result.get(

"status",

"UNKNOWN"

)

)

if result.get("status") in [

"SUCCESS",

"EXISTS",

]:

analysis = result.get(

"analysis",

{}

)

print(

"Analysis ID: "

+ str(

result.get(

"analysis_id",

"UNKNOWN"

)

)

)

print(

"Root Cause: "

+ str(

analysis.get(

"root_cause",

"UNKNOWN"

)

)

)

print(

"Confidence: "

+ str(

analysis.get(

"confidence",

"UNKNOWN"

)

)

)

print(

"Risk: "

+ str(

analysis.get(

"risk",

"UNKNOWN"

)

)

)

print(

"Recommended Action: "

+ str(

analysis.get(

"recommended_action",

"UNKNOWN"

)

)

)

else:

print(

"Error: "

+ str(

result.get(

"error",

"UNKNOWN"

)

)

)

print()

if __name__ == "__main__":

results = analyze_active_incidents()

display_results(results)

36. Run AI Analysis

Run:

python -m app.ai_incident_persistence

The AI should analyze the existing incident.

You should see something similar to:

Analyzing incident:

Incident ID: ...

Router: R1

Type: PRIMARY_WAN_FAILURE

New AI analysis created.

Analysis ID: 1

The exact AI wording can vary.

The important part is:

AI → evidence-based analysis

not:

AI → Cisco configuration

37. AI Deduplication

Run the AI script again.

python -m app.ai_incident_persistence

You should now see:

AI analysis already exists.

The same analysis ID should be returned.

This prevents repeatedly calling Ollama for the same incident.

38. What AI Is Allowed to Do

AI can:

- interpret evidence

- correlate symptoms

- explain likely root cause

- assign confidence

- describe impact

- recommend an engineering action

- explain validation

AI cannot:

- execute Cisco commands

- invent interface names

- invent IP addresses

- claim remediation occurred

- bypass approval

- determine that a configuration is safe by itself

This is a critical design boundary.

39. Deterministic Remediation

For the controlled configuration-drift portion of the workflow, NetPilot uses verified state.

Example:

Interface:

GigabitEthernet2

Expected:

description CUSTOMER-WAN

Current:

description OLD-WAN

The remediation planner constructs:

interface GigabitEthernet2

description CUSTOMER-WAN

It does not ask the LLM:

"What Cisco commands should I run?"

That distinction is critical.

The LLM can say:

The interface configuration does not match the expected state.

The deterministic application decides exactly what commands are allowed.

40. Remediation Safety Policy

The current remediation planner only permits the controlled description-change scenario.

It validates:

- incident type

- drift state

- interface

- current state

- expected state

- command count

- command structure

- allowed configuration prefix

This keeps the lab intentionally bounded.

41. Engineer Approval

The approval workflow is:

Incident

↓

AI RCA

↓

Deterministic Plan

↓

Engineer Review

↓

APPROVED / REJECTED

The engineer sees:

Incident ID

Router

Interface

Current state

Expected state

Commands

Risk

No configuration should execute before approval.

Human approval remains an architectural boundary because someone ultimately owns the operational change and its consequences.

42. Final Script — app/incident_approval.py

The validated module is the persistent approval controller.

Its responsibilities are:

- validate remediation plan

- check router state

- display change review

- request engineer decision

- persist approval

- write audit event

- prevent configuration execution

Standalone execution:

python -m app.incident_approval

Expected:

========================================

NETPILOT PERSISTENT APPROVAL CONTROLLER

========================================

This module requires a persisted incident and deterministic remediation plan.

No Cisco configuration is executed by this module.

The module intentionally does not execute a change simply because it is run.

43. Remediation Execution

The remediation controller adds another safety layer.

Its sequence is:

Persisted Incident

↓

Persisted Approval

↓

Deterministic Plan Validation

↓

Live Pre-Change State

↓

Compare Against Planned State

↓

Execute

↓

Live Post-Change State

↓

Validation

↓

Resolve Incident

This protects against a particularly important race condition:

The network changes after NetPilot created the remediation plan.

44. Why Pre-Change Revalidation Matters

Imagine NetPilot observes:

Current:

description OLD-WAN

Then generates a plan.

Five minutes later an engineer manually changes the interface:

description CUSTOMER-WAN

If NetPilot blindly executes the original plan, its assumptions are stale.

Therefore:

Plan

↓

Re-read live state

↓

Compare

↓

Proceed only if state still matches

This is a real production engineering concept.

45. Final Script — app/incident_remediation.py

The validated remediation controller performs:

- approval verification

- plan validation

- live pre-change validation

- Cisco execution

- post-change validation

- persistence

- audit

- incident resolution

Standalone execution:

python -m app.incident_remediation

Expected:

========================================

NETPILOT PERSISTENT REMEDIATION CONTROLLER

========================================

This module executes only deterministic engineer-approved Cisco configuration.

The PostgreSQL incident database is the system of record.

No interactive change will be executed from this standalone module.

Again, the standalone module intentionally does not perform an unexpected network change.

46. Complete Change-Control Model

Your final architecture is:

┌───────────────┐

│    Ollama     │

│   qwen2.5     │

└───────┬───────┘

│

Analysis

│

▼

┌──────────┐       ┌──────────────┐

│ Cisco    │──────►│   Incident   │

│ Network  │       │    Engine    │

└──────────┘       └──────┬───────┘

│

▼

┌─────────────┐

│ PostgreSQL  │

└──────┬──────┘

│

Deterministic

Plan

│

▼

┌─────────────┐

│  Engineer   │

│   Approval  │

└──────┬──────┘

│

▼

Pre-Change Check

│

▼

Cisco Change

│

▼

Post-Change Check

│

▼

Audit + Resolve

47. Incident Orchestrator

The orchestrator ties everything together.

The intended sequence is:

collect

↓

detect

↓

persist

↓

AI

↓

plan

↓

approval

↓

pre-check

↓

change

↓

post-check

↓

resolve

The validated orchestrator currently successfully performs:

network collection

incident detection

PostgreSQL lookup

AI reuse

AI RCA display

change-control state display

Important final correction before enabling automatic change control

The earlier orchestrator contained a naming collision:

run_change_control

was used both as a function name and a Boolean parameter.

That must not be carried into the final executable guide.

Use a distinct parameter such as:

enable_change_control

before activating the write path.

This is exactly the type of software-engineering defect that should be eliminated before allowing an automation system to touch a network.

48. Unified NOC Workflow

The dashboard/orchestrator should present something like:

========================================

NETPILOT INCIDENT ORCHESTRATOR

========================================

Workflow ID: 20260904131651106246

Status: COMPLETED

ACTIVE INCIDENTS: 1

Incident: PRIMARY_WAN_FAILURE

Incident ID: 20260904125411222763

Router: R1

Status: ACTIVE

Impact:

Primary WAN path is unavailable on R1.

Traffic is using the backup WAN path.

AI STATUS: EXISTS

AI ANALYSIS ID: 1

AI ROOT CAUSE:

Primary WAN interface GigabitEthernet1 is

administratively down, causing the primary

WAN path to fail.

AI CONFIDENCE:

HIGH

AI RISK:

MEDIUM

The exact AI wording can differ.

The architecture should not.

49. Controlled Failure Recovery

Restore R1:

configure terminal

interface GigabitEthernet1

no shutdown

end

Wait for:

- interface recovery

- BGP convergence

- routing convergence

- primary path restoration

Verify:

show ip interface brief

show ip bgp summary

show ip route 0.0.0.0

50. Run NetPilot Again

python -m app.incident_manager

Expected:

Active incidents: 0

The incident should now have:

status = RESOLVED

and:

resolved_at

should be populated.

51. Validate the Database

Your persistence validation script should inspect:

incidents

incident_evidence

ai_analysis

approvals

remediation

validation

audit_events

Run:

python -m app.incident_persistence_validation

The validation should demonstrate the complete lifecycle.

52. Expected Lifecycle

A successful incident should now resemble:

INCIDENT CREATED

↓

EVIDENCE STORED

↓

AI ANALYSIS STORED

↓

APPROVAL STORED

↓

REMEDIATION STORED

↓

PRE-CHANGE VALIDATION

↓

CHANGE EXECUTED

↓

POST-CHANGE VALIDATION

↓

AUDIT EVENT

↓

INCIDENT RESOLVED

This is the most important outcome of Lab 10.

53. Final Validation Checklist

Cisco

- R1 reachable

- R2 reachable

- R3 reachable

- R4 reachable

- R5 reachable

- OSPF healthy

- BGP healthy

- primary WAN healthy

- backup WAN available

NetPilot

- Cisco state collection works

- monitoring works

- incident detection works

- correlation works

- duplicate incidents prevented

- PostgreSQL persistence works

- AI analysis works

- AI analysis deduplicates

- deterministic remediation works

- approval is persisted

- pre-change validation works

- post-change validation works

- audit events exist

- incidents resolve correctly

Failure Scenario

- R1 Gi1 shutdown

- primary BGP fails

- backup path becomes active

- NetPilot detects incident

- PostgreSQL stores incident

- Ollama analyzes incident

- AI result persisted

- remediation plan generated

- engineer approval recorded

- pre-change validation passes

- change executes

- post-change validation passes

- incident resolves

- WAN restored

54. What You Have Built

At this point NetPilot has evolved significantly.

Lab 7

Network automation foundation.

Lab 8

Customer incident operations and AI RCA.

Lab 9

Enterprise WAN routing.

Lab 10

Unified network operations.

The resulting architecture is:

Cisco

│

▼

NetPilot State Collection

│

▼

Monitoring

│

▼

Incident Correlation

│

▼

PostgreSQL

│

├──────────────► Incident History

│

├──────────────► Evidence

│

├──────────────► AI Analysis

│

├──────────────► Approval

│

├──────────────► Remediation

│

├──────────────► Validation

│

└──────────────► Audit

That is much closer to a real engineering platform than a collection of scripts.

55. FDE Questions & Answers

This section comes before the theory section intentionally.

You should be able to answer these without looking at your notes.

Q1. Why did we use BGP for the WAN?

Answer:

BGP provides policy-based path selection and is appropriate for controlling redundant WAN/Internet connectivity.

In this lab, Local Preference allows the customer to define:

ISP-1 = preferred

ISP-2 = backup

OSPF handles internal routing while BGP handles external routing policy.

Q2. Why not simply use OSPF everywhere?

Answer:

OSPF is an interior gateway protocol designed primarily for routing within an autonomous system.

BGP is designed for exchanging routes between autonomous systems and provides significantly more policy-control mechanisms.

A customer WAN architecture may therefore use:

OSPF = internal connectivity

BGP = external/WAN routing policy

Q3. Why does NetPilot need PostgreSQL?

Answer:

An in-memory Python dictionary disappears when the application stops.

PostgreSQL provides persistent operational state.

It allows NetPilot to remember:

- incidents

- evidence

- AI analysis

- approvals

- remediation

- validation

- audit events

That makes the system operationally useful rather than simply ephemeral.

Q4. Why shouldn't the LLM generate Cisco commands?

Answer:

Because an LLM is probabilistic.

A critical network command should be generated from deterministic, verified state whenever possible.

The preferred architecture is:

AI:

"What is happening?"

Application:

"What exact verified change is allowed?"

Engineer:

"Approve or reject."

Automation:

"Execute the approved deterministic change."

This dramatically reduces hallucination risk.

Q5. What is the most important source of truth?

Answer:

Verified live network state and approved configuration policy.

Not the LLM.

The model is a reasoning layer.

Q6. What happens if Ollama is unavailable?

Answer:

The incident should still exist.

Monitoring and incident persistence should continue.

AI analysis can fail independently.

The network workflow should not depend on the LLM being available for basic operational visibility.

Q7. Why store evidence separately from the incident?

Answer:

Because the incident says:

Something is wrong.

Evidence explains:

Why NetPilot believes something is wrong.

This supports:

- auditability

- troubleshooting

- AI analysis

- engineering review

- post-incident analysis

Q8. Why do we need pre-change validation?

Answer:

Because the network can change after the remediation plan is created.

Pre-change validation confirms:

Observed state

=

Expected state when plan was created

=

Current live state

If those assumptions no longer match, the change should stop.

Q9. Why do we need post-change validation?

Answer:

Because successful command execution does not prove successful network behavior.

For example:

Cisco accepted command

does not necessarily mean:

BGP recovered

WAN recovered

customer service recovered

Post-change validation checks the actual outcome.

Q10. Why correlate incidents?

Answer:

Because multiple alerts may represent one underlying failure.

For example:

Interface down

BGP down

Default route changed

Backup path active

are symptoms of:

PRIMARY_WAN_FAILURE

Correlation reduces alert noise and gives engineers a clearer operational picture.

Q11. Why is engineer approval important?

Answer:

Because the engineer remains accountable for network changes.

Automation can prepare and validate the change.

The engineer retains authority to:

- approve

- reject

- delay

- investigate further

That human-control boundary is fundamental to safe write-capable network agents.

Q12. What makes this an FDE lab?

Answer:

You aren't simply configuring Cisco devices.

You are solving a customer problem.

You are considering:

Business impact

Requirements

Architecture

Implementation

Integration

Automation

Observability

AI

Security

Change management

Validation

Documentation

Handoff

That is the FDE mindset.

Q13. What would you tell the customer NetPilot does?

Answer:

A strong customer explanation would be:

NetPilot continuously evaluates Cisco network state, correlates related symptoms into operational incidents, stores the evidence and incident history, uses a local AI model to assist with root-cause analysis, and prepares controlled remediation while keeping configuration execution behind deterministic validation and engineer approval.

Q14. What is the biggest architectural lesson from Lab 10?

Answer:

AI should not be the system of control.

AI provides reasoning.

Application code provides control.

Cisco provides the actual network state.

PostgreSQL provides persistence.

Engineers provide authorization and accountability.

Q15. What would you improve before production?

Answer:

Several things:

- proper secrets management

- RBAC

- stronger authorization

- API authentication

- structured logging

- metrics

- distributed locking

- transaction handling

- rollback

- configuration snapshots

- change windows

- rate limiting

- alert integration

- ticketing integration

- HA PostgreSQL

- HA NetPilot services

- stronger automated tests

- formal approval workflows

- production telemetry

This is where NetPilot begins moving from advanced lab toward product architecture.

56. FDE Customer Presentation Exercise

Pretend you are presenting this solution to a customer.

Explain:

Problem

Engineers spend too much time manually investigating repetitive WAN incidents.

Solution

NetPilot automatically collects network evidence and correlates related symptoms.

AI

Ollama analyzes the verified evidence and provides an evidence-based RCA.

Safety

AI cannot directly modify the network.

Change Management

Deterministic remediation is generated from verified state and requires engineer approval.

Validation

NetPilot validates the network before and after the change.

Audit

Every significant lifecycle event is persisted.

Business Value

Engineers spend less time gathering repetitive evidence and more time solving higher-value network problems.

That is the kind of explanation you should become comfortable delivering as an FDE.

57. Productization Opportunity

This is where NetPilot starts becoming potentially interesting as a business.

The core product concept could eventually become:

NetPilot Network Operations Copilot

Potential capabilities:

Cisco Discovery

↓

Health

↓

Incident Detection

↓

Evidence Collection

↓

AI RCA

↓

Change Recommendation

↓

Engineer Approval

↓

Controlled Automation

↓

Validation

↓

Audit

Potential customers:

- MSPs

- MSSPs

- enterprise network teams

- small NOCs

- Cisco-focused service providers

- organizations with limited network engineering staff

58. What Theory You Need to Study

Now that you've completed the practical work, study the theory behind it.

A. BGP

Study deeply:

- eBGP

- iBGP

- AS numbers

- BGP neighbors

- BGP states

- Local Preference

- MED

- AS Path

- Weight

- next-hop

- next-hop-self

- route advertisement

- route selection

- route convergence

- default routes

- BGP failure behavior

You should eventually be able to look at:

show ip bgp

show ip bgp summary

show ip route

and explain what is happening without relying on NetPilot.

59. OSPF

Study:

- neighbor formation

- adjacency

- areas

- LSAs

- SPF

- metrics

- default route propagation

- convergence

- passive interfaces

- router ID

- DR/BDR

- OSPF troubleshooting

Especially understand:

Why did R2 change its default path when R1's WAN failed?

60. Network Operations

Study:

- NOC operations

- incident management

- event correlation

- alert fatigue

- severity

- impact

- escalation

- incident lifecycle

- change management

- maintenance windows

- post-incident review

- root-cause analysis

- service restoration

- MTTR

- MTTD

- operational runbooks

You are now moving from:

network configuration

toward:

network operations engineering

61. Python

Study:

- functions

- modules

- packages

- dictionaries

- lists

- JSON

- exceptions

- classes

- imports

- environment variables

- logging

- API calls

- database connections

- CRUD operations

- validation

- state management

You don't need to become a software engineer overnight.

But you should become comfortable reading and modifying NetPilot without fear.

62. Netmiko

Study:

- SSH

- ConnectHandler

- send_command

- send_config_set

- connection lifecycle

- timeouts

- error handling

- configuration execution

- command verification

Most importantly:

Understand why Netmiko belongs below your operational decision layer.

63. PostgreSQL

Study:

- databases

- tables

- primary keys

- foreign keys

- indexes

- JSONB

- transactions

- timestamps

- SQL SELECT

- INSERT

- UPDATE

- DELETE

- JOIN

- constraints

- database users

- permissions

Understand why:

incident

has relationships to:

evidence

AI

approval

remediation

validation

audit

64. REST/API Fundamentals

Begin studying:

- HTTP

- GET

- POST

- PUT

- PATCH

- DELETE

- JSON

- REST

- API authentication

- API authorization

- status codes

- idempotency

This becomes especially important in the next NetPilot stages.

65. AI / LLM Theory

Study:

- LLM

- tokens

- context window

- temperature

- inference

- structured output

- hallucination

- prompt engineering

- model validation

- RAG

- embeddings

- vector databases

- tool calling

- agent loops

- guardrails

- human-in-the-loop

The most important concept for NetPilot:

LLM output is untrusted input until application code validates it.

Structured outputs are particularly useful in NetOps because predictable data is easier to validate and compare against actual evidence.

66. AI Safety

Study:

- prompt injection

- hallucination

- output validation

- authorization

- least privilege

- allowlists

- deterministic execution

- human approval

- audit logs

- rollback

- fail-closed behavior

The key mental model:

LLM = reasoning

Application = control

Engineer = authorization

Network = source of truth

67. Cisco FDE Skills

Begin thinking like an FDE.

Study:

Discovery

Can you ask the right questions?

Architecture

Can you design the solution?

Implementation

Can you build it?

Integration

Can you connect Cisco systems to other platforms?

Automation

Can you reduce repetitive engineering work?

Troubleshooting

Can you identify root cause?

Observability

Can you prove what is happening?

Customer Communication

Can you explain the technical solution to a non-specialist?

Handoff

Can another engineer operate what you built?

Documentation

Can you create a runbook?

Business Value

Can you explain why the customer should care?

68. Concepts You Should Be Able to Explain Without Notes

Before moving to Lab 11, make sure you can explain:

BGP Local Preference

BGP next-hop

iBGP

eBGP

OSPF convergence

default routes

WAN failover

incident correlation

MTTD

MTTR

root-cause analysis

deterministic automation

LLM hallucination

structured output

PostgreSQL persistence

JSONB

foreign keys

audit trails

human-in-the-loop

pre-change validation

post-change validation

rollback

least privilege

idempotency

API

REST

observability

telemetry

If you can configure something but cannot explain why the architecture works, keep studying it.

69. Final Lab 10 Architecture

You should now be able to visualize NetPilot as:

┌──────────────────┐

│   Cisco Network  │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Network State    │

│ Collection       │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Monitoring       │

│ + Correlation    │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Incident Engine  │

└────────┬─────────┘

│

▼

┌──────────────────┐

│   PostgreSQL     │

│ System of Record │

└────────┬─────────┘

│

┌────────────┴────────────┐

▼                         ▼

┌────────────────┐       ┌────────────────┐

│    Ollama      │       │ Deterministic  │

│    AI RCA      │       │ Remediation    │

└───────┬────────┘       └───────┬────────┘

│                        │

└────────────┬───────────┘

▼

┌──────────────────┐

│ Engineer Approval│

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Pre-Change Check │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Cisco Change     │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Post-Change      │

│ Validation       │

└────────┬─────────┘

│

▼

┌──────────────────┐

│ Audit + Resolve  │

└──────────────────┘

70. Lab 10 Completion Standard

Do not consider Lab 10 complete merely because the Python scripts run.

You have completed Lab 10 when you can demonstrate this entire customer story:

Healthy Cisco Network

↓

Customer WAN Failure

↓

NetPilot Detects It

↓

NetPilot Correlates Symptoms

↓

PostgreSQL Persists Incident

↓

Evidence Stored

↓

Ollama Performs RCA

↓

AI Analysis Persisted

↓

Deterministic Remediation Prepared

↓

Engineer Reviews

↓

Engineer Approves

↓

Live State Revalidated

↓

Cisco Change Executed

↓

Network Validated

↓

Incident Resolved

↓

Complete Audit Trail