NetPilot v0.6 Lab

AI-Powered Configuration Drift Detection, Approval & Automated Remediation

Lab Objective

In this lab, NetPilot is enhanced from configuration monitoring into an AI-assisted configuration remediation platform.

The completed workflow allows NetPilot to:

- Connect to Cisco routers.

- Collect their running configurations.

- Compare live configurations against golden configurations.

- Detect configuration drift.

- Identify the affected router.

- Identify the affected interface.

- Send the evidence to Qwen running locally through Ollama.

- Have AI diagnose the problem.

- Generate a recommended configuration change.

- Ask the network engineer for approval.

- Apply the change only when approved.

- Verify the configuration after remediation.

- Report the final compliance state.

The most important design principle introduced in v0.6 is:

AI recommends. The engineer approves. NetPilot executes. NetPilot verifies.

1. Lab Topology

The lab consists of three Cisco routers:

R1

/      \

/        \

R2 -------- R3

OSPF is used as the routing protocol.

Management addresses:

| Router | Management IP |
| --- | --- |
| R1 | 192.168.1.201 |
| R2 | 192.168.1.202 |
| R3 | 192.168.1.203 |

2. Software Components

The v0.6 solution uses:

- Python

- Netmiko

- Cisco IOS

- Git

- Ollama

- Qwen

- PowerShell

- EVE-NG or equivalent Cisco lab environment

The Python virtual environment is:

.venv

3. NetPilot v0.6 Architecture

The completed workflow is:

Cisco Network

|

v

Netmiko SSH

|

v

Running Configuration

|

v

Golden Config

|

v

Drift Detection

|

v

Interface Detection

|

v

Ollama / Qwen

|

v

AI Diagnosis

|

v

Recommended Change

|

v

Human Approval

/          \

NO            YES

|              |

v              v

STOP        Apply Change

|

v

Verify Change

|

v

Compliance Check

4. Important v0.6 Concept

NetPilot is intentionally not designed to allow the AI to make unrestricted changes.

The AI provides:

Diagnosis

+

Risk

+

Recommended Configuration

The engineer decides whether NetPilot should execute it.

This is important for a future commercial product because network engineers may not trust an AI system that can automatically modify production infrastructure without authorization.

5. Cisco Client

The Cisco client is responsible for connecting to the routers and executing commands.

Create or update:

app/cisco_client.py

Use the following final version:

import time

# Lets NetPilot pause between connection attempts.

from netmiko import ConnectHandler

# Imports Netmiko so NetPilot can connect to Cisco devices.

from app.config import DEVICES

# Imports the device information from the NetPilot configuration.

MAX_RETRIES = 3

# Defines the maximum number of connection attempts.

RETRY_DELAY = 2

# Defines how many seconds NetPilot waits between attempts.

def get_device(router):

# Finds the device information for the requested router.

if router not in DEVICES:

# Checks whether the router exists in the configuration.

raise ValueError(f"Unknown router: {router}")

# Stops execution if the router is not defined.

return DEVICES[router]

# Returns the device information.

def run_command(router, command):

# Connects to a router and executes a show command.

device = get_device(router)

# Retrieves the device information.

for attempt in range(1, MAX_RETRIES + 1):

# Attempts the connection multiple times if necessary.

print(f"Connecting to {router}...")

try:

# Starts the connection attempt.

connection = ConnectHandler(**device)

# Establishes the SSH connection.

print(f"{router} connected!")

# Confirms the connection succeeded.

output = connection.send_command(command)

# Sends the requested command to the router.

connection.disconnect()

# Closes the SSH connection.

return output

# Returns the router output.

except Exception as error:

# Handles connection errors.

print(f"{router} connection attempt {attempt} failed.")

if attempt == MAX_RETRIES:

# Checks whether all attempts have failed.

raise error

# Returns the original error.

time.sleep(RETRY_DELAY)

# Waits before trying again.

def send_config(router, commands):

# Connects to a router and applies configuration commands.

device = get_device(router)

# Retrieves the device information.

for attempt in range(1, MAX_RETRIES + 1):

# Attempts the connection multiple times.

print(f"Connecting to {router} for configuration...")

try:

# Starts the connection attempt.

connection = ConnectHandler(**device)

# Establishes the SSH connection.

print(f"{router} connected!")

# Confirms the connection succeeded.

output = connection.send_config_set(commands)

# Sends the configuration commands to the router.

connection.save_config()

# Saves the configuration.

connection.disconnect()

# Closes the SSH connection.

return output

# Returns the router response.

except Exception as error:

# Handles connection errors.

print(f"{router} configuration attempt {attempt} failed.")

if attempt == MAX_RETRIES:

# Checks whether all attempts failed.

raise error

# Returns the original error.

time.sleep(RETRY_DELAY)

# Waits before trying again.

6. Configuration File

The Cisco connection information is kept separate from the application logic.

Create/update:

app/config.py

Final version:

DEVICES = {

"R1": {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R2": {

"device_type": "cisco_ios",

"host": "192.168.1.202",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R3": {

"device_type": "cisco_ios",

"host": "192.168.1.203",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

}

Security note: In a real product, passwords should not be stored directly in source code. We'll address secrets management in a later version.

7. Golden Configuration

NetPilot uses a known-good configuration as the baseline.

The files are stored in:

data/golden/

The final files are:

data/golden/R1.cfg

data/golden/R2.cfg

data/golden/R3.cfg

The golden configurations represent the approved state of the network.

8. Golden Configuration Collection

Create/update:

app/golden_config.py

Final version:

from pathlib import Path

# Imports Path so NetPilot can work with files and folders.

from app.cisco_client import run_command

# Imports the function that connects to Cisco routers

# and runs commands.

ROUTERS = ["R1", "R2", "R3"]

# Defines the routers from which NetPilot will collect configurations.

def collect_running_config(router):

# Collects the current running configuration from one router.

print(f"Collecting configuration from {router}...")

# Shows which router NetPilot is working with.

config = run_command(router, "show running-config")

# Connects to the router and collects the running configuration.

return config

# Returns the configuration to the calling function.

def save_golden_config(router, config):

# Saves a router configuration as its golden configuration.

directory = Path("data/golden")

# Defines the golden configuration directory.

directory.mkdir(parents=True, exist_ok=True)

# Creates the directory if it does not already exist.

filename = directory / f"{router}.cfg"

# Creates a filename such as data/golden/R3.cfg.

filename.write_text(config)

# Writes the configuration to the file.

print(f"Saved golden configuration for {router}")

# Confirms that the configuration was saved.

def create_golden_config():

# Creates golden configurations for all routers.

for router in ROUTERS:

# Processes R1, R2, and R3.

config = collect_running_config(router)

# Collects the current configuration.

save_golden_config(router, config)

# Saves the configuration as the golden configuration.

if __name__ == "__main__":

# Checks whether this script is being executed directly.

create_golden_config()

# Starts the golden configuration process.

Run:

python -m app.golden_config

9. AI Drift Analysis

NetPilot sends the detected differences to Qwen.

Create/update:

app/ai_drift.py

Final version:

from app.ollama_client import ask_ollama

# Imports our existing function that sends information to Qwen.

def analyze_drift(

router,

interface,

expected,

current,

missing,

added,

):

# Creates a function that sends configuration drift to the AI.

prompt = f"""

You are NetPilot, an AI network troubleshooting assistant.

Analyze the configuration drift below.

Router:

{router}

Affected Interface:

{interface}

Expected Configuration:

{expected}

Current Configuration:

{current}

Missing Configuration Lines:

{missing}

Unexpected Configuration Lines:

{added}

Explain:

1. What changed?

2. Which router is affected?

3. Which interface is affected?

4. Why could this be a problem?

5. What network technology is affected?

6. What is the risk level: LOW, MEDIUM, or HIGH?

7. What configuration should be restored?

8. Give the exact Cisco configuration commands that should be considered.

Do not make changes to the router.

Only provide a diagnosis and recommended configuration.

"""

answer = ask_ollama(prompt)

# Sends the prompt to Ollama and receives Qwen's response.

return answer

# Returns the AI analysis.

10. Interface Detection

One of the important lessons from v0.6 was that simply sending configuration differences to AI was not enough.

Initially, Qwen could determine that R3 had drift but could not reliably determine which interface was affected.

NetPilot was therefore enhanced to inspect the router's interface configuration.

The final system successfully identified:

Router: R3

Interface: GigabitEthernet2

Expected:

description NETPILOT-GIT-TEST1

Current:

description NETPILOT-v06-DRIFT

This is an important improvement because NetPilot now supplies AI with structured evidence, rather than asking AI to guess from incomplete information.

11. Drift Detector

Create/update:

app/drift_detector.py

The final script performs the complete workflow:

from pathlib import Path

# Lets Python work with files.

from app.cisco_client import run_command

# Lets NetPilot send commands to Cisco routers.

from app.ai_drift import analyze_drift

# Lets NetPilot send configuration differences to Qwen.

from app.config_remediation import apply_configuration

# Lets NetPilot apply an approved configuration change.

routers = ["R1", "R2", "R3"]

# Defines the routers NetPilot will check.

def find_interface_changes(golden_lines, live_lines):

# Attempts to identify interface-specific configuration changes.

current_interface = None

# Stores the interface currently being processed.

changes = []

# Stores detected interface changes.

golden_interfaces = {}

# Stores interface descriptions from the golden configuration.

live_interfaces = {}

# Stores interface descriptions from the live configuration.

for line in golden_lines:

# Processes every line in the golden configuration.

stripped = line.strip()

# Removes unnecessary whitespace.

if stripped.startswith("interface "):

# Checks whether the line identifies an interface.

current_interface = stripped

# Stores the current interface.

elif (

current_interface

and stripped.startswith("description ")

):

# Checks whether the interface has a description.

golden_interfaces[current_interface] = stripped

# Stores the expected description.

current_interface = None

# Resets the interface tracker.

for line in live_lines:

# Processes every line in the live configuration.

stripped = line.strip()

# Removes unnecessary whitespace.

if stripped.startswith("interface "):

# Checks whether the line identifies an interface.

current_interface = stripped

# Stores the current interface.

elif (

current_interface

and stripped.startswith("description ")

):

# Checks whether the interface has a description.

live_interfaces[current_interface] = stripped

# Stores the current description.

for interface, expected in golden_interfaces.items():

# Compares expected interface descriptions.

current = live_interfaces.get(interface)

# Retrieves the current description.

if current and current != expected:

# Checks for a changed description.

changes.append(

(

interface,

expected,

current,

)

)

# Records the interface-specific change.

return changes

# Returns the detected changes.

def check_router(router):

# Checks one router for configuration drift.

print()

print("Checking " + router)

# Displays the router being checked.

golden_file = Path(

"data/golden/" + router + ".cfg"

)

# Finds the golden configuration.

golden = golden_file.read_text()

# Reads the golden configuration.

live = run_command(

router,

"show running-config",

)

# Gets the live configuration.

golden_lines = golden.splitlines()

# Splits the golden configuration into lines.

live_lines = live.splitlines()

# Splits the live configuration into lines.

if golden == live:

# Checks whether the configurations are identical.

print(router + " is compliant")

# Reports that the router is compliant.

return

# Stops processing this router.

print(router + " has configuration drift")

# Reports configuration drift.

missing = []

# Stores configuration missing from the live router.

added = []

# Stores configuration added to the live router.

for line in golden_lines:

# Processes golden configuration lines.

if line not in live_lines:

# Checks whether the line is missing.

missing.append(line)

# Adds the missing line.

for line in live_lines:

# Processes live configuration lines.

if line not in golden_lines:

# Checks whether the line was added.

added.append(line)

# Adds the unexpected line.

interface_changes = find_interface_changes(

golden_lines,

live_lines,

)

# Attempts to identify the affected interface.

if not interface_changes:

# Checks whether an interface-specific change was found.

print("No interface-specific changes found.")

# Reports that no interface was identified.

return

# Stops processing this router.

for interface, expected, current in interface_changes:

# Processes each interface change.

print("Detected interface: " + interface)

# Displays the affected interface.

print("Expected: " + expected)

# Displays the expected configuration.

print("Current: " + current)

# Displays the current configuration.

print()

print("Sending configuration drift to Qwen...")

# Indicates that AI analysis is starting.

answer = analyze_drift(

router,

interface,

expected,

current,

missing,

added,

)

# Sends the complete evidence to Qwen.

print()

print("===== NETPILOT AI DIAGNOSIS =====")

# Displays the AI diagnosis heading.

print(answer)

# Displays the AI response.

approval = input(

"\nApply the recommended configuration? (yes/no): "

)

# Requests human approval.

if approval.lower() != "yes":

# Checks whether the engineer rejected the change.

print(

"Change rejected. "

"No configuration was applied."

)

# Confirms that no change was made.

return

# Stops processing the router.

commands = [

f"interface {interface.replace('interface ', '')}",

expected,

]

# Builds the Cisco configuration commands.

print()

print("Applying approved configuration...")

# Reports that remediation is starting.

result = apply_configuration(

router,

commands,

)

# Applies the approved configuration.

print(result)

# Displays the router response.

print()

print("Verifying configuration...")

# Indicates verification is beginning.

verification = run_command(

router,

"show running-config | section interface",

)

# Retrieves the interface configuration.

if expected in verification:

# Checks whether the expected configuration exists.

print(

"Configuration successfully verified."

)

# Confirms successful remediation.

else:

# Handles verification failure.

print(

"WARNING: Configuration verification failed."

)

# Reports the problem.

for router in routers:

# Processes R1, R2, and R3.

check_router(router)

# Runs the drift detection workflow.

12. Configuration Remediation

Create/update:

app/config_remediation.py

Final version:

from app.cisco_client import send_config

# Imports the function that sends configuration to Cisco routers.

def apply_configuration(router, commands):

# Applies an approved configuration change.

print()

print("===== NETPILOT REMEDIATION =====")

# Displays the remediation heading.

print("Router:", router)

# Displays the router being modified.

print("Commands to be applied:")

# Displays the commands that will be executed.

for command in commands:

# Processes each configuration command.

print(command)

# Displays the command.

result = send_config(router, commands)

# Sends the approved commands to the router.

return result

# Returns the router response.

13. Human Approval

The approval mechanism is intentionally simple.

When NetPilot identifies a problem, it displays the proposed remediation and asks:

Apply the recommended configuration? (yes/no):

If the engineer enters:

no

NetPilot responds:

Change rejected. No configuration was applied.

The router remains unchanged.

If the engineer enters:

yes

NetPilot:

- Connects to the router.

- Applies the configuration.

- Saves the configuration.

- Verifies the result.

This was tested during v0.6 using both responses.

14. Example Drift Scenario

The test scenario involved R3.

The golden configuration contained:

description NETPILOT-GIT-TEST1

The live router contained:

description NETPILOT-v06-DRIFT

NetPilot ultimately identified:

Router: R3

Interface: GigabitEthernet2

The recommended correction was:

interface GigabitEthernet2

description NETPILOT-GIT-TEST1

15. AI Diagnosis

Qwen analyzed the evidence and identified:

Router:

R3

Interface:

GigabitEthernet2

Expected:

description NETPILOT-GIT-TEST1

Current:

description NETPILOT-v06-DRIFT

The AI also classified the change as a LOW risk configuration drift because the modification was an interface description and did not directly alter routing behavior.

This demonstrates an important concept:

NetPilot doesn't simply detect that something changed. It attempts to explain what changed, where it changed, why it matters, and what should be done about it.

16. Testing the Rejection Workflow

Run:

python -m app.drift_detector

When prompted:

Apply the recommended configuration? (yes/no):

Enter:

no

Expected behavior:

Change rejected. No configuration was applied.

NetPilot must not modify the router.

17. Testing the Approval Workflow

Run:

python -m app.drift_detector

When prompted:

Apply the recommended configuration? (yes/no):

Enter:

yes

NetPilot should:

Apply approved configuration

↓

Save configuration

↓

Verify configuration

The expected configuration should then be present on R3.

18. Final Compliance Test

After remediation, run:

python -m app.drift_detector

The final expected result is:

Checking R1

R1 is compliant

Checking R2

R2 is compliant

Checking R3

R3 is compliant

This confirms that NetPilot successfully returned the network to the expected configuration.

19. Final v0.6 Workflow

The completed NetPilot v0.6 workflow is:

1. Connect to Cisco device

↓

2. Collect running configuration

↓

3. Compare with golden configuration

↓

4. Detect configuration drift

↓

5. Identify affected interface

↓

6. Send evidence to Qwen

↓

7. Receive AI diagnosis

↓

8. Generate recommended remediation

↓

9. Ask engineer for approval

↓

┌────┴────┐

↓         ↓

NO        YES

↓         ↓

STOP     Apply Change

↓

Save Config

↓

Verify

↓

Final Compliance

20. What v0.6 Demonstrates

At the completion of this lab, NetPilot has moved beyond simple network scripting.

It now demonstrates the foundation of an AI-assisted network operations platform.

The system can:

Detect

Identify configuration drift.

Understand

Use Qwen to analyze the change.

Recommend

Provide a specific remediation.

Approve

Require human authorization.

Execute

Apply the approved configuration.

Verify

Confirm that the change succeeded.

21. Business/Product Significance

v0.6 is also an important milestone from the business perspective.

The technical concept we're demonstrating is:

AI-assisted network remediation with human-in-the-loop approval.

This could eventually become a product capability for:

- Network operations teams

- Network security teams

- IT departments

- Managed Service Providers

- Cisco-focused consulting organizations

The commercial product should eventually provide the same workflow through a user interface rather than requiring engineers to execute Python modules manually.

The future product experience could look like:

NETPILOT

────────────────────────────────────

⚠ Configuration Drift Detected

Customer: ABC Company

Site: Los Angeles

Device: R3

Interface: GigabitEthernet2

Expected:

NETPILOT-GIT-TEST1

Current:

NETPILOT-v06-DRIFT

Risk:

LOW

AI Recommendation:

Restore expected interface description.

[ APPROVE ]

[ REJECT ]

That is the direction in which the lab is taking us.

22. v0.6 Completion Checklist

Network Automation

- Netmiko connectivity

- Cisco command execution

- Automatic connection retries

- Configuration collection

- Configuration deployment

Configuration Management

- Golden configurations

- Configuration comparison

- Drift detection

- Interface identification

- Configuration remediation

- Post-change verification

Artificial Intelligence

- Ollama integration

- Qwen integration

- AI configuration analysis

- AI-generated diagnosis

- AI-generated recommendation

- Risk classification

Safety

- Human approval required

- Reject workflow

- Approve workflow

- Configuration verification

Product Foundation

- Separation of configuration and application logic

- Automated remediation architecture

- Human-in-the-loop architecture

- Foundation for future dashboard

- Foundation for future multi-device management

- Foundation for future commercial product

23. Final Result

The final v0.6 validation was:

PS E:\IT\Cisco Labs\NetPilot> python -m app.drift_detector

Checking R1

Connecting to R1...

R1 connected!

R1 is compliant

Checking R2

Connecting to R2...

R2 connected!

R2 is compliant

Checking R3

Connecting to R3...

R3 connected!

R3 is compliant

NetPilot v0.6 is complete.

The major accomplishment of this version is that NetPilot can now go from:

Configuration Drift

to:

AI Diagnosis

↓

Recommended Fix

↓

Engineer Approval

↓

Automated Remediation

↓

Verification

↓

Compliant Network

That is a significant step toward turning NetPilot from a collection of networking labs into a real product prototype.