NetPilot Lab v0.7

AI-Assisted Network Operations Dashboard & Remediation

Version: 0.7
Project: NetPilot
Platform: Windows / PowerShell
Network: Cisco IOS routers R1, R2, R3
AI: Ollama / Qwen 2.5 32B
Web Framework: Flask

1. Lab Objective

The objective of NetPilot v0.7 is to transform the existing NetPilot networking automation platform into an AI-assisted network operations dashboard.

By the end of this lab, NetPilot will provide:

- Cisco router connectivity monitoring

- OSPF health monitoring

- Configuration drift detection

- Affected-interface identification

- Expected vs. current configuration

- AI-powered drift analysis

- Recommended remediation

- Human approval before configuration changes

- Cisco configuration remediation

- Post-remediation verification

- Audit logging

- Web-based dashboard

- Fast offline-router detection

The resulting workflow is:

Cisco Routers

|

v

NetPilot Monitoring

|

+---- Connectivity

|

+---- OSPF Health

|

+---- Configuration Drift

|

v

AI Analysis

|

v

Recommended Fix

|

v

Human Approval

|

v

Cisco Router

|

v

Verification

|

v

Audit Log

2. Final Project Structure

The relevant project structure for v0.7 is:

NetPilot

│

├── app

│   ├── __init__.py

│   ├── ai_drift.py

│   ├── ai_git_analysis.py

│   ├── ai_health.py

│   ├── ai_remediation.py

│   ├── audit_log.py

│   ├── cisco_client.py

│   ├── config.py

│   ├── config_remediation.py

│   ├── drift_detector.py

│   ├── evidence_collector.py

│   ├── git_manager.py

│   ├── golden_config.py

│   ├── intent_engine.py

│   ├── main.py

│   ├── network_health.py

│   ├── ollama_client.py

│   └── web.py

│

├── data

│   └── golden

│       ├── R1.cfg

│       ├── R2.cfg

│       └── R3.cfg

│

└── tests

The Flask template is:

app

└── templates

└── dashboard.html

3. Activate the Python Environment

Open PowerShell in the NetPilot directory:

cd "E:\IT\Cisco Labs\NetPilot"

Activate the virtual environment:

.\.venv\Scripts\Activate.ps1

The prompt should show:

(.venv) PS E:\IT\Cisco Labs\NetPilot>

4. Install Flask

Flask is required for the NetPilot web dashboard.

python -m pip install flask

Verify Flask:

python -c "import flask; print(flask.__version__)"

5. Configure the Cisco Routers

NetPilot v0.7 uses three Cisco routers.

| Router | IP Address |
| --- | --- |
| R1 | 192.168.1.201 |
| R2 | 192.168.1.202 |
| R3 | 192.168.1.203 |

The routers use the NetPilot SSH account.

6. Final app/cisco_client.py

This module provides the Cisco connectivity layer.

The v0.7 version includes a short timeout for monitoring so that offline devices are detected quickly.

Configuration operations receive a longer timeout and retry behavior.

Replace:

app/cisco_client.py

with:

from netmiko import ConnectHandler

import time

DEVICES = {

"R1": {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R2": {

"device_type": "cisco_ios",

"host": "192.168.1.202",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R3": {

"device_type": "cisco_ios",

"host": "192.168.1.203",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

}

def connect_to_router(

device_name,

timeout=5,

retries=3

):

"""

Connect to a Cisco router.

timeout:

Maximum time allowed for the connection attempt.

retries:

Number of connection attempts.

"""

if device_name not in DEVICES:

print(

"Unknown device: "

+ device_name

)

return None

device = DEVICES[device_name].copy()

device["conn_timeout"] = timeout

print(

"Connecting to "

+ device_name

+ "..."

)

for attempt in range(1, retries + 1):

try:

connection = ConnectHandler(

**device

)

print(

device_name

+ " connected!"

)

return connection

except Exception as error:

print(

device_name

+ " connection attempt "

+ str(attempt)

+ " failed."

)

if attempt < retries:

time.sleep(1)

else:

print(

device_name

+ " could not be reached."

)

print(error)

return None

def run_command(

device_name,

command,

timeout=5

):

"""

Connect to a router and execute a command.

"""

connection = connect_to_router(

device_name,

timeout=timeout,

retries=1

)

if connection is None:

return None

try:

output = connection.send_command(

command

)

return output

finally:

connection.disconnect()

def run_config(

device_name,

commands,

timeout=10

):

"""

Connect to a router and send configuration commands.

Configuration operations use a longer timeout

than dashboard monitoring.

"""

connection = connect_to_router(

device_name,

timeout=timeout,

retries=3

)

if connection is None:

return None

try:

output = connection.send_config_set(

commands

)

return output

finally:

connection.disconnect()

7. Final app/config_remediation.py

This module performs the approved Cisco configuration change.

It requires explicit engineer approval before sending the configuration.

Replace:

app/config_remediation.py

with:

from app.cisco_client import run_config, run_command

from app.audit_log import log_remediation

def apply_configuration(

router,

interface,

old_configuration,

new_configuration

):

"""

Applies an approved configuration change.

"""

print()

print("===== NETPILOT REMEDIATION =====")

print("Router: " + router)

print("Interface: " + interface)

print()

print("Current configuration:")

print(old_configuration)

print()

print("Recommended configuration:")

print(new_configuration)

print()

approval = input(

"Apply this change? (yes/no): "

)

if approval.lower() != "yes":

print()

print("Change cancelled by engineer.")

log_remediation(

router,

interface,

old_configuration,

new_configuration,

"NO",

"NOT APPLIED"

)

return False

commands = [

"interface " + interface,

new_configuration

]

print()

print(

"Applying approved configuration..."

)

output = run_config(

router,

commands

)

print(output)

print()

print("Configuration applied.")

print()

print("Verifying configuration...")

verification = run_command(

router,

"show running-config interface "

+ interface

)

print()

print("Verification output:")

print(verification)

if (

verification is not None

and new_configuration in verification

):

result = "SUCCESS"

print()

print("Verification successful.")

else:

result = "VERIFICATION FAILED"

print()

print("Verification failed.")

log_remediation(

router,

interface,

old_configuration,

new_configuration,

"YES",

result

)

return result == "SUCCESS"

8. Final app/web.py

This is the main Flask application.

It provides:

- Dashboard

- Router health

- OSPF status

- Drift detection

- Interface identification

- AI analysis

- Events

- Remediation approval

- Server-side validation

Replace:

app/web.py

with:

from flask import Flask, render_template, request, redirect, url_for

from pathlib import Path

from app.cisco_client import DEVICES, run_command

from app.ai_drift import analyze_drift

from app.config_remediation import apply_configuration

app = Flask(__name__)

# ============================================================

# ROUTER HEALTH

# ============================================================

def get_router_health(router):

ospf = run_command(

router,

"show ip ospf neighbor"

)

if ospf is None:

return {

"status": "OFFLINE",

"ospf": "UNKNOWN"

}

if "FULL" in ospf:

return {

"status": "ONLINE",

"ospf": "HEALTHY"

}

return {

"status": "ONLINE",

"ospf": "WARNING"

}

# ============================================================

# PARSE CISCO INTERFACES

# ============================================================

def parse_interfaces(configuration):

interfaces = {}

current_interface = None

if not configuration:

return interfaces

for raw_line in configuration.splitlines():

line = raw_line.strip()

if line.startswith("interface "):

current_interface = line[

len("interface "):

].strip()

interfaces[current_interface] = []

continue

if current_interface is not None and line:

interfaces[current_interface].append(

line

)

return interfaces

# ============================================================

# INTERFACE DESCRIPTION

# ============================================================

def get_description(interface_lines):

for line in interface_lines:

if line.startswith("description "):

return line

return None

# ============================================================

# DRIFT DETECTION

# ============================================================

def get_drift_status(router):

golden_path = Path(

f"data/golden/{router}.cfg"

)

if not golden_path.exists():

return {

"status": "UNKNOWN",

"interface": "",

"expected": "",

"current": "",

"diagnosis": ""

}

golden = golden_path.read_text(

encoding="utf-8",

errors="ignore"

)

live = run_command(

router,

"show running-config"

)

if live is None:

return {

"status": "UNKNOWN",

"interface": "",

"expected": "",

"current": "",

"diagnosis": ""

}

golden_interfaces = parse_interfaces(

golden

)

live_interfaces = parse_interfaces(

live

)

all_interfaces = sorted(

set(golden_interfaces.keys())

|

set(live_interfaces.keys())

)

for interface in all_interfaces:

golden_description = get_description(

golden_interfaces.get(

interface,

[]

)

)

live_description = get_description(

live_interfaces.get(

interface,

[]

)

)

if golden_description != live_description:

expected = (

golden_description

if golden_description

else ""

)

current = (

live_description

if live_description

else ""

)

missing = []

added = []

if expected:

missing.append(

expected

)

if current:

added.append(

current

)

try:

diagnosis = analyze_drift(

router,

missing,

added

)

except Exception as error:

diagnosis = (

"AI analysis unavailable: "

+ str(error)

)

return {

"status": "DRIFT",

"interface": interface,

"expected": expected,

"current": current,

"diagnosis": diagnosis

}

return {

"status": "COMPLIANT",

"interface": "",

"expected": "",

"current": "",

"diagnosis": ""

}

# ============================================================

# EVENTS

# ============================================================

def create_events(router, health, drift):

events = []

if health["status"] == "OFFLINE":

events.append(

{

"router": router,

"type": "DEVICE",

"severity": "HIGH",

"title": "Router Offline",

"message":

f"{router} could not be reached."

}

)

return events

if health["ospf"] == "WARNING":

events.append(

{

"router": router,

"type": "OSPF",

"severity": "MEDIUM",

"title": "OSPF Warning",

"message":

f"{router} is online, but no FULL "

"OSPF neighbor was detected."

}

)

if drift["status"] == "DRIFT":

events.append(

{

"router": router,

"type": "DRIFT",

"severity": "HIGH",

"title": "Configuration Drift",

"message":

f"{router} has configuration drift.",

"interface":

drift["interface"],

"expected":

drift["expected"],

"current":

drift["current"],

"diagnosis":

drift["diagnosis"]

}

)

if (

health["status"] == "ONLINE"

and health["ospf"] == "HEALTHY"

and drift["status"] == "COMPLIANT"

):

events.append(

{

"router": router,

"type": "HEALTH",

"severity": "INFO",

"title": "Network Healthy",

"message":

f"{router} is online, OSPF is healthy, "

"and configuration is compliant."

}

)

return events

# ============================================================

# DASHBOARD

# ============================================================

@app.route("/")

def dashboard():

routers = []

events = []

online = 0

offline = 0

ospf_healthy = 0

ospf_warnings = 0

drift_count = 0

for router in DEVICES:

health = get_router_health(

router

)

drift = get_drift_status(

router

)

if health["status"] == "ONLINE":

online += 1

else:

offline += 1

if health["ospf"] == "HEALTHY":

ospf_healthy += 1

elif health["ospf"] == "WARNING":

ospf_warnings += 1

if drift["status"] == "DRIFT":

drift_count += 1

routers.append(

{

"name":

router,

"status":

health["status"],

"ospf":

health["ospf"],

"drift_status":

drift["status"],

"interface":

drift["interface"],

"expected":

drift["expected"],

"current":

drift["current"],

"diagnosis":

drift["diagnosis"]

}

)

events.extend(

create_events(

router,

health,

drift

)

)

summary = {

"devices":

len(DEVICES),

"online":

online,

"offline":

offline,

"ospf_healthy":

ospf_healthy,

"warnings":

ospf_warnings,

"drift":

drift_count,

"events":

len(events)

}

return render_template(

"dashboard.html",

routers=routers,

summary=summary,

events=events

)

# ============================================================

# APPROVE REMEDIATION

# ============================================================

@app.route(

"/remediate",

methods=["POST"]

)

def remediate():

router = request.form.get(

"router",

""

).strip()

interface = request.form.get(

"interface",

""

).strip()

submitted_expected = request.form.get(

"expected",

""

).strip()

submitted_current = request.form.get(

"current",

""

).strip()

if not router:

return redirect(

url_for("dashboard")

)

if not interface:

return redirect(

url_for("dashboard")

)

if router not in DEVICES:

return redirect(

url_for("dashboard")

)

golden_path = Path(

f"data/golden/{router}.cfg"

)

if not golden_path.exists():

print(

"REMEDIATION BLOCKED: "

"Golden configuration not found."

)

return redirect(

url_for("dashboard")

)

golden = golden_path.read_text(

encoding="utf-8",

errors="ignore"

)

golden_interfaces = parse_interfaces(

golden

)

if interface not in golden_interfaces:

print(

"REMEDIATION BLOCKED: "

"Interface is not present in golden configuration."

)

return redirect(

url_for("dashboard")

)

golden_expected = get_description(

golden_interfaces.get(

interface,

[]

)

)

golden_expected = (

golden_expected

if golden_expected

else ""

)

if submitted_expected != golden_expected:

print(

"REMEDIATION BLOCKED: "

"Submitted configuration does not match "

"the golden configuration."

)

return redirect(

url_for("dashboard")

)

live = run_command(

router,

"show running-config interface "

+ interface

)

if live is None:

print(

"REMEDIATION BLOCKED: "

"Could not verify current router state."

)

return redirect(

url_for("dashboard")

)

live_interfaces = parse_interfaces(

live

)

actual_current = get_description(

live_interfaces.get(

interface,

[]

)

)

actual_current = (

actual_current

if actual_current

else ""

)

if actual_current != submitted_current:

print(

"REMEDIATION BLOCKED: "

"Router configuration changed "

"since the approval page was generated."

)

return redirect(

url_for("dashboard")

)

if actual_current == golden_expected:

print(

"REMEDIATION BLOCKED: "

"Router is already compliant."

)

return redirect(

url_for("dashboard")

)

print()

print(

"===== APPROVED NETPILOT REMEDIATION ====="

)

print(

"Router: "

+ router

)

print(

"Interface: "

+ interface

)

print(

"Current: "

+ actual_current

)

print(

"Expected: "

+ golden_expected

)

result = apply_configuration(

router,

interface,

actual_current,

golden_expected

)

print(

"Remediation result: "

+ str(result)

)

return redirect(

url_for("dashboard")

)

# ============================================================

# APPLICATION START

# ============================================================

if __name__ == "__main__":

app.run(

host="127.0.0.1",

port=5000,

debug=True

)

9. Final app/templates/dashboard.html

Create or replace:

app/templates/dashboard.html

with the following complete dashboard.

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta

name="viewport"

content="width=device-width, initial-scale=1.0"

>

<title>NetPilot</title>

<style>

* {

box-sizing: border-box;

}

body {

margin: 0;

font-family:

Arial,

Helvetica,

sans-serif;

background:

linear-gradient(

135deg,

#020617,

#0f172a,

#111827

);

color: #e2e8f0;

min-height: 100vh;

}

header {

padding: 28px 45px;

background:

rgba(

15,

23,

42,

0.95

);

border-bottom:

1px solid #334155;

box-shadow:

0 4px 20px

rgba(0, 0, 0, 0.35);

}

header h1 {

margin: 0;

font-size: 34px;

letter-spacing: 1px;

color: #f8fafc;

}

header p {

margin: 7px 0 0;

color: #94a3b8;

font-size: 14px;

}

.container {

max-width: 1450px;

margin: auto;

padding:

35px 40px 50px;

}

h2 {

margin:

30px 0 20px;

color: #f8fafc;

font-size: 22px;

}

.summary {

display: grid;

grid-template-columns:

repeat(5, 1fr);

gap: 18px;

}

.summary-card {

background:

rgba(

30,

41,

59,

0.9

);

border:

1px solid #334155;

border-radius: 14px;

padding: 22px;

box-shadow:

0 8px 25px

rgba(0, 0, 0, 0.25);

}

.summary-card span {

display: block;

color: #94a3b8;

font-size: 11px;

text-transform:

uppercase;

letter-spacing: 1px;

}

.summary-card strong {

display: block;

margin-top: 9px;

font-size: 32px;

color: #f8fafc;

}

.routers {

display: grid;

grid-template-columns:

repeat(

auto-fit,

minmax(450px, 1fr)

);

gap: 22px;

}

.router {

background:

rgba(

30,

41,

59,

0.95

);

border:

1px solid #334155;

border-radius: 15px;

padding: 25px;

box-shadow:

0 10px 30px

rgba(0, 0, 0, 0.3);

}

.router-header {

display: flex;

justify-content:

space-between;

align-items: center;

margin-bottom: 20px;

}

.router-name {

font-size: 25px;

font-weight: bold;

color: #f8fafc;

}

.badge {

padding:

7px 14px;

border-radius: 20px;

font-size: 11px;

font-weight: bold;

letter-spacing: 0.5px;

}

.online {

background: #064e3b;

color: #6ee7b7;

}

.offline {

background: #7f1d1d;

color: #fecaca;

}

.info {

display: grid;

grid-template-columns:

1fr 1fr;

gap: 12px;

}

.info-box {

background:

#0f172a;

border:

1px solid #334155;

border-radius: 9px;

padding: 15px;

}

.label {

font-size: 10px;

color: #94a3b8;

text-transform:

uppercase;

letter-spacing: 0.8px;

}

.value {

margin-top: 7px;

font-weight: bold;

color: #f8fafc;

}

.compliant {

margin-top: 20px;

padding: 16px;

background:

#022c22;

border:

1px solid #065f46;

border-radius: 9px;

color: #6ee7b7;

font-weight: bold;

}

.drift {

margin-top: 20px;

padding: 22px;

background:

linear-gradient(

145deg,

#2a1111,

#1f1111

);

border:

1px solid #991b1b;

border-radius: 12px;

}

.drift-title {

margin-bottom: 20px;

font-size: 20px;

font-weight: bold;

color: #fca5a5;

}

.drift-item {

margin-bottom: 17px;

}

.drift-label {

margin-bottom: 7px;

font-size: 10px;

color: #94a3b8;

text-transform:

uppercase;

letter-spacing: 0.8px;

}

.drift-value {

padding: 13px;

background:

#020617;

border:

1px solid #475569;

border-radius: 7px;

font-family:

Consolas,

"Courier New",

monospace;

color: #f8fafc;

word-break: break-word;

}

.expected {

border-left:

4px solid #22c55e;

}

.current {

border-left:

4px solid #ef4444;

}

.ai {

margin-top: 22px;

padding: 20px;

background:

linear-gradient(

145deg,

#111827,

#17153a

);

border:

1px solid #6366f1;

border-radius: 10px;

box-shadow:

0 0 20px

rgba(

99,

102,

241,

0.12

);

}

.ai-title {

margin-bottom: 13px;

font-size: 18px;

font-weight: bold;

color: #c7d2fe;

}

.ai-response {

white-space: pre-wrap;

line-height: 1.65;

font-size: 14px;

color: #cbd5e1;

}

.remediation {

margin-top: 22px;

padding: 20px;

background:

#0c1a2e;

border:

1px solid #2563eb;

border-radius: 10px;

}

.remediation-title {

margin-bottom: 12px;

font-size: 18px;

font-weight: bold;

color: #93c5fd;

}

.remediation-warning {

margin-bottom: 16px;

padding: 12px;

background:

#172554;

border:

1px solid #1d4ed8;

border-radius: 7px;

color: #bfdbfe;

font-size: 13px;

line-height: 1.5;

}

.command-box {

margin-bottom: 18px;

padding: 15px;

background:

#020617;

border:

1px solid #475569;

border-radius: 7px;

font-family:

Consolas,

"Courier New",

monospace;

color: #86efac;

white-space: pre-wrap;

line-height: 1.6;

}

.approve-button {

width: 100%;

padding: 14px 18px;

border: none;

border-radius: 8px;

background:

linear-gradient(

135deg,

#2563eb,

#4f46e5

);

color: white;

font-size: 14px;

font-weight: bold;

cursor: pointer;

transition:

transform 0.15s,

opacity 0.15s;

}

.approve-button:hover {

transform:

translateY(-1px);

opacity: 0.9;

}

.approve-button:active {

transform:

translateY(0);

}

.events {

display: grid;

gap: 12px;

}

.event {

padding: 17px;

border-radius: 9px;

background:

#1e293b;

border:

1px solid #334155;

}

.event-high {

border-left:

4px solid #ef4444;

}

.event-medium {

border-left:

4px solid #f59e0b;

}

.event-info {

border-left:

4px solid #22c55e;

}

.event-title {

font-weight: bold;

color: #f8fafc;

margin-bottom: 5px;

}

.event-message {

color: #94a3b8;

font-size: 13px;

}

footer {

margin-top: 50px;

padding-top: 20px;

border-top:

1px solid #334155;

text-align: center;

color: #64748b;

font-size: 13px;

}

@media (max-width: 1100px) {

.summary {

grid-template-columns:

repeat(3, 1fr);

}

}

@media (max-width: 800px) {

.summary {

grid-template-columns:

repeat(2, 1fr);

}

.routers {

grid-template-columns:

1fr;

}

}

@media (max-width: 550px) {

.container {

padding: 20px;

}

.summary {

grid-template-columns:

1fr;

}

.info {

grid-template-columns:

1fr;

}

}

</style>

</head>

<body>

<header>

<h1>NetPilot</h1>

<p>

AI Network Operations Platform

</p>

</header>

<div class="container">

<h2>

Network Overview

</h2>

<div class="summary">

<div class="summary-card">

<span>

Devices

</span>

<strong>

{{ summary.devices }}

</strong>

</div>

<div class="summary-card">

<span>

Online

</span>

<strong>

{{ summary.online }}

</strong>

</div>

<div class="summary-card">

<span>

Offline

</span>

<strong>

{{ summary.offline }}

</strong>

</div>

<div class="summary-card">

<span>

OSPF Healthy

</span>

<strong>

{{ summary.ospf_healthy }}

</strong>

</div>

<div class="summary-card">

<span>

Configuration Drift

</span>

<strong>

{{ summary.drift }}

</strong>

</div>

</div>

<h2>

Network Events

</h2>

<div class="events">

{% for event in events %}

<div

class="

event

{% if event.severity == 'HIGH' %}

event-high

{% elif event.severity == 'MEDIUM' %}

event-medium

{% else %}

event-info

{% endif %}

"

>

<div class="event-title">

{{ event.title }}

— {{ event.router }}

</div>

<div class="event-message">

{{ event.message }}

</div>

</div>

{% endfor %}

</div>

<h2>

Network Devices

</h2>

<div class="routers">

{% for router in routers %}

<div class="router">

<div class="router-header">

<div class="router-name">

{{ router.name }}

</div>

{% if router.status == "ONLINE" %}

<div class="badge online">

ONLINE

</div>

{% else %}

<div class="badge offline">

OFFLINE

</div>

{% endif %}

</div>

<div class="info">

<div class="info-box">

<div class="label">

Connectivity

</div>

<div class="value">

{{ router.status }}

</div>

</div>

<div class="info-box">

<div class="label">

OSPF

</div>

<div class="value">

{{ router.ospf }}

</div>

</div>

</div>

{% if router.drift_status == "COMPLIANT" %}

<div class="compliant">

✓ Configuration Compliant

</div>

{% elif router.drift_status == "DRIFT" %}

<div class="drift">

<div class="drift-title">

⚠ Configuration Drift Detected

</div>

<div class="drift-item">

<div class="drift-label">

Affected Interface

</div>

<div class="drift-value">

{{ router.interface }}

</div>

</div>

<div class="drift-item">

<div class="drift-label">

Expected Configuration

</div>

<div

class="

drift-value

expected

"

>

{{ router.expected }}

</div>

</div>

<div class="drift-item">

<div class="drift-label">

Current Configuration

</div>

<div

class="

drift-value

current

"

>

{{ router.current }}

</div>

</div>

{% if router.diagnosis %}

<div class="ai">

<div class="ai-title">

🤖 NetPilot AI Analysis

</div>

<div class="ai-response">

{{ router.diagnosis }}

</div>

</div>

{% endif %}

{% if router.interface and router.expected %}

<div class="remediation">

<div class="remediation-title">

🛠 Recommended Remediation

</div>

<div class="remediation-warning">

Review the configuration below before

approving. Clicking the button will send

the approved configuration to the Cisco

router.

</div>

<div class="command-box">

interface {{ router.interface }}

{{ router.expected }}

</div>

<form

method="POST"

action="/remediate"

>

<input

type="hidden"

name="router"

value="{{ router.name }}"

>

<input

type="hidden"

name="interface"

value="{{ router.interface }}"

>

<input

type="hidden"

name="current"

value="{{ router.current }}"

>

<input

type="hidden"

name="expected"

value="{{ router.expected }}"

>

<button

type="submit"

class="approve-button"

onclick="

return confirm(

'Are you sure you want to apply this configuration to {{ router.name }}?'

);

"

>

✓ APPROVE & REMEDIATE

</button>

</form>

</div>

{% endif %}

</div>

{% endif %}

</div>

{% endfor %}

</div>

<footer>

NetPilot v0.7

— AI-Assisted Network Operations

</footer>

</div>

</body>

</html>

10. Start the NetPilot Dashboard

From the NetPilot directory:

python -m app.web

You should see Flask start the development server.

Open a browser and go to:

http://127.0.0.1:5000

11. Verify the Dashboard

The dashboard should display:

Network Overview

Devices

3

Online

3

Offline

0

OSPF Healthy

3

Configuration Drift

0

Each router should show:

R1

ONLINE

Connectivity: ONLINE

OSPF: HEALTHY

Configuration Compliant

R2

ONLINE

Connectivity: ONLINE

OSPF: HEALTHY

Configuration Compliant

R3

ONLINE

Connectivity: ONLINE

OSPF: HEALTHY

Configuration Compliant

12. Test Offline Detection

To demonstrate NetPilot detecting an unavailable device, power off R2.

Refresh the dashboard.

NetPilot should identify R2 as:

OFFLINE

The dashboard should also create a network event:

Router Offline — R2

R2 could not be reached.

The v0.7 Cisco client uses a short connection timeout so the dashboard does not wait excessively long for an unavailable router.

Power R2 back on after the demonstration.

13. Controlled Configuration Drift Demonstration

Now we demonstrate the most important v0.7 feature.

With R3 online, introduce a controlled configuration change.

Open a second PowerShell window in the NetPilot directory.

Run:

python -c "from app.cisco_client import run_config; print(run_config('R3', ['interface GigabitEthernet2', 'description NETPILOT-DRIFT-DEMO']))"

This intentionally changes R3's interface description.

The golden configuration contains:

description NETPILOT-GIT-TEST1

The live router will now contain:

description NETPILOT-DRIFT-DEMO

14. Refresh the Dashboard

Return to:

http://127.0.0.1:5000

Refresh the page.

R3 should now show:

⚠ Configuration Drift Detected

The dashboard should identify:

Affected Interface

GigabitEthernet2

15. Expected Configuration

The dashboard should show:

description NETPILOT-GIT-TEST1

This is the configuration stored in the NetPilot golden configuration.

16. Current Configuration

The dashboard should show:

description NETPILOT-DRIFT-DEMO

This is the configuration currently detected on R3.

17. AI Analysis

The dashboard will then display:

🤖 NetPilot AI Analysis

The local Ollama/Qwen model analyzes the detected drift and provides an explanation/recommendation.

The architecture is:

R3

|

| show running-config

v

NetPilot

|

| Compare with golden configuration

v

Drift Detected

|

v

Ollama

|

v

Qwen 2.5 32B

|

v

AI Analysis

18. Recommended Remediation

The dashboard should display:

🛠 Recommended Remediation

The Cisco configuration should resemble:

interface GigabitEthernet2

description NETPILOT-GIT-TEST1

The engineer can review the proposed change before allowing NetPilot to modify the router.

19. Approve the Remediation

Click:

✓ APPROVE & REMEDIATE

The browser asks for confirmation.

Confirm the change.

NetPilot then performs additional validation on the server side.

It verifies:

- Router exists

- Interface exists in golden configuration

- Expected configuration matches golden configuration

- Current router configuration has not unexpectedly changed

- Router is still actually out of compliance

Only after those checks pass does NetPilot call the remediation engine.

20. Cisco Remediation

NetPilot sends:

interface GigabitEthernet2

description NETPILOT-GIT-TEST1

to R3.

The remediation engine then retrieves:

show running-config interface GigabitEthernet2

and verifies that the expected configuration is present.

21. Verification

A successful remediation produces:

Verification successful.

The audit system records the remediation event.

The result is:

SUCCESS

22. Final Dashboard State

Refresh the dashboard.

R3 should now return to:

✓ Configuration Compliant

The configuration drift counter should return to:

0

The network should again show all three routers as compliant.

23. Complete v0.7 Workflow

The complete demonstration is:

R3

|

v

Configuration Changed

|

v

NetPilot Detects Drift

|

v

GigabitEthernet2

|

v

Expected vs Current

|

v

Ollama / Qwen

|

v

AI Analysis

|

v

Recommended Fix

|

v

Human Approval

|

v

Server Validation

|

v

Cisco Configuration

|

v

Verification

|

v

Audit Log

|

v

COMPLIANT

24. Lab v0.7 Completion Checklist

| Feature | Status |
| --- | --- |
| Flask Dashboard | ✅ |
| Network Overview | ✅ |
| Router Connectivity | ✅ |
| Fast Offline Detection | ✅ |
| OSPF Monitoring | ✅ |
| Golden Configuration | ✅ |
| Drift Detection | ✅ |
| Affected Interface | ✅ |
| Expected Configuration | ✅ |
| Current Configuration | ✅ |
| Ollama/Qwen AI Analysis | ✅ |
| Recommended Remediation | ✅ |
| Human Approval | ✅ |
| Server-Side Validation | ✅ |
| Cisco Remediation | ✅ |
| Post-Change Verification | ✅ |
| Audit Logging | ✅ |
| Dashboard Styling | ✅ |

25. Lab v0.7 Final Result

At the completion of Lab v0.7, NetPilot has progressed from a collection of network automation scripts into an AI-assisted network operations platform.

The platform can observe the network, identify configuration problems, use a local LLM to analyze the problem, recommend a corrective action, obtain human approval, make the Cisco change, and verify the result.

The important architectural principle established in v0.7 is:

AI recommends. NetPilot validates. A human approves. NetPilot executes. NetPilot verifies.

That principle will carry forward into the next versions of NetPilot.