LAB LESSON 08: NetPilot Customer Incident Operations & AI Root-Cause Analysis

AI-Assisted Cisco Incident Investigation, Deterministic Remediation, Engineer Approval, Pre-Change Validation, and Post-Change Verification

Environment: EVE-NG Cisco IOS Lab + Windows NetPilot Host

Platform: Python / Flask / Netmiko / Ollama

AI Model: Qwen 2.5 32B via Ollama

Topology: Existing 3-Router Lab — R1, R2, R3

Goal: Build an end-to-end FDE-style customer incident workflow from detection through verified remediation and resolution.

Lab Overview & Objectives

This lab transforms NetPilot from a basic network monitoring and remediation tool into an AI-assisted network incident operations platform.

The lab simulates a realistic customer configuration incident involving a Cisco router. NetPilot will collect network evidence, identify configuration drift, use a local AI model to analyze the incident, generate a recommendation, apply policy controls, obtain explicit engineer approval, revalidate the live device state, perform deterministic Cisco remediation, verify the result, and record an audit event.

The workflow implemented in this lab is:

Discovery → Detection → Evidence → Analysis → Recommendation → Policy Validation → Engineer Approval → Pre-Change Revalidation → Deployment → Post-Change Validation → Audit → Resolution

FDE DESIGN PRINCIPLE

The objective is not simply to make AI generate Cisco commands.

NetPilot must separate:

AI reasoning

from

network execution

The AI is allowed to analyze evidence and recommend actions.

NetPilot's deterministic remediation logic is responsible for constructing the actual Cisco commands from verified network state.

An engineer must approve the change before execution.

NetPilot must then validate the network state again immediately before making the change.

Lab Topology

The existing EVE-NG topology is used.

| Device | Management IP | Role |
| --- | --- | --- |
| R1 | 192.168.1.201 | Cisco Router |
| R2 | 192.168.1.202 | Cisco Router / Incident Device |
| R3 | 192.168.1.203 | Cisco Router |
| NetPilot | Windows Host | AI Network Operations Platform |
| Ollama | 192.168.1.217 | Local AI Server |

R2 will be used as the controlled incident device.

NetPilot v0.8 Architecture

The completed lab contains the following application components:

app/

│

├── cisco_client.py

├── drift_detector.py

├── incident_engine.py

├── ai_incident.py

├── incident_recommendation.py

├── incident_approval.py

├── remediation_planner.py

├── incident_remediation.py

└── audit_log.py

Each component has a specific responsibility.

| Component | Responsibility |
| --- | --- |
| cisco_client.py | Cisco connectivity and CLI execution |
| drift_detector.py | Golden configuration comparison |
| incident_engine.py | Incident creation and evidence collection |
| ai_incident.py | AI root-cause analysis |
| incident_recommendation.py | AI recommendation policy |
| incident_approval.py | Change approval controls |
| remediation_planner.py | Deterministic remediation planning |
| incident_remediation.py | Approval, revalidation, deployment, verification |
| audit_log.py | Change audit recording |

Phase 1: Verify the NetPilot Environment

Before beginning the incident exercise, verify that the NetPilot environment is operational.

Step 1: Open PowerShell

Navigate to the NetPilot project directory.

E:\IT\Cisco Labs\NetPilot

Run:

cd "E:\IT\Cisco Labs\NetPilot"

Step 2: Verify Python

Run:

python --version

Python should return a valid installed version.

Step 3: Verify Flask

Run:

python -c "import flask; print(flask.__version__)"

A valid Flask version should be displayed.

Step 4: Verify Netmiko

Run:

python -c "import netmiko; print(netmiko.__version__)"

A valid Netmiko version should be displayed.

Phase 2: Verify Cisco Connectivity

NetPilot communicates with the Cisco routers using Netmiko.

The configured management addresses are:

R1 = 192.168.1.201

R2 = 192.168.1.202

R3 = 192.168.1.203

Step 5: Verify R2 Connectivity

Run:

python -c "from app.cisco_client import run_command; print(run_command('R2','show ip interface brief'))"

Verify that R2 returns its interface information.

The expected management interface should be operational.

Phase 3: Establish the Golden Configuration

NetPilot uses a golden configuration as the intended state of the network.

For this lab, R2's GigabitEthernet2 interface should contain:

interface GigabitEthernet2

description LINK_TO_R3

This represents the intended configuration.

The golden configuration is used as the source of truth for drift detection.

Phase 4: Introduce the Controlled Customer Incident

This phase intentionally introduces a harmless configuration change.

The objective is to simulate a customer configuration consistency incident.

Step 6: Connect to R2

From the R2 console or SSH session, enter:

enable

configure terminal

Step 7: Modify the Interface Description

Enter:

interface GigabitEthernet2

description CUSTOMER-INCIDENT-TEST

end

This changes the interface description from:

description LINK_TO_R3

to:

description CUSTOMER-INCIDENT-TEST

Step 8: Verify the Change

Run:

show running-config interface GigabitEthernet2

Verify that the output contains:

interface GigabitEthernet2

description CUSTOMER-INCIDENT-TEST

This represents the customer-reported configuration issue.

Phase 5: Detect Configuration Drift

NetPilot must identify that the live Cisco configuration differs from the golden configuration.

Step 9: Run Drift Detection

From the NetPilot PowerShell terminal:

python -c "from app.drift_detector import get_drift_status; print(get_drift_status('R2'))"

Expected result:

status: DRIFT

interface: GigabitEthernet2

expected: description LINK_TO_R3

current: description CUSTOMER-INCIDENT-TEST

The result establishes four important facts:

- Drift exists.

- The affected device is R2.

- The affected interface is GigabitEthernet2.

- The expected configuration is known.

FDE VALIDATION POINT

At this stage, NetPilot should not immediately modify the router.

Detection and remediation are separate operations.

The engineer must first understand:

What changed?

Why did it change?

What is the impact?

What evidence supports the conclusion?

Phase 6: Create the Customer Incident

NetPilot now creates a formal incident record.

The incident contains:

Incident ID

Router

Incident Type

Customer Description

Creation Time

Status

The simulated customer description is:

Customer reports a configuration consistency issue involving R2.

Phase 7: Collect Network Evidence

NetPilot collects evidence from the affected router.

The evidence collection engine gathers:

show ip interface brief

show ip ospf neighbor

show ip ospf interface brief

show ip route

show running-config

show interfaces

NetPilot also collects configuration drift information.

This creates a structured incident evidence package.

Phase 8: Validate Network Health

The evidence should demonstrate that the configuration change is isolated to the interface description.

For this lab, R2 should show:

GigabitEthernet2       10.0.23.1       up       up

OSPF should show R3 as a full neighbor.

Expected relationship:

R2 GigabitEthernet2

│

│ OSPF

│

▼

R3

The routing table should also contain OSPF routes.

FDE DESIGN PRINCIPLE: OBSERVATION VS ROOT CAUSE

A network engineer should distinguish between:

Observed condition

and

Underlying root cause

For example:

Observed:

R2 is unreachable.

does not automatically mean:

Root cause:

The router is powered off.

The second statement requires evidence.

Similarly, in this lab:

Observed:

GigabitEthernet2 description differs from the golden configuration.

is supported by direct configuration evidence.

Phase 9: AI Root-Cause Analysis

NetPilot sends the collected evidence to the local Ollama server.

The configured AI endpoint is:

http://192.168.1.217:11434

The configured model is:

qwen2.5:32b

The AI receives:

- Customer description

- Incident type

- Interface status

- OSPF state

- Routing information

- Configuration drift

- Other collected evidence

AI Safety Rules

The AI analysis must follow these rules:

- Do not invent facts.

- Use only supplied evidence.

- Separate facts from assumptions.

- Do not make network changes.

- Do not blindly generate executable configuration.

- Recommend actions only when evidence supports them.

- Identify customer impact.

- Identify risk.

- Explain the evidence supporting the RCA.

- Require human approval before remediation.

Phase 10: Review the AI Analysis

The expected analysis should identify:

ROOT CAUSE:

Configuration drift on GigabitEthernet2.

Expected confidence:

High

Expected risk:

Low

Expected impact:

Documentation/configuration consistency issue.

The AI should recognize that the interface is still operational and that OSPF is functioning.

FDE DESIGN PRINCIPLE

AI should assist the engineer rather than replace the engineer.

The intended workflow is:

Cisco Evidence

↓

NetPilot

↓

Ollama / Qwen

↓

Root-Cause Analysis

↓

Engineer Review

Not:

Cisco

↓

AI

↓

Automatically Execute CLI

Phase 11: Generate the Remediation Recommendation

NetPilot converts the AI analysis into a structured recommendation.

The recommendation should include:

Decision

Confidence

Risk

Recommended Action

Validation

Policy Reason

For this incident, the recommendation should indicate:

Decision: REVIEW

Risk: Low

Confidence: High

The recommended action is:

Restore the interface description to the golden configuration.

Phase 12: Build a Deterministic Remediation Plan

This is one of the most important safety mechanisms in NetPilot.

The AI suggested a conceptual Cisco configuration.

However, NetPilot does not directly execute the AI-generated configuration.

Instead, NetPilot reads the verified drift information.

The verified interface is:

GigabitEthernet2

The verified golden configuration is:

description LINK_TO_R3

NetPilot deterministically generates:

interface GigabitEthernet2

description LINK_TO_R3

Why Deterministic Remediation Matters

The AI previously produced:

interface <interface-name>

description LINK_TO_R3

This demonstrates why AI-generated commands should not be treated as the source of truth.

NetPilot already knows the actual interface.

Therefore:

Verified network data → command generation

rather than:

LLM text → command execution

This is a critical production engineering principle.

Phase 13: Policy Validation

Before asking for approval, NetPilot validates the remediation plan.

The policy verifies:

- Router is online.

- Drift exists.

- Interface is known.

- Current configuration is known.

- Expected configuration is known.

- Exactly two commands exist.

- Interface command matches the verified interface.

- Configuration command matches the golden configuration.

- Remediation type is currently allowed by policy.

Expected result:

Result: PASSED

Phase 14: Engineer Change Review

NetPilot presents the proposed change to the engineer.

The review should display:

Incident ID

Router

Affected Interface

Current Configuration

Expected Configuration

Proposed Cisco Configuration

Risk

Validation Procedure

For this lab:

CURRENT:

description CUSTOMER-INCIDENT-TEST

EXPECTED:

description LINK_TO_R3

Proposed configuration:

interface GigabitEthernet2

description LINK_TO_R3

Phase 15: Engineer Approval

NetPilot requires the engineer to provide their name.

Example:

Engineer name: Daniel

NetPilot then requests explicit approval:

Approve this Cisco change? (yes/no):

Enter:

yes

NetPilot records:

Approval Status: APPROVED

Engineer: Daniel

Approval Timestamp

Approval Reason

FDE DESIGN PRINCIPLE: HUMAN-IN-THE-LOOP

Network automation should maintain a clear authorization boundary.

The system should be able to:

Detect → Analyze → Recommend

without necessarily being allowed to:

Change

The engineer provides the authorization to cross that boundary.

Phase 16: Pre-Change State Revalidation

This is the final safety control before Cisco configuration is modified.

NetPilot re-reads the live configuration:

show running-config interface GigabitEthernet2

It compares the live configuration against the configuration that was originally reviewed.

Previously verified:

description CUSTOMER-INCIDENT-TEST

Live state:

description CUSTOMER-INCIDENT-TEST

Expected golden state:

description LINK_TO_R3

Because the live state still matches the engineer-reviewed state, remediation is allowed.

Expected result:

PRE-CHANGE VALIDATION PASSED.

Why Pre-Change Revalidation Matters

Consider this real-world scenario:

10:00 AM

NetPilot detects drift.

10:05 AM

Engineer reviews the proposed change.

10:07 AM

Another engineer modifies the router.

10:08 AM

NetPilot attempts the original remediation.

Without revalidation, NetPilot could modify a network state that the engineer never approved.

With revalidation:

Original State

↓

Engineer Review

↓

Live State Recheck

↓

Mismatch

↓

BLOCK CHANGE

This provides a basic optimistic concurrency/change-control mechanism.

Phase 17: Apply the Approved Cisco Configuration

After successful pre-change validation, NetPilot executes the approved commands:

interface GigabitEthernet2

description LINK_TO_R3

The configuration is sent to R2 through Netmiko.

NetPilot should report:

Cisco configuration applied.

Phase 18: Post-Change Validation

NetPilot immediately verifies the configuration.

It executes:

show running-config interface GigabitEthernet2

The expected result is:

interface GigabitEthernet2

description LINK_TO_R3

NetPilot confirms that the expected configuration exists.

Expected result:

Verification successful.

Phase 19: Audit the Change

NetPilot records the remediation event.

The audit record should contain:

Router

Interface

Previous Configuration

New Configuration

Engineer Approval

Result

Expected result:

Audit event recorded.

This creates an operational record of the network change.

Phase 20: Resolve the Incident

After successful post-change validation, NetPilot updates the incident state:

RESOLVED

The final workflow becomes:

OPEN

↓

INVESTIGATED

↓

AI_ANALYZED

↓

REMEDIATION_READY

↓

APPROVED

↓

RESOLVED

Phase 21: Verify Configuration Compliance

The final test confirms that the network has returned to its intended state.

Run:

python -c "from app.drift_detector import get_drift_status; print(get_drift_status('R2'))"

Expected output:

R2 is compliant

and:

{

'status': 'COMPLIANT',

'interface': '',

'expected': '',

'current': '',

'diagnosis': ''

}

This confirms that the configuration drift has been eliminated.

Phase 22: Final Incident Validation

The completed incident should demonstrate:

| Validation | Expected Result |
| --- | --- |
| R2 connectivity | ONLINE |
| Interface discovery | SUCCESS |
| OSPF neighbors | FULL |
| Routing table | Operational |
| Drift detection | DRIFT |
| AI analysis | HIGH confidence |
| Risk | LOW |
| Remediation plan | READY_FOR_APPROVAL |
| Policy validation | PASSED |
| Engineer approval | APPROVED |
| Pre-change validation | PASSED |
| Configuration deployment | SUCCESS |
| Post-change validation | SUCCESS |
| Audit logging | RECORDED |
| Incident | RESOLVED |
| Final drift status | COMPLIANT |

Phase 23: Test the Safety Model

A strong FDE lab should test not only the successful path but also the failure paths.

The following scenarios should be understood.

Scenario A: Router Offline

If R2 is offline:

Router Connectivity

↓

OFFLINE

↓

Remediation BLOCKED

NetPilot must not attempt configuration remediation.

Scenario B: Engineer Rejects Change

If the engineer enters:

no

NetPilot should produce:

CHANGE_REJECTED

No configuration should be sent to Cisco.

Scenario C: Pre-Change State Changed

If another engineer changes the interface after approval but before execution:

Reviewed State

↓

Live State

↓

MISMATCH

↓

CHANGE BLOCKED

NetPilot must not apply the original remediation.

Scenario D: Verification Failure

If the configuration is applied but the expected state cannot be verified:

REMEDIATION_FAILED

The incident must not be falsely marked as resolved.

Final Architecture

The completed NetPilot v0.8 architecture follows this model:

CUSTOMER INCIDENT

│

▼

┌───────────────┐

│   NetPilot    │

│   Detection   │

└───────┬───────┘

│

▼

┌───────────────┐

│    Cisco      │

│    Evidence   │

└───────┬───────┘

│

▼

┌───────────────┐

│ Ollama / Qwen │

│   AI Analysis │

└───────┬───────┘

│

▼

┌───────────────┐

│     Policy    │

│   Validation  │

└───────┬───────┘

│

▼

┌───────────────┐

│    Engineer   │

│    Approval   │

└───────┬───────┘

│

▼

┌───────────────┐

│ Pre-Change    │

│ Revalidation  │

└───────┬───────┘

│

▼

┌───────────────┐

│ Deterministic │

│ Cisco Change  │

└───────┬───────┘

│

▼

┌───────────────┐

│ Post-Change   │

│ Validation    │

└───────┬───────┘

│

▼

┌───────────────┐

│ Audit / Record│

└───────┬───────┘

│

▼

RESOLVED

│

▼

COMPLIANT

Lab Review & Interview Knowledge Check

Q: Why should an AI model not directly control Cisco configuration?

A: LLM output is probabilistic and can contain incorrect, ambiguous, or hallucinated commands. NetPilot should use AI for analysis and recommendations while deterministic application logic generates commands from verified network state.

Q: Why is human approval important?

A: Human approval creates an authorization boundary between recommendation and network modification. This provides operational accountability and prevents autonomous changes from being made without engineer authorization.

Q: What is pre-change state revalidation?

A: It is the process of checking the live network state immediately before applying an approved change to ensure that the device still matches the state that the engineer reviewed.

Q: Why is pre-change revalidation important?

A: Network state can change between detection and execution. Revalidation prevents NetPilot from applying a stale remediation plan to a device whose configuration has changed.

Q: What is the difference between AI-generated remediation and deterministic remediation?

A: AI-generated remediation is produced from probabilistic model output. Deterministic remediation is generated from verified network data and predefined policy rules. NetPilot uses the latter for actual configuration execution.

Q: What evidence did NetPilot use to diagnose this incident?

A: NetPilot used Cisco interface status, OSPF neighbor information, OSPF interface information, routing information, running configuration, detailed interface information, and golden configuration drift data.

Q: Why did NetPilot classify the incident as low risk?

A: The affected interface remained operational, OSPF remained established, routing remained functional, and the detected difference was an interface description rather than a forwarding or routing parameter.

Q: What is the difference between configuration drift and an outage?

A: Configuration drift means the live configuration differs from the intended configuration. An outage means network functionality is impaired or unavailable. Configuration drift can exist without causing an outage.

Q: Why is audit logging important?

A: Audit logging provides traceability for network changes, including what changed, where it changed, who approved it, and whether verification succeeded. This is important for operations, troubleshooting, accountability, and compliance.

Q: What does COMPLIANT mean in this lab?

A: It means NetPilot compared the relevant live configuration against the defined golden configuration and found no active configuration drift.

FDE Skills Demonstrated

By completing this lab, the engineer demonstrates experience with:

- Cisco network troubleshooting

- Network configuration management

- Configuration drift detection

- Incident investigation

- Evidence-driven troubleshooting

- OSPF validation

- Routing-table analysis

- Network automation

- Python

- Netmiko

- REST-style AI integration

- Local LLM deployment

- Ollama

- Qwen

- AI-assisted root-cause analysis

- Risk assessment

- Change management

- Human-in-the-loop automation

- Deterministic remediation

- Pre-change validation

- Post-change validation

- Audit logging

- Customer incident handling

- Production-oriented network safety

- FDE-style operational workflows

Lab Completion Checklist

Environment

☐ NetPilot environment operational
☐ Cisco R1/R2/R3 reachable
☐ Ollama operational
☐ Qwen 2.5 32B available

Detection

☐ Configuration drift introduced
☐ Drift detected on R2
☐ Affected interface identified
☐ Golden configuration identified

Investigation

☐ Cisco evidence collected
☐ Interface health verified
☐ OSPF health verified
☐ Routing health verified

AI Operations

☐ AI RCA generated
☐ Confidence assessed
☐ Risk assessed
☐ Customer summary generated

Change Management

☐ Deterministic remediation generated
☐ Policy validation passed
☐ Engineer approval obtained
☐ Pre-change state revalidated

Deployment

☐ Cisco configuration applied
☐ Post-change verification performed
☐ Audit event recorded
☐ Incident marked RESOLVED

Final Validation

☐ R2 configuration verified
☐ Drift detector reports COMPLIANT

LAB COMPLETION