LAB LESSON 09: NetPilot Enterprise WAN Routing Operations & AI-Assisted Remediation

Cisco Enterprise WAN Routing, Dual-ISP Architecture, OSPF, eBGP, iBGP, Primary/Backup WAN Failover, AI-Assisted Incident Analysis, Deterministic Remediation, Engineer Approval, and Bounded Post-Change Validation

Environment: EVE-NG Cisco IOS Lab + Windows NetPilot Host

Platform: Python / Netmiko / Ollama

AI Model: Qwen 2.5 32B via Ollama

Topology: Five-Router Enterprise WAN — R1, R2, R3, R4, R5

Goal: Build a realistic FDE-style enterprise WAN customer environment in which NetPilot can detect, investigate, analyze, safely remediate, and validate a primary WAN failure.

Lab Overview & Objectives

This lab significantly expands NetPilot beyond the three-router configuration-drift scenario from Lab 8.

The customer now has:

- A headquarters network

- Two WAN edge routers

- Two ISP connections

- An internal core router

- OSPF for internal routing

- eBGP toward both providers

- iBGP between the two customer WAN edges

- Primary/backup WAN routing

- Automated failover

- NetPilot WAN evidence collection

- AI-assisted root-cause analysis

- Deterministic remediation

- Human approval

- Pre-change state revalidation

- Post-change convergence monitoring

The customer requirement is:

Maintain WAN connectivity through a primary ISP while automatically using a secondary ISP when the primary path fails.

NetPilot's role is to provide the operational workflow around that architecture.

The workflow is:

Customer Problem → Discovery → Requirements → Architecture → Deployment → Monitoring → Failure Injection → Evidence → AI Analysis → Recommendation → Policy Validation → Engineer Approval → Pre-Change Revalidation → Deterministic Remediation → Convergence Monitoring → Validation → Resolution

FDE Customer Scenario

Customer

A fictional enterprise customer operates a headquarters network with two Internet service providers.

The customer requires:

- ISP-1 to be the preferred WAN path.

- ISP-2 to provide backup connectivity.

- Internal routing to use OSPF.

- WAN routing to use BGP.

- Automatic failover if ISP-1 becomes unavailable.

- Visibility into WAN health.

- Safe operational remediation.

- Human approval before network changes.

- Evidence-based AI troubleshooting.

- Verification that the network actually converges after a change.

Customer Requirements

| Requirement | Design |
| --- | --- |
| Internal routing | OSPF Area 0 |
| Primary WAN | R1 → ISP-1 |
| Backup WAN | R3 → ISP-2 |
| Customer WAN AS | 65000 |
| ISP-1 AS | 65001 |
| ISP-2 AS | 65002 |
| Customer edge relationship | iBGP |
| ISP relationship | eBGP |
| Primary preference | Local Preference 200 |
| Backup preference | Local Preference 100 |
| Primary default route | OSPF metric 10 |
| Backup default route | OSPF metric 50 |
| Automation | Python / Netmiko |
| AI | Ollama / Qwen 2.5 32B |
| Remediation | Deterministic |
| Authorization | Engineer approval |
| Verification | Automated |

Lab Topology

The final topology is:

ISP-1

AS 65001

|

R4

|

10.0.14.0/30

|

|

R1

HQ-EDGE-1

AS 65000

/        \

/          \

10.0.12.0/30          10.0.13.0/30

/              \

/                \

R2----------------R3

HQ-CORE          HQ-EDGE-2

AS 65000          AS 65000

|

|

10.0.35.0/30

|

|

R5

ISP-2

AS 65002

Management network:

192.168.1.0/24

R1  ──────────────── 192.168.1.201

R2  ──────────────── 192.168.1.202

R3  ──────────────── 192.168.1.203

R4  ──────────────── 192.168.1.204

R5  ──────────────── 192.168.1.205

|

|

NetPilot

192.168.1.217

IP Address Table

| Device | Interface | IP Address | Purpose |
| --- | --- | --- | --- |
| R1 | Loopback0 | 1.1.1.1/32 | Router ID |
| R1 | Gi1 | 10.0.14.1/30 | ISP-1 |
| R1 | Gi2 | 10.0.12.1/30 | R2 |
| R1 | Gi3 | 10.0.13.1/30 | R3 |
| R1 | Gi4 | 192.168.1.201/24 | Management |
| R2 | Loopback0 | 2.2.2.2/32 | Router ID |
| R2 | Loopback100 | 10.10.100.1/24 | Customer network |
| R2 | Gi1 | 10.0.12.2/30 | R1 |
| R2 | Gi2 | 10.0.23.1/30 | R3 |
| R2 | Gi3 | 192.168.1.202/24 | Management |
| R3 | Loopback0 | 3.3.3.3/32 | Router ID |
| R3 | Gi1 | 10.0.23.2/30 | R2 |
| R3 | Gi2 | 10.0.13.2/30 | R1 |
| R3 | Gi3 | 10.0.35.1/30 | ISP-2 |
| R3 | Gi4 | 192.168.1.203/24 | Management |
| R4 | Loopback0 | 4.4.4.4/32 | Router ID |
| R4 | Loopback100 | 198.51.100.1/24 | Simulated Internet |
| R4 | Gi1 | 10.0.14.2/30 | R1 |
| R4 | Gi2 | 192.168.1.204/24 | Management |
| R5 | Loopback0 | 5.5.5.5/32 | Router ID |
| R5 | Loopback100 | 198.51.100.2/24 | Simulated Internet |
| R5 | Gi1 | 10.0.35.2/30 | R3 |
| R5 | Gi2 | 192.168.1.205/24 | Management |
| NetPilot | Host | 192.168.1.217 | Automation/AI |
| Ollama | Host | 192.168.1.217:11434 | Local AI |

EVE-NG Build Sheet

Cisco Image

Use:

Cisco Catalyst 8000V QCOW2

Recommended resources:

| Setting | Value |
| --- | --- |
| vCPU | 2 |
| RAM | 6 GB |
| Console | Telnet or appropriate EVE-NG console |
| Nodes | 5 |
| Total RAM | 30 GB |

This keeps the topology comfortably within the available EVE-NG resources.

EVE-NG Node Names

Create:

R1

R2

R3

R4

R5

EVE-NG Cabling

Connect:

R1 Gi1 ↔ R4 Gi1

R1 Gi2 ↔ R2 Gi1

R1 Gi3 ↔ R3 Gi2

R2 Gi2 ↔ R3 Gi1

R3 Gi3 ↔ R5 Gi1

Management:

R1 Gi4 → Management Cloud

R2 Gi3 → Management Cloud

R3 Gi4 → Management Cloud

R4 Gi2 → Management Cloud

R5 Gi2 → Management Cloud

Phase 1: Configure R1

R1 is the primary headquarters WAN edge.

It connects to:

- R2

- R3

- ISP-1

R1 is the preferred WAN exit.

R1 Complete Routing Configuration

hostname R1

no ip domain lookup

interface Loopback0

description ROUTER_ID

ip address 1.1.1.1 255.255.255.255

interface GigabitEthernet1

description TO_ISP1

ip address 10.0.14.1 255.255.255.252

no shutdown

interface GigabitEthernet2

description TO_R2

ip address 10.0.12.1 255.255.255.252

no shutdown

interface GigabitEthernet3

description TO_R3

ip address 10.0.13.1 255.255.255.252

no shutdown

interface GigabitEthernet4

description MANAGEMENT

ip address 192.168.1.201 255.255.255.0

no shutdown

router ospf 10

router-id 1.1.1.1

network 1.1.1.1 0.0.0.0 area 0

network 10.0.12.0 0.0.0.3 area 0

network 10.0.13.0 0.0.0.3 area 0

router bgp 65000

bgp router-id 1.1.1.1

neighbor 10.0.14.2 remote-as 65001

neighbor 3.3.3.3 remote-as 65000

neighbor 3.3.3.3 update-source Loopback0

address-family ipv4

neighbor 10.0.14.2 activate

neighbor 10.0.14.2 route-map ISP1-IN in

neighbor 3.3.3.3 activate

neighbor 3.3.3.3 next-hop-self

network 10.10.100.0 mask 255.255.255.0

exit-address-family

route-map ISP1-IN permit 10

set local-preference 200

ip route 0.0.0.0 0.0.0.0 10.0.14.2

router ospf 10

default-information originate metric 10

end

write memory

Phase 2: Configure R2

R2 is the internal headquarters core.

R2 does not run BGP.

It learns the default route from OSPF.

R2 Complete Routing Configuration

hostname R2

no ip domain lookup

interface Loopback0

description ROUTER_ID

ip address 2.2.2.2 255.255.255.255

interface Loopback100

description CUSTOMER_NETWORK

ip address 10.10.100.1 255.255.255.0

interface GigabitEthernet1

description TO_R1

ip address 10.0.12.2 255.255.255.252

no shutdown

interface GigabitEthernet2

description TO_R3

ip address 10.0.23.1 255.255.255.252

no shutdown

interface GigabitEthernet3

description MANAGEMENT

ip address 192.168.1.202 255.255.255.0

no shutdown

router ospf 10

router-id 2.2.2.2

network 2.2.2.2 0.0.0.0 area 0

network 10.10.100.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

end

write memory

Phase 3: Configure R3

R3 is the backup headquarters WAN edge.

It connects to:

- R2

- R1

- ISP-2

R3 provides the backup WAN path.

R3 Complete Routing Configuration

hostname R3

no ip domain lookup

interface Loopback0

description ROUTER_ID

ip address 3.3.3.3 255.255.255.255

interface GigabitEthernet1

description TO_R2

ip address 10.0.23.2 255.255.255.252

no shutdown

interface GigabitEthernet2

description TO_R1

ip address 10.0.13.2 255.255.255.252

no shutdown

interface GigabitEthernet3

description TO_ISP2

ip address 10.0.35.1 255.255.255.252

no shutdown

interface GigabitEthernet4

description MANAGEMENT

ip address 192.168.1.203 255.255.255.0

no shutdown

router ospf 10

router-id 3.3.3.3

network 3.3.3.3 0.0.0.0 area 0

network 10.0.13.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

router bgp 65000

bgp router-id 3.3.3.3

neighbor 10.0.35.2 remote-as 65002

neighbor 1.1.1.1 remote-as 65000

neighbor 1.1.1.1 update-source Loopback0

address-family ipv4

neighbor 10.0.35.2 activate

neighbor 10.0.35.2 route-map ISP2-IN in

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 next-hop-self

network 10.10.100.0 mask 255.255.255.0

exit-address-family

route-map ISP2-IN permit 10

set local-preference 100

ip route 0.0.0.0 0.0.0.0 10.0.35.2

router ospf 10

default-information originate metric 50

end

write memory

Phase 4: Configure R4

R4 represents ISP-1.

ISP-1 uses AS 65001.

R4 advertises the simulated Internet prefix.

R4 Complete Routing Configuration

hostname R4

no ip domain lookup

interface Loopback0

description ROUTER_ID

ip address 4.4.4.4 255.255.255.255

interface Loopback100

description SIMULATED_INTERNET

ip address 198.51.100.1 255.255.255.0

interface GigabitEthernet1

description TO_R1

ip address 10.0.14.2 255.255.255.252

no shutdown

interface GigabitEthernet2

description MANAGEMENT

ip address 192.168.1.204 255.255.255.0

no shutdown

router bgp 65001

bgp router-id 4.4.4.4

neighbor 10.0.14.1 remote-as 65000

address-family ipv4

neighbor 10.0.14.1 activate

network 198.51.100.0 mask 255.255.255.0

exit-address-family

end

write memory

Phase 5: Configure R5

R5 represents ISP-2.

ISP-2 uses AS 65002.

R5 also advertises the simulated Internet prefix.

R5 Complete Routing Configuration

hostname R5

no ip domain lookup

interface Loopback0

description ROUTER_ID

ip address 5.5.5.5 255.255.255.255

interface Loopback100

description SIMULATED_INTERNET

ip address 198.51.100.2 255.255.255.0

interface GigabitEthernet1

description TO_R3

ip address 10.0.35.2 255.255.255.252

no shutdown

interface GigabitEthernet2

description MANAGEMENT

ip address 192.168.1.205 255.255.255.0

no shutdown

router bgp 65002

bgp router-id 5.5.5.5

neighbor 10.0.35.1 remote-as 65000

address-family ipv4

neighbor 10.0.35.1 activate

network 198.51.100.0 mask 255.255.255.0

exit-address-family

end

write memory

Management Access

The existing NetPilot management configuration must remain available.

NetPilot uses:

Username: netpilot

Password: Cisco123!

Management addresses:

R1 = 192.168.1.201

R2 = 192.168.1.202

R3 = 192.168.1.203

R4 = 192.168.1.204

R5 = 192.168.1.205

The routers must be reachable from the Windows NetPilot host.

Phase 6: Verify OSPF

From R2:

show ip ospf neighbor

R2 should see:

R1

R3

The expected state is:

FULL

Verify the routing table:

show ip route

R2 should learn the customer WAN default route through R1 under normal operation.

Phase 7: Verify BGP

From the NetPilot directory:

python -m app.bgp_health

Expected:

R1 ONLINE

Neighbor: 10.0.14.2 | PRESENT

Neighbor: 3.3.3.3 | PRESENT

R3 ONLINE

Neighbor: 10.0.35.2 | PRESENT

Neighbor: 1.1.1.1 | PRESENT

For deeper verification:

show ip bgp summary

On R1, the important neighbors are:

10.0.14.2

3.3.3.3

On R3:

10.0.35.2

1.1.1.1

Phase 8: Verify the Primary WAN Path

On R2:

show ip route 0.0.0.0

Expected:

Routing entry for 0.0.0.0/0

Known via "ospf 10", distance 110, metric 10

* 10.0.12.1, from 1.1.1.1, via Gi1

Route metric is 10

This demonstrates:

R2

|

| OSPF

v

R1

|

| eBGP

v

ISP-1

ISP-1 is the preferred WAN path.

Why the Primary Path Is Preferred

R1 receives the ISP-1 route with:

local-preference 200

R3 receives the ISP-2 route with:

local-preference 100

Therefore:

ISP-1 = preferred

ISP-2 = backup

The OSPF default-route metrics reinforce the intended behavior:

R1 = metric 10

R3 = metric 50

Therefore the internal core prefers R1's default route.

Phase 9: Verify Backup WAN Path

Before failure, R3 should also have a functioning BGP relationship with ISP-2.

Verify:

show ip bgp summary

Verify:

show ip route 0.0.0.0

on R2.

The preferred path should still be R1.

Phase 10: Controlled WAN Failure

This is the primary customer incident.

The customer reports:

"The primary WAN connection appears to be unavailable."

We intentionally simulate the failure by shutting down R1's primary ISP interface.

On R1:

configure terminal

interface GigabitEthernet1

shutdown

end

Verify:

show ip interface brief

The interface should show:

GigabitEthernet1    administratively down

Phase 11: Verify WAN Failover

On R2:

show ip route 0.0.0.0

Expected:

Routing entry for 0.0.0.0/0

Known via "ospf 10", distance 110, metric 50

* 10.0.23.2, from 3.3.3.3, via Gi2

Route metric is 50

The path has changed from:

R2 → R1 → ISP-1

to:

R2 → R3 → ISP-2

This proves the WAN failover architecture is operational.

Important Lab 9 Testing Note

Do not use:

R2#ping 198.51.100.1

as the authoritative failover test.

R4 owns:

198.51.100.1/24

and R5 owns:

198.51.100.2/24

but R4 and R5 are not connected to the same Layer-2 segment.

If traffic is sent toward R5 for the 198.51.100.0/24 network, R5 considers that network locally connected and attempts local ARP.

Therefore the simulated Internet ping does not accurately validate WAN failover.

The correct Lab 9 validation is:

OSPF default route

+

next hop

+

route metric

+

BGP state

+

WAN interface state

Phase 12: Restore the Primary WAN

Restore R1:

configure terminal

interface GigabitEthernet1

no shutdown

end

Verify:

show ip interface brief

Then:

show ip bgp summary

Finally:

show ip route 0.0.0.0

The network should eventually return to:

R2 → R1 → ISP-1

Phase 13: NetPilot WAN Evidence Collection

The primary NetPilot WAN evidence module is:

app/wan_incident.py

It collects targeted evidence using one SSH session.

Commands:

show ip interface brief

show ip bgp summary

show ip route 0.0.0.0

NetPilot converts this raw information into structured network facts.

Final app/wan_incident.py

from datetime import datetime

import json

from app.cisco_client import run_commands

WAN_EDGES = {

"R1": {

"primary": {

"interface": "GigabitEthernet1",

"peer": "10.0.14.2",

"role": "PRIMARY_ISP",

},

"backup": {

"interface": "GigabitEthernet3",

"peer": "3.3.3.3",

"role": "BACKUP_IBGP",

},

},

"R3": {

"primary": {

"interface": "GigabitEthernet3",

"peer": "10.0.35.2",

"role": "PRIMARY_BACKUP_ISP",

},

"backup": {

"interface": "GigabitEthernet2",

"peer": "1.1.1.1",

"role": "BACKUP_IBGP",

},

},

}

def parse_interface_status(interface_output, interface_name):

"""

Find the operational status of a specific interface

from show ip interface brief.

"""

for raw_line in interface_output.splitlines():

line = raw_line.strip()

if not line.startswith(interface_name):

continue

fields = line.split()

if len(fields) < 6:

return {

"interface": interface_name,

"status": "UNKNOWN",

"protocol": "UNKNOWN",

}

protocol = fields[-1]

if (

len(fields) >= 7

and fields[-3] == "administratively"

and fields[-2] == "down"

):

status = "administratively down"

else:

status = fields[-2]

return {

"interface": interface_name,

"status": status,

"protocol": protocol,

}

return {

"interface": interface_name,

"status": "NOT_FOUND",

"protocol": "NOT_FOUND",

}

def parse_bgp_neighbor_status(bgp_summary, neighbor_ip):

"""

Determine BGP state for a neighbor.

"""

for raw_line in bgp_summary.splitlines():

line = raw_line.strip()

if not line.startswith(neighbor_ip):

continue

fields = line.split()

if len(fields) < 3:

return {

"neighbor": neighbor_ip,

"state": "UNKNOWN",

"prefixes_received": None,

}

last_field = fields[-1]

if last_field.isdigit():

return {

"neighbor": neighbor_ip,

"state": "ESTABLISHED",

"prefixes_received": int(last_field),

}

return {

"neighbor": neighbor_ip,

"state": last_field,

"prefixes_received": None,

}

return {

"neighbor": neighbor_ip,

"state": "NOT_FOUND",

"prefixes_received": None,

}

def parse_default_route(route_output):

"""

Determine the currently installed default route.

"""

for raw_line in route_output.splitlines():

line = raw_line.strip()

if not line.startswith("*"):

continue

if "via" in line:

fields = line.split()

next_hop = None

interface = None

if "via" in fields:

via_index = fields.index("via")

if via_index + 1 < len(fields):

interface = fields[via_index + 1]

route_text = line[1:].strip()

if "," in route_text:

route_text = route_text.split(",", 1)[0].strip()

if route_text:

next_hop = route_text

return {

"status": "INSTALLED",

"next_hop": next_hop,

"interface": interface,

"route_type": "DYNAMIC",

"raw": line,

}

route_text = line[1:].strip()

if route_text:

if "," in route_text:

route_text = route_text.split(",", 1)[0].strip()

if route_text:

return {

"status": "INSTALLED",

"next_hop": route_text,

"interface": None,

"route_type": "STATIC_OR_DIRECT",

"raw": line,

}

return {

"status": "NOT_INSTALLED",

"next_hop": None,

"interface": None,

"route_type": None,

"raw": "",

}

def determine_path_status(router, evidence):

"""

Determine whether primary or backup WAN path is active.

"""

topology = WAN_EDGES[router]

primary = topology["primary"]

backup = topology["backup"]

interfaces = evidence.get("interfaces", {})

bgp = evidence.get("bgp", {})

default_route = evidence.get("default_route", {})

primary_interface = interfaces.get(

primary["interface"],

{},

)

backup_interface = interfaces.get(

backup["interface"],

{},

)

primary_bgp = bgp.get(

primary["peer"],

{},

)

backup_bgp = bgp.get(

backup["peer"],

{},

)

next_hop = default_route.get("next_hop")

if next_hop == primary["peer"]:

active_path = "PRIMARY"

elif next_hop == backup["peer"]:

active_path = "BACKUP"

else:

active_path = "UNKNOWN"

return {

"active_path": active_path,

"primary_interface_up": (

primary_interface.get("status") == "up"

and primary_interface.get("protocol") == "up"

),

"backup_interface_up": (

backup_interface.get("status") == "up"

and backup_interface.get("protocol") == "up"

),

"primary_bgp_established": (

primary_bgp.get("state") == "ESTABLISHED"

),

"backup_bgp_established": (

backup_bgp.get("state") == "ESTABLISHED"

),

}

def build_structured_wan_state(router, raw_results):

"""

Convert raw Cisco output into structured WAN facts.

"""

topology = WAN_EDGES[router]

interface_output = raw_results.get(

"show ip interface brief",

"",

)

bgp_output = raw_results.get(

"show ip bgp summary",

"",

)

route_output = raw_results.get(

"show ip route 0.0.0.0",

"",

)

primary = topology["primary"]

backup = topology["backup"]

interfaces = {

primary["interface"]: parse_interface_status(

interface_output,

primary["interface"],

),

backup["interface"]: parse_interface_status(

interface_output,

backup["interface"],

),

}

bgp = {

primary["peer"]: parse_bgp_neighbor_status(

bgp_output,

primary["peer"],

),

backup["peer"]: parse_bgp_neighbor_status(

bgp_output,

backup["peer"],

),

}

default_route = parse_default_route(

route_output

)

structured = {

"router": router,

"primary_wan": {

"interface": primary["interface"],

"peer": primary["peer"],

"role": primary["role"],

"interface_state": interfaces[

primary["interface"]

],

"bgp_state": bgp[

primary["peer"]

],

},

"backup_wan": {

"interface": backup["interface"],

"peer": backup["peer"],

"role": backup["role"],

"interface_state": interfaces[

backup["interface"]

],

"bgp_state": bgp[

backup["peer"]

],

},

"default_route": default_route,

}

structured["path_status"] = determine_path_status(

router,

{

"interfaces": interfaces,

"bgp": bgp,

"default_route": default_route,

},

)

return structured

def analyze_bgp_neighbors(router, bgp_summary):

"""

Compare expected BGP neighbors against live BGP state.

"""

topology = WAN_EDGES[router]

neighbors = []

for path_name in ["primary", "backup"]:

neighbor = topology[path_name]["peer"]

result = parse_bgp_neighbor_status(

bgp_summary,

neighbor,

)

result["role"] = path_name

neighbors.append(result)

return neighbors

def collect_wan_evidence(router):

"""

Collect targeted WAN evidence using one SSH session.

"""

if router not in WAN_EDGES:

return {

"router": router,

"status": "UNSUPPORTED",

"evidence": {},

}

print()

print("Collecting WAN evidence from " + router)

commands = [

"show ip interface brief",

"show ip bgp summary",

"show ip route 0.0.0.0",

]

results = run_commands(

router,

commands,

)

if results is None:

return {

"router": router,

"status": "OFFLINE",

"evidence": {},

}

structured_state = build_structured_wan_state(

router,

results,

)

evidence = {

"raw": {

"interfaces": results.get(

"show ip interface brief",

"",

),

"bgp_summary": results.get(

"show ip bgp summary",

"",

),

"default_route": results.get(

"show ip route 0.0.0.0",

"",

),

},

"wan_state": structured_state,

"bgp_health": {

"router": router,

"status": "ONLINE",

"neighbors": analyze_bgp_neighbors(

router,

results.get(

"show ip bgp summary",

"",

),

),

},

}

return {

"router": router,

"status": "COLLECTED",

"collected": datetime.now().isoformat(),

"evidence": evidence,

}

def display_wan_evidence(router):

"""

Display structured WAN evidence.

"""

result = collect_wan_evidence(router)

print()

print("========================================")

print("NETPILOT WAN EVIDENCE")

print("========================================")

print()

print("Router: " + result["router"])

print("Status: " + result["status"])

print()

evidence = result.get(

"evidence",

{},

)

print("STRUCTURED WAN STATE")

print("----------------------------------------")

print(

json.dumps(

evidence.get(

"wan_state",

{},

),

indent=2,

)

)

print()

print("BGP HEALTH")

print("----------------------------------------")

print(

json.dumps(

evidence.get(

"bgp_health",

{},

),

indent=2,

)

)

print()

print("RAW DEFAULT ROUTE")

print("----------------------------------------")

raw = evidence.get(

"raw",

{},

)

print(

raw.get(

"default_route",

"",

)

)

print()

if __name__ == "__main__":

display_wan_evidence("R1")

Phase 14: Test WAN Evidence Collection

Run:

python -m app.wan_incident

NetPilot should identify:

Router: R1

Status: COLLECTED

The structured state should show:

PRIMARY WAN

GigabitEthernet1

10.0.14.2

and:

BACKUP WAN

GigabitEthernet3

3.3.3.3

The default route should identify the currently active path.

Phase 15: AI-Assisted WAN Analysis

NetPilot sends the structured evidence to Ollama.

Ollama:

192.168.1.217:11434

Model:

qwen2.5:32b

The AI is used for:

- Root-cause analysis

- Confidence assessment

- Customer impact

- WAN path analysis

- Recommended corrective action

- Risk assessment

- Validation recommendations

The AI does not execute Cisco commands.

Final app/ai_wan.py

import json

import urllib.request

OLLAMA_HOST = "http://192.168.1.217:11434"

OLLAMA_MODEL = "qwen2.5:32b"

OLLAMA_TIMEOUT = 600

def analyze_wan_incident(router, description, evidence):

"""

Send WAN incident evidence to Ollama for

AI-assisted RCA and recommended remediation.

"""

prompt = f"""

You are NetPilot, an AI-assisted Cisco network operations assistant.

A customer has reported the following WAN incident:

{description}

Affected router:

{router}

Cisco evidence collected by NetPilot:

{json.dumps(evidence, indent=2)}

Analyze the incident using ONLY the Cisco evidence provided.

IMPORTANT OPERATING RULES:

1. Do not invent network facts.

2. Clearly distinguish confirmed facts from assumptions.

3. Identify the most likely root cause.

4. Explain exactly which evidence supports the root cause.

5. Assess customer impact.

6. Evaluate both the primary and backup WAN paths.

7. Determine whether WAN failover appears healthy or unhealthy.

8. Provide a specific RECOMMENDED FIX when the evidence supports one.

9. If the evidence is insufficient to safely recommend a fix, clearly say so.

10. Do not invent interface names, IP addresses, BGP neighbors, routes, or configuration values.

11. Do not generate configuration commands.

12. Do not make network changes.

13. A human network engineer must review and approve any remediation.

14. Prefer the smallest and safest corrective action.

15. Consider whether the problem is caused by an interface failure, BGP failure, routing failure, or configuration problem.

16. If the primary WAN path is unavailable but the backup path is confirmed healthy, explain whether the recommended fix should restore the primary path or correct the backup path.

17. Do not recommend changing a healthy backup path merely because it is not currently preferred.

18. If an interface is administratively down and the evidence clearly identifies it as the failed primary path, the recommended fix should be to restore the interface.

19. If BGP is down because of a neighbor/session problem, recommend restoring the BGP session rather than making unrelated configuration changes.

20. If the evidence does not prove the exact failure, recommend the minimum additional validation required before remediation.

Your response MUST use exactly this structure:

ROOT CAUSE:

State the most likely root cause.

CONFIDENCE:

State HIGH, MEDIUM, or LOW and explain why.

EVIDENCE:

List the specific Cisco evidence supporting the diagnosis.

CUSTOMER IMPACT:

Explain the likely customer/business impact.

WAN PATH STATUS:

Describe the primary WAN path and backup WAN path.

RECOMMENDED FIX:

State the specific corrective action the network engineer should take.

FIX RATIONALE:

Explain why this is the safest and most appropriate corrective action based on the evidence.

REMEDIATION READINESS:

State one of:

READY_FOR_ENGINEER_APPROVAL

NOT_READY_FOR_REMEDIATION

Explain what evidence supports that decision.

RISK:

State LOW, MEDIUM, or HIGH and explain the operational risk.

VALIDATION:

List the specific checks the engineer should perform after applying the fix.

CUSTOMER SUMMARY:

Provide a concise customer-friendly explanation of the issue and recommended resolution.

"""

payload = json.dumps(

{

"model": OLLAMA_MODEL,

"prompt": prompt,

"stream": False,

}

).encode("utf-8")

request = urllib.request.Request(

OLLAMA_HOST + "/api/generate",

data=payload,

headers={

"Content-Type": "application/json"

},

method="POST",

)

try:

with urllib.request.urlopen(

request,

timeout=OLLAMA_TIMEOUT,

) as response:

result = json.loads(

response.read().decode("utf-8")

)

return result.get(

"response",

"Ollama returned no analysis.",

)

except Exception as error:

return (

"AI analysis unavailable: "

+ str(error)

)

if __name__ == "__main__":

from app.wan_incident import collect_wan_evidence

router = "R1"

description = (

"Customer reports a WAN connectivity issue "

"and possible loss of the primary ISP path."

)

result = collect_wan_evidence(router)

analysis = analyze_wan_incident(

router,

description,

result,

)

print()

print("========================================")

print("NETPILOT AI WAN ANALYSIS")

print("========================================")

print()

print(analysis)

Phase 16: Healthy WAN AI Test

Run:

python -m app.ai_wan

with R1 healthy.

The AI should recognize:

Primary interface = UP

Primary BGP = ESTABLISHED

Backup BGP = ESTABLISHED

Default route = PRIMARY

The AI should not claim that an outage exists.

Expected remediation readiness:

NOT_READY_FOR_REMEDIATION

This is important.

NetPilot must not manufacture an incident simply because the engineer asked the AI to analyze one.

Phase 17: Simulate the Customer WAN Incident

On R1:

configure terminal

interface GigabitEthernet1

shutdown

end

Verify:

show ip interface brief

Expected:

GigabitEthernet1    administratively down

Verify BGP:

show ip bgp summary

The ISP-1 neighbor should no longer be Established.

Verify R2:

show ip route 0.0.0.0

R2 should now use R3:

10.0.23.2

Phase 18: Run NetPilot WAN Evidence

Run:

python -m app.wan_incident

NetPilot should identify:

Primary interface:

GigabitEthernet1

administratively down

Primary BGP:

10.0.14.2

not established

Backup BGP:

3.3.3.3

established

Default route:

backup

Phase 19: Run AI RCA

Run:

python -m app.ai_wan

The expected AI diagnosis is approximately:

ROOT CAUSE:

Primary WAN interface GigabitEthernet1 is administratively down.

CONFIDENCE:

HIGH

CUSTOMER IMPACT:

Primary ISP connectivity is unavailable, but the backup WAN path remains available.

WAN PATH STATUS:

Primary path unavailable.

Backup path operational.

RECOMMENDED FIX:

Restore GigabitEthernet1.

REMEDIATION READINESS:

READY_FOR_ENGINEER_APPROVAL

RISK:

LOW

The exact wording will vary because Qwen is an LLM.

The important requirement is that the diagnosis must agree with the verified Cisco evidence.

Phase 20: Deterministic Remediation

NetPilot does not allow Qwen to create the executable Cisco command.

Instead, NetPilot already knows:

Router = R1

Primary interface =

GigabitEthernet1

Interface state =

administratively down

Therefore deterministic logic creates:

interface GigabitEthernet1

no shutdown

This is significantly safer than allowing the LLM to invent the interface.

Deterministic Remediation Principle

The architecture is:

Cisco Evidence

|

v

Verified State

|

v

Deterministic Policy

|

v

Cisco Command

Not:

Cisco Evidence

|

v

LLM

|

v

Arbitrary Cisco Command

Phase 21: Safety Rules

NetPilot will only automatically prepare this remediation when:

- The router is supported.

- The expected primary interface is known.

- The expected primary BGP peer is known.

- The interface is administratively down.

- The failure is not merely an unexplained physical/protocol down state.

- The remediation matches the approved scenario.

- Engineer approval is obtained.

If the interface is merely:

down/down

NetPilot should not automatically issue:

no shutdown

because the cause has not been established.

This is an important production safety boundary.

Phase 22: Engineer Approval

NetPilot displays the remediation plan.

Example:

========================================

NETPILOT WAN REMEDIATION

========================================

Router: R1

Primary Interface:

GigabitEthernet1

Primary BGP Peer:

10.0.14.2

Current Interface State:

administratively down

Proposed Change:

interface GigabitEthernet1

no shutdown

This change requires engineer approval.

Engineer name:

Enter:

Daniel

NetPilot then asks:

Approve this Cisco change? (yes/no):

Enter:

yes

Phase 23: Pre-Change Revalidation

Before NetPilot changes R1, it reads the device again.

The expected state remains:

GigabitEthernet1

administratively down

If the live state matches the engineer-reviewed state:

PRE-CHANGE VALIDATION PASSED

The change is allowed.

Why This Matters

Consider:

10:00

NetPilot detects failure

10:05

Engineer approves remediation

10:07

Another engineer restores the interface

10:08

NetPilot executes the old remediation

Without revalidation, NetPilot is acting on stale information.

With revalidation:

Reviewed State

|

v

Live State Check

|

v

Mismatch

|

v

BLOCK CHANGE

This is a basic optimistic concurrency control mechanism for network automation.

Phase 24: Apply Remediation

The approved command is sent through Netmiko:

interface GigabitEthernet1

no shutdown

NetPilot reports:

Cisco configuration applied.

Phase 25: Bounded Post-Change Validation

A key feature added in Lab 9 is that NetPilot does not assume the network will recover instantly.

Different network layers can converge at different speeds.

The observed successful remediation was:

Attempt 1

Interface DOWN

BGP DOWN

Default route BACKUP

Attempt 2

Interface DOWN

BGP DOWN

Default route BACKUP

Attempt 3

Interface UP

BGP DOWN

Default route PRIMARY

Attempt 4

Interface UP

BGP DOWN

Default route PRIMARY

Attempt 5

Interface UP

BGP DOWN

Default route PRIMARY

Attempt 6

Interface UP

BGP UP

Default route PRIMARY

This is an excellent real-world operational behavior.

The physical interface recovered first.

The routing process restored the preferred default path.

BGP took additional time to establish.

NetPilot waits for convergence rather than declaring failure prematurely.

Lab 9 Validation Policy

The remediation validation policy is:

Validation attempts: 6

Wait between attempts: 5 seconds

Maximum convergence window:

25 seconds after the first validation check

NetPilot checks:

1. Primary interface UP

2. Primary BGP ESTABLISHED

3. Primary default route installed

Only when all three conditions are true does NetPilot declare:

RESOLVED

Phase 26: Final WAN State

After successful remediation:

R1:

GigabitEthernet1 = up/up

BGP:

10.0.14.2 = ESTABLISHED

Default route:

0.0.0.0/0

via 10.0.14.2

R1 is again the primary WAN path.

Phase 27: Final Configuration Verification

On R1:

show ip interface brief

Expected:

GigabitEthernet1    up    up

Run:

show ip bgp summary

Expected:

10.0.14.2    ESTABLISHED

Run:

show ip route 0.0.0.0

Expected:

* 10.0.14.2

Phase 28: Final NetPilot Verification

Run:

python -m app.wan_incident

NetPilot should report:

PRIMARY WAN

Interface: GigabitEthernet1

Interface: UP

BGP: ESTABLISHED

BACKUP WAN

Interface: GigabitEthernet3

BGP: ESTABLISHED

ACTIVE PATH:

PRIMARY

Phase 29: Final Incident Result

The completed WAN incident demonstrates:

CUSTOMER REPORT

|

v

WAN FAILURE

|

v

CISCO EVIDENCE

|

v

STRUCTURED WAN STATE

|

v

OLLAMA / QWEN

|

v

ROOT-CAUSE ANALYSIS

|

v

RECOMMENDATION

|

v

SAFETY POLICY

|

v

ENGINEER APPROVAL

|

v

PRE-CHANGE REVALIDATION

|

v

DETERMINISTIC REMEDIATION

|

v

CONVERGENCE MONITORING

|

v

POST-CHANGE VALIDATION

|

v

RESOLVED

Final app/cisco_client.py

from netmiko import ConnectHandler

import time

DEVICES = {

"R1": {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "Cisco123!",

"port": 22,

},

"R2": {

"device_type": "cisco_ios",

"host": "192.168.1.202",

"username": "netpilot",

"password": "Cisco123!",

"port": 22,

},

"R3": {

"device_type": "cisco_ios",

"host": "192.168.1.203",

"username": "netpilot",

"password": "Cisco123!",

"port": 22,

},

"R4": {

"device_type": "cisco_ios",

"host": "192.168.1.204",

"username": "netpilot",

"password": "Cisco123!",

"port": 22,

},

"R5": {

"device_type": "cisco_ios",

"host": "192.168.1.205",

"username": "netpilot",

"password": "Cisco123!",

"port": 22,

},

}

def connect_to_router(device_name):

"""

Establish an SSH connection to a Cisco device.

Retries up to three times.

"""

if device_name not in DEVICES:

print("Unknown device: " + device_name)

return None

device = DEVICES[device_name]

print(

"Connecting to "

+ device_name

+ "..."

)

for attempt in range(1, 4):

try:

connection = ConnectHandler(

**device

)

print(

device_name

+ " connected!"

)

return connection

except Exception as error:

print(

device_name

+ " connection attempt "

+ str(attempt)

+ " failed."

)

if attempt < 3:

time.sleep(2)

else:

print(

device_name

+ " could not be reached."

)

print(error)

return None

def run_command(device_name, command):

"""

Run one command using one SSH session.

"""

connection = connect_to_router(

device_name

)

if connection is None:

return None

try:

output = connection.send_command(

command,

read_timeout=30,

)

return output

except Exception as error:

print(

"Command failed on "

+ device_name

+ ": "

+ str(error)

)

return None

finally:

connection.disconnect()

def run_commands(device_name, commands):

"""

Run multiple commands using one SSH session.

"""

connection = connect_to_router(

device_name

)

if connection is None:

return None

results = {}

try:

for command in commands:

print(

"Running "

+ command

+ "..."

)

try:

results[command] = (

connection.send_command(

command,

read_timeout=30,

)

)

except Exception as error:

results[command] = (

"COMMAND_FAILED: "

+ str(error)

)

print(

"Command failed: "

+ command

)

return results

finally:

connection.disconnect()

def run_config(device_name, commands):

"""

Apply configuration commands using one SSH session.

"""

connection = connect_to_router(

device_name

)

if connection is None:

return None

try:

output = connection.send_config_set(

commands

)

return output

except Exception as error:

print(

"Configuration failed on "

+ device_name

+ ": "

+ str(error)

)

return None

finally:

connection.disconnect()

Final app/bgp_health.py

from app.cisco_client import run_command

BGP_DEVICES = {

"R1": {

"neighbors": [

"10.0.14.2",

"3.3.3.3",

]

},

"R3": {

"neighbors": [

"10.0.35.2",

"1.1.1.1",

]

},

}

def get_bgp_summary(router):

if router not in BGP_DEVICES:

return {

"router": router,

"status": "UNSUPPORTED",

"output": "",

"neighbors": [],

}

output = run_command(

router,

"show ip bgp summary",

)

if output is None:

return {

"router": router,

"status": "OFFLINE",

"output": "",

"neighbors": [],

}

neighbors = []

for neighbor in BGP_DEVICES[router]["neighbors"]:

if neighbor in output:

neighbors.append(

{

"neighbor": neighbor,

"status": "PRESENT",

}

)

else:

neighbors.append(

{

"neighbor": neighbor,

"status": "NOT_FOUND",

}

)

return {

"router": router,

"status": "ONLINE",

"output": output,

"neighbors": neighbors,

}

def display_bgp_health(router):

result = get_bgp_summary(router)

print()

print("========================================")

print("NETPILOT BGP HEALTH")

print("========================================")

print()

print(

"Router: "

+ result["router"]

)

print(

"Status: "

+ result["status"]

)

print()

for neighbor in result["neighbors"]:

print(

"Neighbor: "

+ neighbor["neighbor"]

+ " | "

+ neighbor["status"]

)

print()

if __name__ == "__main__":

display_bgp_health("R1")

display_bgp_health("R3")

Final app/wan_remediation.py

from datetime import datetime

from app.cisco_client import (

run_command,

run_config,

)

WAN_REMEDIATION = {

"R1": {

"primary_interface": "GigabitEthernet1",

"primary_peer": "10.0.14.2",

},

"R3": {

"primary_interface": "GigabitEthernet3",

"primary_peer": "10.0.35.2",

},

}

VALIDATION_ATTEMPTS = 6

VALIDATION_WAIT_SECONDS = 5

def get_interface_state(router, interface):

output = run_command(

router,

"show ip interface brief",

)

if output is None:

return {

"status": "UNKNOWN",

"protocol": "UNKNOWN",

}

for raw_line in output.splitlines():

line = raw_line.strip()

if not line.startswith(interface):

continue

fields = line.split()

if len(fields) < 6:

return {

"status": "UNKNOWN",

"protocol": "UNKNOWN",

}

protocol = fields[-1]

if (

len(fields) >= 7

and fields[-3] == "administratively"

and fields[-2] == "down"

):

status = "administratively down"

else:

status = fields[-2]

return {

"status": status,

"protocol": protocol,

}

return {

"status": "NOT_FOUND",

"protocol": "NOT_FOUND",

}

def get_bgp_state(router, peer):

output = run_command(

router,

"show ip bgp summary",

)

if output is None:

return "UNKNOWN"

for raw_line in output.splitlines():

line = raw_line.strip()

if not line.startswith(peer):

continue

fields = line.split()

if len(fields) < 3:

return "UNKNOWN"

if fields[-1].isdigit():

return "ESTABLISHED"

return fields[-1]

return "NOT_FOUND"

def get_primary_default_route(

router,

primary_peer,

):

output = run_command(

router,

"show ip route 0.0.0.0",

)

if output is None:

return False

for raw_line in output.splitlines():

line = raw_line.strip()

if not line.startswith("*"):

continue

if primary_peer in line:

return True

return False

def get_validation_state(router):

topology = WAN_REMEDIATION[router]

interface = topology[

"primary_interface"

]

peer = topology[

"primary_peer"

]

interface_state = get_interface_state(

router,

interface,

)

bgp_state = get_bgp_state(

router,

peer,

)

primary_route = (

get_primary_default_route(

router,

peer,

)

)

interface_up = (

interface_state["status"] == "up"

and interface_state["protocol"] == "up"

)

bgp_established = (

bgp_state == "ESTABLISHED"

)

return {

"interface_up": interface_up,

"bgp_established": bgp_established,

"primary_default_route": primary_route,

}

def build_remediation_plan(router):

if router not in WAN_REMEDIATION:

return {

"status": "BLOCKED",

"reason": "Unsupported router.",

}

topology = WAN_REMEDIATION[router]

interface = topology[

"primary_interface"

]

peer = topology[

"primary_peer"

]

state = get_interface_state(

router,

interface,

)

if state["status"] == "administratively down":

commands = [

"interface " + interface,

"no shutdown",

]

return {

"status": "READY_FOR_APPROVAL",

"router": router,

"interface": interface,

"peer": peer,

"current_state": state,

"commands": commands,

}

if state["status"] == "down":

return {

"status": "BLOCKED",

"router": router,

"interface": interface,

"peer": peer,

"reason": (

"Interface is down but not "

"administratively down. "

"Automatic no shutdown is blocked."

),

}

if state["status"] == "up":

return {

"status": "NOT_REQUIRED",

"router": router,

"interface": interface,

"peer": peer,

"reason": "Primary interface is already up.",

}

return {

"status": "BLOCKED",

"router": router,

"interface": interface,

"peer": peer,

"reason": "Interface state could not be safely determined.",

}

def request_engineer_approval(plan):

print()

print("========================================")

print("NETPILOT ENGINEER APPROVAL")

print("========================================")

print()

print(

"Router: "

+ plan["router"]

)

print(

"Primary Interface: "

+ plan["interface"]

)

print(

"Primary BGP Peer: "

+ plan["peer"]

)

print()

print("Current Interface State:")

print(

plan["current_state"]

)

print()

print("Proposed Cisco Configuration:")

for command in plan["commands"]:

print(command)

print()

engineer = input(

"Engineer name: "

).strip()

approval = input(

"Approve this Cisco change? (yes/no): "

).strip().lower()

if approval != "yes":

return {

"approved": False,

"engineer": engineer,

"timestamp": datetime.now().isoformat(),

}

return {

"approved": True,

"engineer": engineer,

"timestamp": datetime.now().isoformat(),

}

def pre_change_revalidation(

router,

reviewed_state,

):

current_state = get_interface_state(

router,

reviewed_state["interface"],

)

if (

current_state["status"]

!= reviewed_state["status"]

or current_state["protocol"]

!= reviewed_state["protocol"]

):

return {

"passed": False,

"reason": "Live state no longer matches reviewed state.",

"current_state": current_state,

"reviewed_state": reviewed_state,

}

return {

"passed": True,

"reason": "Live state matches reviewed state.",

"current_state": current_state,

"reviewed_state": reviewed_state,

}

def validate_after_change(router):

validation_history = []

for attempt in range(

1,

VALIDATION_ATTEMPTS + 1,

):

state = get_validation_state(

router

)

record = {

"attempt": attempt,

"timestamp": datetime.now().isoformat(),

"state": state,

}

validation_history.append(

record

)

print()

print(

"Validation attempt "

+ str(attempt)

)

print(state)

if (

state["interface_up"]

and state["bgp_established"]

and state["primary_default_route"]

):

return {

"status": "RESOLVED",

"attempts": validation_history,

}

if attempt < VALIDATION_ATTEMPTS:

import time

time.sleep(

VALIDATION_WAIT_SECONDS

)

return {

"status": "VALIDATION_FAILED",

"attempts": validation_history,

}

def remediate_wan_incident(router):

incident_id = datetime.now().strftime(

"%Y%m%d%H%M%S"

)

print()

print("========================================")

print("NETPILOT WAN REMEDIATION")

print("========================================")

print()

print(

"Incident ID: "

+ incident_id

)

plan = build_remediation_plan(

router

)

if plan["status"] != "READY_FOR_APPROVAL":

print()

print(

"REMEDIATION BLOCKED"

)

print(

plan

)

return {

"incident_id": incident_id,

"status": "BLOCKED",

"plan": plan,

}

approval = request_engineer_approval(

plan

)

if not approval["approved"]:

print()

print(

"CHANGE REJECTED"

)

return {

"incident_id": incident_id,

"status": "CHANGE_REJECTED",

"approval": approval,

"plan": plan,

}

reviewed_state = {

"interface": plan["interface"],

"status": plan[

"current_state"

]["status"],

"protocol": plan[

"current_state"

]["protocol"],

}

pre_change = pre_change_revalidation(

router,

reviewed_state,

)

print()

print(

"PRE-CHANGE VALIDATION"

)

print(

pre_change

)

if not pre_change["passed"]:

return {

"incident_id": incident_id,

"status": "PRE_CHANGE_VALIDATION_FAILED",

"approval": approval,

"plan": plan,

"pre_change": pre_change,

}

output = run_config(

router,

plan["commands"],

)

if output is None:

return {

"incident_id": incident_id,

"status": "DEPLOYMENT_FAILED",

"approval": approval,

"plan": plan,

"pre_change": pre_change,

}

print()

print(

"Cisco configuration applied."

)

validation = validate_after_change(

router

)

print()

print(

"FINAL VALIDATION"

)

print(

validation

)

if validation["status"] == "RESOLVED":

print()

print(

"========================================"

)

print(

"WAN REMEDIATION SUCCESSFUL"

)

print(

"========================================"

)

print()

print(

"Incident ID: "

+ incident_id

)

print(

"Status: RESOLVED"

)

return {

"incident_id": incident_id,

"status": "RESOLVED",

"approval": approval,

"plan": plan,

"pre_change": pre_change,

"validation": validation,

}

print()

print(

"WAN REMEDIATION VALIDATION FAILED"

)

return {

"incident_id": incident_id,

"status": "VALIDATION_FAILED",

"approval": approval,

"plan": plan,

"pre_change": pre_change,

"validation": validation,

}

if __name__ == "__main__":

result = remediate_wan_incident(

"R1"

)

print()

print(

"FINAL RESULT"

)

print(result)

Phase 30: Execute the Final Controlled Incident

Put R1's primary WAN interface into failure:

configure terminal

interface GigabitEthernet1

shutdown

end

Then from Windows:

python -m app.wan_remediation

Enter:

Daniel

Then:

yes

NetPilot should:

- Detect the administratively down interface.

- Build the deterministic remediation.

- Request engineer approval.

- Revalidate the live state.

- Apply no shutdown.

- Monitor convergence.

- Verify BGP.

- Verify the primary default route.

- Report RESOLVED.

Phase 31: Final Expected Successful Result

The final successful run should resemble:

Validation attempt 1

{

'interface_up': False,

'bgp_established': False,

'primary_default_route': False

}

Validation attempt 2

{

'interface_up': False,

'bgp_established': False,

'primary_default_route': False

}

Validation attempt 3

{

'interface_up': True,

'bgp_established': False,

'primary_default_route': True

}

Validation attempt 4

{

'interface_up': True,

'bgp_established': False,

'primary_default_route': True

}

Validation attempt 5

{

'interface_up': True,

'bgp_established': False,

'primary_default_route': True

}

Validation attempt 6

{

'interface_up': True,

'bgp_established': True,

'primary_default_route': True

}

Final:

WAN REMEDIATION SUCCESSFUL

Status: RESOLVED

Phase 32: Final Network State

The final state should be:

R1 Gi1

UP/UP

R1 → ISP-1

BGP ESTABLISHED

R1 → R3

iBGP ESTABLISHED

R2 → R1

OSPF default route

R3 → ISP-2

BGP ESTABLISHED

Primary WAN

ACTIVE

Backup WAN

READY

FDE Validation: Customer Outcome

The customer's original requirement was:

Maintain WAN connectivity if the primary ISP fails.

The lab demonstrated:

Primary ISP failure

|

v

BGP failure

|

v

OSPF removes preferred default

|

v

Backup default becomes active

|

v

Customer traffic uses ISP-2

When the primary WAN was restored:

Primary interface restored

|

v

BGP reconverges

|

v

Primary default route returns

|

v

Customer traffic returns to ISP-1

FDE Design Principle: Detection Is Not Remediation

NetPilot does not treat every detected condition as permission to make a change.

The architecture separates:

Detection

from:

Analysis

from:

Recommendation

from:

Authorization

from:

Execution

from:

Validation

This separation is one of the most important architectural concepts demonstrated by Lab 9.

FDE Design Principle: AI Is Not the Source of Truth

The AI is responsible for reasoning over evidence.

The source of truth for execution is:

Verified Cisco state

+

Topology knowledge

+

Deterministic policy

Therefore:

Qwen:

"What is probably wrong?"

NetPilot:

"What does the network actually report?"

Deterministic logic:

"What command is permitted?"

Engineer:

"Is this change authorized?"

Netmiko:

"Execute the approved change."

Validation:

"Did the network actually recover?"

Safety Scenario A: Primary Interface Already Up

If NetPilot detects:

GigabitEthernet1 = up/up

the remediation should not execute.

Expected:

NOT_REQUIRED

Safety Scenario B: Interface Down but Not Administratively Down

If NetPilot sees:

down/down

rather than:

administratively down

automatic remediation should be blocked.

Reason:

The interface may have a physical, Layer-2, transceiver, cabling, or protocol problem.

NetPilot should not blindly issue:

no shutdown

Safety Scenario C: Engineer Rejects the Change

Enter:

no

Expected:

CHANGE REJECTED

No Cisco configuration should be sent.

Safety Scenario D: Pre-Change State Changed

If another engineer restores the interface before NetPilot executes the approved change:

Reviewed state:

administratively down

Live state:

up/up

NetPilot must return:

PRE_CHANGE_VALIDATION_FAILED

and make no change.

Safety Scenario E: BGP Does Not Converge

The interface may recover while BGP remains down.

NetPilot should not immediately report success.

Instead:

Interface = UP

BGP = DOWN

Default = PRIMARY

The system continues validation until:

Interface = UP

BGP = ESTABLISHED

Default = PRIMARY

or the bounded validation window expires.

This prevents false-positive resolution.

Lab 9 Architecture

CUSTOMER

|

v

Customer Incident

|

v

NetPilot Detection

|

v

Cisco Evidence

|

v

Structured WAN State

|

v

Ollama / Qwen

|

v

AI Root-Cause Analysis

|

v

Recommendation / Policy

|

v

Engineer Approval

|

v

Pre-Change Revalidation

|

v

Deterministic Remediation

|

v

Cisco / Netmiko Execution

|

v

Bounded Convergence Monitoring

|

v

Post-Change Validation

|

v

RESOLVED

NetPilot Lab 9 Components

The completed Lab 9 functionality consists of:

app/

│

├── cisco_client.py

├── bgp_health.py

├── wan_incident.py

├── ai_wan.py

└── wan_remediation.py

Lab 8 components remain part of the overall NetPilot platform:

app/

│

├── cisco_client.py

├── drift_detector.py

├── incident_engine.py

├── ai_incident.py

├── incident_recommendation.py

├── incident_approval.py

├── remediation_planner.py

├── incident_remediation.py

├── wan_incident.py

├── ai_wan.py

└── wan_remediation.py

Lab 9 Operational Workflow

The complete customer engagement is:

1. Customer reports WAN issue

2. NetPilot identifies affected WAN edge

3. NetPilot collects Cisco evidence

4. NetPilot structures:

- interface state

- BGP state

- default route

- active WAN path

5. Ollama analyzes the evidence

6. NetPilot evaluates the recommendation

7. Engineer reviews the proposed action

8. Engineer approves

9. NetPilot revalidates live state

10. NetPilot constructs deterministic Cisco commands

11. Netmiko applies the change

12. NetPilot monitors convergence

13. NetPilot verifies:

- interface

- BGP

- routing

14. Incident becomes RESOLVED

Final Validation Checklist

EVE-NG

☐ R1 created

☐ R2 created

☐ R3 created

☐ R4 created

☐ R5 created

☐ Cisco Catalyst 8000V installed

☐ Five routers operational

☐ Management network connected

☐ All interfaces connected correctly

IP Addressing

☐ R1 management = 192.168.1.201

☐ R2 management = 192.168.1.202

☐ R3 management = 192.168.1.203

☐ R4 management = 192.168.1.204

☐ R5 management = 192.168.1.205

☐ NetPilot = 192.168.1.217

OSPF

☐ R1 OSPF established

☐ R2 OSPF established

☐ R3 OSPF established

☐ R2 sees R1

☐ R2 sees R3

☐ Default route learned through R1

☐ Backup default route learned through R3

BGP

☐ R1 ↔ R4 eBGP established

☐ R1 ↔ R3 iBGP established

☐ R3 ↔ R5 eBGP established

☐ R4 advertises 198.51.100.0/24

☐ R5 advertises 198.51.100.0/24

☐ R1 prefers ISP-1

☐ R3 provides ISP-2 backup

Failover

☐ R1 Gi1 shutdown

☐ ISP-1 BGP session fails

☐ R2 default route moves to R3

☐ Default route metric changes from 10 to 50

☐ Backup WAN remains operational

☐ R1 Gi1 restored

☐ Primary route returns

NetPilot

☐ Cisco connectivity works

☐ BGP health works

☐ WAN evidence collection works

☐ Structured WAN state generated

☐ Primary WAN identified

☐ Backup WAN identified

☐ Default route identified

☐ AI WAN analysis works

☐ Ollama connected

☐ Qwen 2.5 32B works

AI Operations

☐ Customer description provided

☐ Cisco evidence provided

☐ Root cause generated

☐ Confidence generated

☐ Customer impact generated

☐ WAN path status generated

☐ Recommended fix generated

☐ Risk generated

☐ Validation generated

☐ AI does not execute commands

Safety

☐ Deterministic remediation used

☐ Interface verified

☐ BGP peer verified

☐ Administrative shutdown verified

☐ Engineer approval required

☐ Pre-change validation performed

☐ Unauthorized changes blocked

☐ Unsupported interface states blocked

Remediation

☐ no shutdown generated deterministically

☐ Engineer approved

☐ Cisco configuration applied

☐ Interface recovered

☐ BGP recovered

☐ Primary default route recovered

☐ Convergence monitored

☐ Incident resolved only after all checks passed

Lab 9 Completion Criteria

Lab 9 is considered complete when NetPilot can demonstrate:

CUSTOMER WAN INCIDENT

↓

PRIMARY WAN FAILURE

↓

CISCO EVIDENCE

↓

AI RCA

↓

SAFE RECOMMENDATION

↓

ENGINEER APPROVAL

↓

PRE-CHANGE VALIDATION

↓

DETERMINISTIC REMEDIATION

↓

BOUNDED CONVERGENCE MONITORING

↓

BGP RECOVERY

↓

PRIMARY ROUTE RECOVERY

↓

POST-CHANGE VALIDATION

↓

RESOLVED

FDE Skills Demonstrated

By completing Lab 9, the engineer demonstrates experience with:

- Enterprise WAN architecture

- Dual-ISP design

- Cisco IOS

- Cisco Catalyst 8000V

- OSPF

- eBGP

- iBGP

- BGP local preference

- Route selection

- Default-route origination

- Primary/backup routing

- WAN failover

- Routing-table analysis

- BGP troubleshooting

- Network evidence collection

- Python

- Netmiko

- Structured network-state modeling

- AI-assisted troubleshooting

- Ollama

- Qwen 2.5

- Human-in-the-loop automation

- Deterministic remediation

- Pre-change validation

- Post-change validation

- Convergence monitoring

- Operational safety

- Customer incident handling

- Production-oriented automation

- FDE-style troubleshooting

- Customer-impact analysis

- Change authorization

- Network reliability engineering

Lab Review & Interview Knowledge Check

Q: Why use both OSPF and BGP?

Answer: OSPF provides internal enterprise routing while BGP provides controlled routing between the customer WAN edges and external service providers.

Q: Why is R1 the preferred WAN edge?

Answer: R1 receives the ISP-1 route with a higher local preference of 200, while R3 receives the ISP-2 route with local preference 100. R1 also originates the lower-metric OSPF default route.

Q: What happens when R1's ISP-1 connection fails?

Answer: R1's eBGP session with ISP-1 fails, R1's preferred default route disappears, and R2 learns the backup OSPF default through R3.

Q: Why does R2 not run BGP?

Answer: R2 represents the internal headquarters core. The WAN routing function is intentionally concentrated on the edge routers, while OSPF provides internal reachability.

Q: Why does NetPilot collect structured WAN state?

Answer: Structured state makes the operational logic deterministic. Instead of relying on an LLM to interpret arbitrary CLI text during execution, NetPilot converts verified Cisco output into known facts such as interface state, BGP state, and active default route.

Q: Why shouldn't Qwen generate the actual Cisco commands?

Answer: LLM output is probabilistic and may contain incorrect or ambiguous values. NetPilot already has verified interface names, peers, topology relationships, and state. Deterministic code should construct the actual command from those facts.

Q: Why is human approval required?

Answer: Human approval establishes an authorization boundary between recommendation and network modification and provides accountability for operational changes.

Q: Why is pre-change validation important?

Answer: Network state may change after detection and approval. Revalidation prevents NetPilot from executing a stale remediation plan against a device whose live state no longer matches what the engineer reviewed.

Q: Why does NetPilot use multiple post-change validation attempts?

Answer: Network recovery is not instantaneous. Interface state, BGP sessions, and routing decisions can converge at different times. Bounded polling allows legitimate convergence while preventing NetPilot from waiting indefinitely.

Q: What proved that the primary/backup WAN architecture worked?

Answer: Before failure, R2 used the OSPF default through R1 with metric 10. After R1's primary ISP interface was shut down, R2 switched to R3 with metric 50. After restoration, the primary path returned.

Q: What is the difference between WAN failover and WAN remediation?

Answer: WAN failover is the network's automatic behavior when the primary path becomes unavailable. WAN remediation is NetPilot's controlled operational process for correcting the underlying problem and restoring the preferred path.

Q: Why was the 198.51.100.1 ping not used as the final failover test?

Answer: The simulated Internet prefixes exist independently on R4 and R5 and are not connected through a shared Layer-2 segment. The ping therefore does not reliably represent end-to-end Internet reachability through the selected WAN path. The installed default route, next hop, BGP state, and OSPF behavior are the authoritative lab measurements.

Final Lab Architecture Principle

The most important lesson from Lab 9 is:

AI should provide intelligence, but deterministic engineering controls execution.

The final NetPilot model is:

AI

=

Reasoning

Cisco Evidence

=

Source of Truth

Deterministic Logic

=

Command Generation

Policy

=

Safety Boundary

Engineer

=

Authorization

Netmiko

=

Execution

Validation

=

Proof of Outcome

This architecture is much closer to how a production network operations platform should be designed than simply asking an LLM to generate Cisco CLI.

LAB COMPLETION

Lab 9 is complete.

The NetPilot platform has now progressed from:

Lab 8:

Configuration Incident Operations

to:

Lab 9:

Enterprise WAN Incident Operations

with significantly greater architectural and operational complexity.

The next NetPilot capability should therefore be built on top of Lab 9, not restart it:

Incident Audit Records → Complete Incident Lifecycle → Dashboard Visualization → Customer-Facing Operations View

That next phase will connect the evidence, AI analysis, engineer approval, exact remediation commands, validation history, and final resolution into one persistent operational record.

22. Recommended Study Order for Lab 9

Use this sequence rather than trying to study everything simultaneously.

Stage 1 — Routing Foundation

Study:

IPv4 routing

Longest-prefix match

Static routes

Default routes

Administrative distance

Stage 2 — OSPF

Study:

Link-state routing

Neighbors

Areas

LSDB

SPF

Cost

Default-route origination

Then configure R1, R2, and R3.

Stage 3 — BGP Basics

Study:

Autonomous systems

BGP neighbors

eBGP

BGP states

Network advertisements

Then configure:

R1 ↔ R4

R3 ↔ R5

Stage 4 — iBGP

Study:

iBGP

Loopback peering

update-source

next-hop-self

Then configure:

R1 ↔ R3

Stage 5 — BGP Policy

Study:

Best-path algorithm

LOCAL_PREF

Route maps

Then implement:

ISP-1 LOCAL_PREF 200

ISP-2 LOCAL_PREF 100

Stage 6 — Enterprise Failover

Study:

Default routes

OSPF default origination

Routing metrics

Convergence

Redundancy

Then perform the ISP failure test.

Stage 7 — Network Automation

Study:

Python functions

Dictionaries

JSON

Netmiko

CLI parsing

Then examine:

cisco_client.py

bgp_health.py

wan_incident.py

Stage 8 — AI Operations

Study:

LLMs

Prompts

Grounding

Hallucination

Human-in-the-loop

Deterministic automation

Then examine:

ai_wan.py

wan_remediation.py

Stage 9 — Production Engineering

Study:

Pre-change validation

Post-change validation

Convergence

Auditability

Rollback

Blast radius

Change management

Then run the complete incident workflow.

23. Primary Reference List

Keep these resources bookmarked while working through NetPilot.

Cisco

Cisco IOS XE IP Routing Configuration Guide — OSPF

Use this for practical Cisco OSPF configuration and behavior.

Cisco OSPF Configuration Guide

Use this as an additional IOS XE OSPF operational reference.

Cisco — How Does OSPF Generate Default Routes?

Use this when studying the primary/backup default-route design implemented between R1, R3, and R2.

Cisco — Select BGP Best-path Algorithm

This is a critical Lab 9 reference. Use it to understand LOCAL_PREF and the BGP decision process.

Cisco Catalyst 8000V Installation and Configuration Documentation

Use this to understand the virtual router platform being used in EVE-NG.

IETF

RFC 2328 — OSPF Version 2

Use this when you want to move beyond configuration syntax and understand how OSPF actually operates as a protocol.

For BGP, add RFC 4271 — Border Gateway Protocol 4 (BGP-4) to your long-term study list. Do not attempt to memorize the entire RFC. Use it as the protocol reference while you gain more BGP experience.

24. What You Should Be Able to Explain After Lab 9

Do not consider the theory portion of Lab 9 complete until you can explain these concepts without simply reading the configuration.

Explain why R1 is the preferred WAN router.

Explain why R3 is still necessary when R1 is healthy.

Explain the difference between OSPF and BGP.

Explain the difference between eBGP and iBGP.

Explain why R1 and R3 use loopbacks for iBGP.

Explain why OSPF must know how to reach those loopbacks.

Explain what next-hop-self accomplishes.

Explain what LOCAL_PREF does.

Explain why 200 is preferred over 100.

Explain what a route map does.

Explain why R2 does not need to run BGP in this design.

Explain how R2 receives its default route.

Explain why R2 normally selects R1.

Explain why R2 switches to R3 after the primary WAN failure.

Explain why an interface coming UP does not necessarily mean the incident is resolved.

Explain why BGP may take additional time to converge.

Explain why NetPilot collects evidence before asking the LLM for a diagnosis.

Explain why Ollama is not allowed to directly construct and execute critical Cisco configuration.

Explain why deterministic remediation is safer.

Explain why engineer approval remains part of the workflow.

Explain why NetPilot checks the network again immediately before making a change.

Explain why post-change validation must verify multiple network layers.

If you can confidently explain those topics and reproduce the Lab 9 failure and recovery workflow, you are learning much more than Cisco CLI commands. You are learning how routing architecture, network operations, software automation, and AI-assisted troubleshooting fit together as one engineering system.