NetPilot v0.2 — Step-by-Step Implementation Guide

1. Purpose of NetPilot v0.2

NetPilot v0.1 successfully demonstrated that Python could collect Cisco network information and send it to Ollama/Qwen2.5:32B for troubleshooting.

During testing, however, we discovered an important problem.

The AI could identify that two routers had an OSPF configuration mismatch, but it could potentially recommend changing the wrong router.

For example:

R2 Gi2 → Area 0

R3 Gi2 → Area 1

The AI correctly recognized the mismatch but could recommend changing R2 simply because R3 was the other side of the problem.

For v0.2, we introduced Network Intent.

The purpose was to give NetPilot a definition of what the network should look like so the AI could compare:

ACTUAL NETWORK

vs.

INTENDED NETWORK

This allows NetPilot to determine which device violates the intended design.

2. NetPilot v0.2 Architecture

The v0.2 workflow became:

Cisco Network

/      |      \

R1     R2       R3

\      |      /

\     |     /

Live Evidence

|

▼

Evidence Collector

|

▼

Network Intent

|

▼

AI Prompt

|

▼

Ollama

|

▼

Qwen2.5:32B

|

▼

AI Diagnosis

The major addition is the Network Intent layer.

3. Create the Network Intent Directory

Inside the NetPilot project, create:

data

The project should now contain:

NetPilot

│

├── app

│

├── data

│

└── .venv

Inside data, create:

network_intent.json

The final path is:

data/network_intent.json

4. Create network_intent.json

The purpose of this file is to describe the network's expected state.

Add:

{

"network_name": "NetPilot v0.2 OSPF Lab",

"ospf": {

"process_id": 1,

"expected_neighbors": {

"R1": ["R2", "R3"],

"R2": ["R1", "R3"],

"R3": ["R1", "R2"]

},

"expected_areas": {

"R1": {

"GigabitEthernet1": 0,

"GigabitEthernet2": 0,

"Loopback0": 0

},

"R2": {

"GigabitEthernet1": 0,

"GigabitEthernet2": 0,

"Loopback0": 0

},

"R3": {

"GigabitEthernet1": 0,

"GigabitEthernet2": 0,

"Loopback0": 0

}

}

}

}

What this does

The file tells NetPilot:

R1 should have:

R2 as neighbor

R3 as neighbor

R2 should have:

R1 as neighbor

R3 as neighbor

R3 should have:

R1 as neighbor

R2 as neighbor

It also tells NetPilot:

R1 Gi1 = Area 0

R1 Gi2 = Area 0

R2 Gi1 = Area 0

R2 Gi2 = Area 0

R3 Gi1 = Area 0

R3 Gi2 = Area 0

This becomes the expected state against which the live network is compared.

5. Create intent_engine.py

Create:

app/intent_engine.py

Add:

import json

# Imports Python's built-in JSON library.

# JSON is the format used by our network intent file.

def load_network_intent():

# Creates a function called load_network_intent.

# This function will load the expected network configuration.

with open("data/network_intent.json", "r") as file:

# Opens the network_intent.json file.

#

# "r" means read mode.

#

# The file is temporarily represented by the variable "file".

intent = json.load(file)

# Reads the JSON file and converts it into

# a Python dictionary.

#

# A dictionary allows Python to work with the

# network intent information.

return intent

# Sends the loaded network intent back to

# whatever part of NetPilot called this function.

6. Why We Created intent_engine.py

Before v0.2, NetPilot primarily dealt with information collected from the routers.

Now NetPilot has two separate types of information:

LIVE INFORMATION

↓

Cisco routers

and:

EXPECTED INFORMATION

↓

network_intent.json

The intent engine is responsible for loading the second one.

7. Create evidence_collector.py

Create:

app/evidence_collector.py

Add:

from app.cisco_client import run_command

# Imports the run_command function from our existing

# Cisco client.

#

# This allows NetPilot to execute commands on the routers.

def collect_network_evidence():

# Creates a function that collects troubleshooting

# information from the Cisco network.

evidence = {}

# Creates an empty dictionary.

#

# We will store all router evidence inside this dictionary.

commands = [

"show ip ospf neighbor",

# Shows the OSPF neighbors currently established.

"show ip ospf interface brief",

# Shows OSPF information for the router's interfaces.

"show ip protocols",

# Shows routing protocol configuration and information.

"show ip route ospf"

# Shows routes learned through OSPF.

]

routers = ["R1", "R2", "R3"]

# Creates a list containing the routers

# that NetPilot needs to investigate.

for router in routers:

# Goes through each router one at a time.

evidence[router] = {}

# Creates a separate section in our evidence dictionary

# for the current router.

for command in commands:

# Goes through each troubleshooting command.

print(f"Collecting {router}: {command}")

# Displays what NetPilot is currently doing.

output = run_command(router, command)

# Connects to the router and runs the command.

evidence[router][command] = output

# Saves the command output inside the evidence

# belonging to that router.

return evidence

# Returns all of the collected router evidence.

8. Why We Added More Evidence

In v0.1, NetPilot was already collecting Cisco information.

For v0.2, we wanted the AI to have more evidence available when comparing the actual network against the intended network.

NetPilot now collects:

show ip ospf neighbor

to determine whether the expected neighbors actually exist.

It collects:

show ip ospf interface brief

to determine which interfaces are participating in OSPF.

It collects:

show ip protocols

to understand the routing protocol configuration.

And:

show ip route ospf

to determine whether OSPF routes are actually being learned.

9. Update main.py

The next major change was to app/main.py.

The purpose of main.py is now to coordinate the entire v0.2 workflow.

The complete version is:

import json

# Imports Python's JSON library.

# We use this to format the network intent and

# router evidence before sending them to the AI.

from app.intent_engine import load_network_intent

# Imports the function that loads our expected network design.

from app.evidence_collector import collect_network_evidence

# Imports the function that collects live information

# from R1, R2, and R3.

from app.ollama_client import ask_ollama

# Imports the function that sends our prompt

# to Ollama/Qwen.

def main():

# Defines the main function of NetPilot.

print("\n===== NETPILOT v0.2 =====\n")

# Displays the NetPilot v0.2 heading.

print("Loading network intent...")

# Tells us that NetPilot is loading

# the expected network configuration.

intent = load_network_intent()

# Loads network_intent.json into the variable "intent".

print("Collecting live network evidence...\n")

# Tells us NetPilot is now collecting

# information directly from the Cisco routers.

evidence = collect_network_evidence()

# Runs the evidence collector.

#

# This gathers information from R1, R2, and R3.

intent_text = json.dumps(intent, indent=2)

# Converts the Python dictionary containing the

# network intent into readable JSON text.

#

# "indent=2" makes the information easier to read.

evidence_text = json.dumps(evidence, indent=2)

# Converts the router evidence into readable JSON text.

prompt = f"""

You are NetPilot, an AI network troubleshooting agent.

You are troubleshooting a Cisco OSPF network.

You have two important sources of information.

==================================================

NETWORK INTENT

==================================================

This is how the network is supposed to be configured:

{intent_text}

==================================================

LIVE NETWORK EVIDENCE

==================================================

This is the actual information collected directly

from the Cisco routers:

{evidence_text}

==================================================

YOUR TASK

==================================================

Compare the LIVE NETWORK EVIDENCE against the NETWORK INTENT.

Determine:

1. What network behavior is currently broken?

2. Which OSPF adjacency or connectivity is missing?

3. Which router or routers are affected?

4. What evidence proves the problem?

5. Which specific router is violating the intended network design?

6. Do NOT recommend changing a correctly configured router

just to match a misconfigured router.

7. Use the NETWORK INTENT as the source of truth.

8. Identify the exact interface and configuration that

should be corrected.

9. Provide the Cisco commands required to fix the problem.

10. Explain your reasoning in a way that a network engineer

can verify.

Respond using these sections:

PROBLEM

AFFECTED DEVICES

EVIDENCE

ROOT CAUSE

DEVICE VIOLATING NETWORK INTENT

RECOMMENDED FIX

CONFIDENCE LEVEL

"""

# Creates the actual prompt that will be sent

# to the AI.

#

# The prompt contains both:

#

# 1. Expected network state

# 2. Actual network state

print("Sending evidence to Ollama and Qwen...\n")

# Lets us know the AI portion of the process

# is beginning.

answer = ask_ollama(prompt)

# Sends the completed prompt to Ollama.

#

# Ollama uses Qwen2.5:32B to analyze it.

#

# The AI response is stored in "answer".

print("\n===== NETPILOT AI DIAGNOSIS =====\n")

# Displays a heading before the AI's response.

print(answer)

# Displays the diagnosis generated by Qwen.

if __name__ == "__main__":

# Checks whether this file is being executed directly.

main()

# Starts the NetPilot program.

10. Understanding Where the AI Prompt Comes From

This was an important part of understanding the NetPilot architecture.

The prompt is created inside main.py.

First:

intent = load_network_intent()

loads the expected network state.

Then:

evidence = collect_network_evidence()

collects the actual network state.

Then:

intent_text = json.dumps(intent, indent=2)

and:

evidence_text = json.dumps(evidence, indent=2)

convert that information into readable text.

Then the two pieces are inserted into:

prompt = f"""

...

{intent_text}

...

{evidence_text}

...

"""

So the prompt is essentially:

NETWORK INTENT

+

LIVE CISCO EVIDENCE

↓

PROMPT

11. Sending the Prompt to Ollama

The next line is:

answer = ask_ollama(prompt)

This means:

Take the prompt we just created and send it to the ask_ollama() function.

The function is located in:

app/ollama_client.py

Inside that function:

def ask_ollama(prompt: str) -> str:

the prompt arrives.

Then:

"content": prompt,

places the contents of the prompt into the message sent to Ollama.

In simple terms:

main.py

↓

Creates prompt

↓

ask_ollama(prompt)

↓

ollama_client.py

↓

"content": prompt

↓

Ollama

↓

Qwen2.5:32B

12. The Existing ollama_client.py

We did not need to fundamentally change the Ollama client for v0.2.

The important existing code remains:

import requests

# Allows Python to communicate with Ollama through HTTP.

from app.config import OLLAMA_HOST, OLLAMA_MODEL, OLLAMA_TIMEOUT

# Loads the Ollama server address, model name,

# and timeout settings.

def ask_ollama(prompt: str) -> str:

# Receives the prompt created by main.py.

url = f"{OLLAMA_HOST}/api/chat"

# Creates the Ollama chat API URL.

payload = {

"model": OLLAMA_MODEL,

# Tells Ollama which AI model to use.

"messages": [

{

"role": "system",

# Defines instructions for the AI.

"content": (

"You are NetPilot, an AI network operations assistant. "

"You specialize in enterprise Cisco networking, "

"routing, switching, security, and troubleshooting."

),

# Defines NetPilot's role and area of expertise.

},

{

"role": "user",

# Identifies the actual troubleshooting request.

"content": prompt,

# Places the prompt created by main.py here.

},

],

"stream": False,

# Requests the complete response instead of

# receiving it piece by piece.

}

response = requests.post(

url,

# Sends the request to Ollama.

json=payload,

# Sends the information as JSON.

timeout=OLLAMA_TIMEOUT,

# Defines how long Python waits for Ollama.

)

response.raise_for_status()

# Checks whether Ollama returned an HTTP error.

data = response.json()

# Converts Ollama's JSON response into Python data.

return data["message"]["content"]

# Extracts the actual response generated by Qwen

# and sends it back to main.py.

13. Run NetPilot v0.2

With the OSPF problem intentionally left broken, run:

python -m app.main

NetPilot performs:

1. Load Network Intent

↓

2. Collect R1 evidence

↓

3. Collect R2 evidence

↓

4. Collect R3 evidence

↓

5. Combine expected + actual state

↓

6. Build AI prompt

↓

7. Send prompt to Ollama

↓

8. Qwen2.5:32B analyzes the evidence

↓

9. Display diagnosis

14. What We Were Testing

The primary test was the intentionally broken R2-R3 OSPF relationship.

The AI needed to recognize:

R2 Gi2

10.0.23.1

and:

R3 Gi2

10.0.23.2

were supposed to operate in the same OSPF area.

The important question was not simply:

"Do R2 and R3 have different configurations?"

The important question was:

"Which router violates the network intent?"

15. Why This Is Different From v0.1

v0.1

Cisco Output

↓

Qwen

↓

Diagnosis

v0.2

Cisco Output

+

Network Intent

↓

Qwen

↓

Compare Actual vs Expected

↓

Identify Violation

↓

Diagnosis

This is the key architectural improvement.

16. What We Added in v0.2

The actual v0.2 additions were:

1. Network Intent

data/network_intent.json

2. Intent Loader

app/intent_engine.py

3. Dedicated Evidence Collector

app/evidence_collector.py

4. Updated AI Prompt

main.py was modified to provide Qwen with:

Expected Network State

+

Actual Network State

5. Intent-Based AI Reasoning

Qwen was explicitly instructed to:

Compare actual configuration against intended configuration.

6. Safer Remediation Reasoning

Qwen was instructed:

Do NOT recommend changing a correctly configured router

just to match a misconfigured router.

7. Structured Diagnosis

The AI was asked to return:

PROBLEM

AFFECTED DEVICES

EVIDENCE

ROOT CAUSE

DEVICE VIOLATING NETWORK INTENT

RECOMMENDED FIX

CONFIDENCE LEVEL

17. Final v0.2 Architecture

At the end of v0.2, NetPilot's architecture became:

┌─────────────────┐

│   Cisco Routers │

│   R1 R2 R3      │

└────────┬────────┘

│

▼

┌─────────────────┐

│    Evidence     │

│    Collector    │

└────────┬────────┘

│

│

┌──────────────────┘

│

▼

┌─────────────────────┐

│   Network Intent     │

│ network_intent.json  │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│     main.py         │

│ Builds AI Prompt    │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│       Ollama        │

│  192.168.1.217      │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│    Qwen2.5:32B      │

│    AI Reasoning     │

└──────────┬──────────┘

│

▼

┌─────────────────────┐

│ NetPilot Diagnosis  │

└─────────────────────┘

18. The Main v0.2 Achievement

The most important addition wasn't another Cisco command or another Python function.

It was introducing the concept of:

"What should the network look like?"