NetPilot v0.4 Lab — AI-Powered Golden Configuration & Drift Detection

Lab Objective

In this lab, we expanded NetPilot from basic network troubleshooting into AI-assisted configuration drift detection.

The objective was to:

- Create golden configurations for R1, R2, and R3.

- Compare the live Cisco configurations against those golden configurations.

- Intentionally introduce an OSPF configuration deviation.

- Detect the deviation with Python.

- Send the configuration evidence to Ollama/Qwen2.5:32B.

- Have AI determine what changed and which router should be corrected.

- Identify limitations in the AI's ability to identify the exact interface requiring correction.

Our environment:

Laptop

192.168.1.38

|

| Home Network

|

EVE-NG

192.168.1.83

|

+---------------- R1

|                 192.168.1.201

|

+---------------- R2

|                 192.168.1.202

|

+---------------- R3

192.168.1.203

Ollama is running on the desktop:

Ollama Desktop

192.168.1.217

AI model:

qwen2.5:32b

Part 1 — Lab Topology

The Cisco topology consists of three routers:

R1

/      \

/        \

R2 -------- R3

OSPF was initially configured so that the routers formed healthy adjacencies.

The point of this lab was not to create an OSPF problem immediately.

Instead, we first established the correct network configuration and used it as our golden configuration.

Part 2 — NetPilot Project Structure

Our NetPilot project was organized approximately like this:

NetPilot

│

├── app

│   ├── main.py

│   ├── config.py

│   ├── ollama_client.py

│   ├── cisco_client.py

│   ├── golden_config.py

│   ├── drift_detector.py

│   ├── ai_drift.py

│   ├── test_netmiko.py

│   ├── test_cisco_client.py

│   └── test_ai_drift.py

│

└── data

└── golden

├── R1.cfg

├── R2.cfg

└── R3.cfg

The important idea is that:

data/golden/

contains the approved configurations.

Part 3 — Cisco Management Configuration

Each router was connected to the home network using a management interface.

The management addresses were:

R1 = 192.168.1.201

R2 = 192.168.1.202

R3 = 192.168.1.203

The management interface on R1 was configured as:

interface GigabitEthernet3

ip address 192.168.1.201 255.255.255.0

no shutdown

R2:

interface GigabitEthernet3

ip address 192.168.1.202 255.255.255.0

no shutdown

R3:

interface GigabitEthernet3

ip address 192.168.1.203 255.255.255.0

no shutdown

These interfaces were used for NetPilot's management access.

The OSPF links remained separate from the management network.

Part 4 — Verify OSPF

The original OSPF topology was:

R1 Gi1 ---- R2 Gi1

10.0.12.1   10.0.12.2

R1 Gi2 ---- R3 Gi1

10.0.13.1   10.0.13.2

R2 Gi2 ---- R3 Gi2

10.0.23.1   10.0.23.2

The routers used loopback addresses:

R1 = 10.255.1.1

R2 = 10.255.2.2

R3 = 10.255.3.3

We verified OSPF using:

show ip ospf neighbor

A healthy R1 showed:

Neighbor ID     Pri   State           Dead Time   Address         Interface

3.3.3.3           1   FULL/DR         00:00:35    10.0.13.2       GigabitEthernet2

2.2.2.2           1   FULL/DR         00:00:31    10.0.12.2       GigabitEthernet1

This confirmed that the network was healthy before creating the configuration drift.

Part 5 — Cisco NetPilot User

NetPilot used a Cisco username:

netpilot

The password used in this lab was:

NetPilotLab2026!

The user was used by Netmiko to connect to the routers.

Part 6 — cisco_client.py

Create:

app/cisco_client.py

Complete code:

from netmiko import ConnectHandler

# Imports Netmiko so Python can connect to Cisco devices.

DEVICES = {

"R1": {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R2": {

"device_type": "cisco_ios",

"host": "192.168.1.202",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

"R3": {

"device_type": "cisco_ios",

"host": "192.168.1.203",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

},

}

# Stores the connection information for each router.

def run_command(device_name, command):

# Creates a function that runs a command on a router.

device = DEVICES[device_name]

# Gets the connection information for the requested router.

connection = ConnectHandler(**device)

# Uses Netmiko to connect to the router.

try:

# Starts a section where Python will try to run the command.

output = connection.send_command(command)

# Sends the Cisco command to the router.

return output

# Returns the command output to our program.

finally:

# This runs even if something goes wrong.

connection.disconnect()

# Closes the SSH connection.

Part 7 — Test Netmiko

Before connecting Netmiko to the rest of NetPilot, we created:

app/test_netmiko.py

Complete code:

from netmiko import ConnectHandler

# Imports Netmiko so Python can connect to Cisco.

device = {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "NetPilotLab2026!",

"port": 22,

}

# Contains the information Netmiko needs to connect to R1.

print("Connecting to R1...")

# Tells us that the connection attempt is starting.

connection = ConnectHandler(**device)

# Attempts to establish the SSH connection.

print("Connected!")

# If we get here, Netmiko successfully connected.

output = connection.send_command("show ip ospf neighbor")

# Sends a simple Cisco command.

print(output)

# Displays the router's response.

connection.disconnect()

# Closes the SSH connection.

Run:

python -m app.test_netmiko

Successful output demonstrated that Netmiko could connect to R1 and retrieve OSPF information.

Part 8 — Test cisco_client.py

We also created:

app/test_cisco_client.py

Complete code:

from app.cisco_client import run_command

# Imports the same function used by drift_detector.py.

print("Testing cisco_client.py...")

# Tells us the test is starting.

output = run_command("R1", "show ip ospf neighbor")

# Uses our actual NetPilot Cisco function to connect to R1

# and run the OSPF neighbor command.

print()

# Prints a blank line.

print(output)

# Displays the router's response.

Run:

python -m app.test_cisco_client

This verified that our actual NetPilot Cisco client was working.

Part 9 — Create the Golden Configuration

The golden configuration represents the approved network state.

We created:

app/golden_config.py

Complete code:

from app.cisco_client import run_command

# Imports the function that connects to the Cisco routers

# and runs show commands.

from pathlib import Path

# Imports Path, which makes it easier to work with files and folders.

ROUTERS = ["R1", "R2", "R3"]

# Creates a list containing the routers we want to collect.

def collect_running_config(router):

# Collects the current configuration from one router.

print(f"Collecting configuration from {router}...")

# Shows which router NetPilot is currently working on.

config = run_command(router, "show running-config")

# Connects to the router and runs "show running-config".

return config

# Sends the configuration back to the program.

def save_golden_config(router, config):

# Saves the router configuration as a golden configuration.

directory = Path("data/golden")

# Defines the folder where our golden configurations will live.

directory.mkdir(parents=True, exist_ok=True)

# Creates the data/golden folder if it doesn't already exist.

filename = directory / f"{router}.cfg"

# Creates a filename such as:

# data/golden/R1.cfg

filename.write_text(config)

# Writes the router's configuration into the file.

print(f"Saved golden configuration for {router}")

# Tells us that the configuration was saved successfully.

def create_golden_config():

# Creates the golden configuration for all routers.

for router in ROUTERS:

# Goes through R1, then R2, then R3.

config = collect_running_config(router)

# Gets the running configuration from the router.

save_golden_config(router, config)

# Saves that configuration as the golden configuration.

if __name__ == "__main__":

# Checks whether we are running this file directly.

create_golden_config()

# Starts the entire golden configuration process.

Run:

python -m app.golden_config

This created:

data/golden/R1.cfg

data/golden/R2.cfg

data/golden/R3.cfg

These files became our approved baseline.

Part 10 — Understand the router Variable

An important Python concept encountered during this lab was the router variable.

We have:

ROUTERS = ["R1", "R2", "R3"]

Then:

for router in ROUTERS:

This means:

First:

router = "R1"

Second:

router = "R2"

Third:

router = "R3"

The function:

def collect_running_config(router):

uses router as a parameter.

The parameter is basically an empty box waiting for a value.

When we call:

collect_running_config(router)

the value from the loop is passed into the function.

Conceptually:

ROUTERS

|

+--- R1 ---> collect_running_config("R1")

|

+--- R2 ---> collect_running_config("R2")

|

+--- R3 ---> collect_running_config("R3")

The important distinction is:

Parameter = placeholder

Argument  = actual value being passed

For example:

def say_hello(name):

print("Hello " + name)

name is the parameter.

When we do:

say_hello("Daniel")

"Daniel" is the argument.

The same concept is being used with:

def collect_running_config(router):

Part 11 — Intentionally Create Configuration Drift

After creating the golden configuration, we intentionally changed R3's OSPF configuration.

The golden configuration expected:

ip ospf 1 area 0

R3 was intentionally changed so that its affected interface used:

ip ospf 1 area 1

This created a configuration drift.

The important concept is:

Golden Configuration

↓

Area 0

Live R3 Configuration

↓

Area 1

↓

DRIFT

We intentionally left the OSPF problem in place so NetPilot could diagnose it.

Part 12 — Initial Drift Detector

We created:

app/drift_detector.py

The purpose was to compare the golden configuration with the live configuration.

Complete code:

from pathlib import Path

# Lets Python work with files.

from app.cisco_client import run_command

# Lets NetPilot send commands to Cisco routers.

from app.ai_drift import analyze_drift

# Lets NetPilot send configuration differences to Qwen.

routers = ["R1", "R2", "R3"]

# These are the routers NetPilot will check.

def check_router(router):

# Checks one router for configuration drift.

print()

# Prints a blank line.

print("Checking " + router)

# Shows which router we are checking.

golden_file = Path("data/golden/" + router + ".cfg")

# Finds the golden configuration for this router.

golden = golden_file.read_text()

# Reads the golden configuration.

live = run_command(router, "show running-config")

# Gets the current configuration from the Cisco router.

golden_lines = golden.splitlines()

# Breaks the golden configuration into individual lines.

live_lines = live.splitlines()

# Breaks the live configuration into individual lines.

if golden == live:

# Checks whether the configurations are exactly the same.

print(router + " is compliant")

# Tells us there is no drift.

return

# Stops checking this router.

print(router + " has configuration drift")

# Tells us that something changed.

missing = []

# Creates an empty list for configuration that is missing.

added = []

# Creates an empty list for configuration that was added.

for line in golden_lines:

# Looks at every line in the golden configuration.

if line not in live_lines:

# Checks if the golden line is missing from the live router.

missing.append(line)

# Adds the missing line to our list.

for line in live_lines:

# Looks at every line in the live configuration.

if line not in golden_lines:

# Checks if this live line wasn't in the golden configuration.

added.append(line)

# Adds the unexpected line to our list.

print()

# Adds spacing.

print("Sending configuration drift to Qwen...")

# Tells us that the AI analysis is starting.

answer = analyze_drift(router, missing, added)

# Sends the router and configuration differences to Qwen.

print()

# Adds spacing.

print("===== NETPILOT AI DIAGNOSIS =====")

# Displays the AI diagnosis heading.

print(answer)

# Displays Qwen's answer.

for router in routers:

# Goes through R1, R2, and R3.

check_router(router)

# Checks the current router.

Part 13 — Create the AI Drift Analyzer

We created:

app/ai_drift.py

Complete code:

from app.ollama_client import ask_ollama

# Imports our existing function that sends information to Qwen.

def analyze_drift(router, missing, added):

# Creates a function that sends configuration drift

# to the AI for analysis.

prompt = f"""

You are NetPilot, an AI network troubleshooting assistant.

Analyze the configuration drift below.

Router:

{router}

Configuration expected by the golden configuration:

{missing}

Configuration currently found on the router:

{added}

Explain:

1. What changed?

2. Why could this be a problem?

3. What network technology is affected?

4. Which router should be corrected?

5. What configuration should be restored?

Do not make changes to the router.

Only provide a diagnosis and recommended configuration.

"""

# Creates the instructions that we will send to Qwen.

answer = ask_ollama(prompt)

# Sends the prompt to Ollama and gets Qwen's response.

return answer

# Sends the AI's answer back to our program.

Part 14 — Connect NetPilot to Ollama

Our existing ollama_client.py was used to communicate with the Ollama server.

Complete code:

import requests

# Imports the requests library so Python can communicate with Ollama.

from app.config import OLLAMA_HOST, OLLAMA_MODEL, OLLAMA_TIMEOUT

# Imports our Ollama settings such as the server address,

# model name, and timeout.

def ask_ollama(prompt: str) -> str:

# Creates a function that sends a prompt to Ollama

# and returns the AI's response.

url = f"{OLLAMA_HOST}/api/chat"

# Builds the Ollama API URL.

payload = {

"model": OLLAMA_MODEL,

"messages": [

{

"role": "system",

"content": (

"You are NetPilot, an AI network operations assistant. "

"You specialize in enterprise Cisco networking, "

"routing, switching, security, and troubleshooting."

),

},

{

"role": "user",

"content": prompt,

},

],

"stream": False,

}

# Creates the information that will be sent to Ollama.

response = requests.post(

url,

json=payload,

timeout=OLLAMA_TIMEOUT,

)

# Sends the request to the Ollama server.

response.raise_for_status()

# Stops the program if Ollama returns an error.

data = response.json()

# Converts Ollama's response into Python data.

return data["message"]["content"]

# Extracts the AI's response and sends it back to our program.

Part 15 — Understand Where prompt Comes From

A key Python concept from this lab was understanding:

"content": prompt,

At first, it may look like prompt magically exists.

It doesn't.

The prompt comes from:

def analyze_drift(router, missing, added):

Inside that function we create:

prompt = f"""

...

"""

Then we call:

answer = ask_ollama(prompt)

That passes the prompt into:

def ask_ollama(prompt: str) -> str:

So the flow is:

drift_detector.py

|

| router

| missing

| added

↓

ai_drift.py

|

| creates prompt

↓

ask_ollama(prompt)

|

↓

ollama_client.py

|

↓

Ollama

|

↓

Qwen2.5:32B

Part 16 — AI Test Script

Before connecting the AI to live configuration collection, we tested the AI independently.

Create:

app/test_ai_drift.py

Complete code:

from app.ai_drift import analyze_drift

# Imports our AI drift analysis function.

router = "R3"

# Tells NetPilot which router has the problem.

missing = ["ip ospf 1 area 0"]

# This is what the golden configuration expects.

added = ["ip ospf 1 area 1"]

# This is what we found on the live router.

answer = analyze_drift(router, missing, added)

# Sends the configuration difference to Qwen.

print()

# Prints a blank line.

print("===== NETPILOT AI DIAGNOSIS =====")

# Prints a heading.

print(answer)

# Displays Qwen's answer.

Run:

python -m app.test_ai_drift

Part 17 — AI Diagnosis

Qwen correctly analyzed the intentionally created R3 deviation.

The AI determined:

Router affected:

R3

It recognized:

Expected:

ip ospf 1 area 0

versus:

Actual:

ip ospf 1 area 1

The AI correctly recommended restoring:

ip ospf 1 area 0

This represented an improvement over an earlier NetPilot test where the AI identified the OSPF area mismatch but incorrectly recommended changing R2.

Part 18 — Important Limitation Discovered

Although the AI correctly identified R3, it did not provide enough detail about the exact interface that required correction.

For an enterprise network operations system, this is not precise enough.

The ideal answer should be:

Router: R3

Interface: GigabitEthernet2

OSPF Process: 1

Current Area: 1

Expected Area: 0

Recommended Change:

interface GigabitEthernet2

ip ospf 1 area 0

Instead, the AI essentially gave us:

R3 needs to be corrected.

Restore:

ip ospf 1 area 0

This is useful, but NetPilot needs to become more precise.

Part 19 — Why This Matters

This discovery became an important design requirement for future NetPilot versions.

We don't want NetPilot to simply say:

"OSPF is broken."

We want:

R3

↓

GigabitEthernet2

↓

OSPF Process 1

↓

Current Area 1

↓

Expected Area 0

↓

Network intent violated

Eventually, NetPilot should be able to understand:

Device

+

Interface

+

Protocol

+

Configuration

+

Topology

+

Golden Configuration

+

Network Intent

↓

AI Reasoning

↓

Precise Diagnosis

Part 20 — Final NetPilot v0.4 Architecture

At the end of this lab, our architecture looked like:

Cisco Network

|

R1 / R2 / R3

|

▼

Netmiko/Python

|

▼

Running Configuration

|

▼

Drift Detector

|

Compare with Golden

|

▼

Configuration Drift

|

▼

AI Drift Analyzer

|

▼

Ollama

192.168.1.217

|

▼

Qwen2.5:32B

|

▼

AI Diagnosis

Part 21 — What We Learned

The most important lessons from v0.4 were:

1. Golden Configuration

A golden configuration provides an approved baseline for the network.

2. Configuration Drift

NetPilot can compare the live configuration against the approved baseline.