NetPilot v0.5 Lab — AI-Assisted Human-Approved Network Remediation

Lab Objective

In this lab, NetPilot was enhanced from an AI-based network diagnostic tool into an AI-assisted remediation system.

The goal was to allow NetPilot to:

- Collect OSPF information from Cisco routers.

- Send the network evidence to the local Qwen model running through Ollama.

- Have AI identify the specific router and interface with the problem.

- Have AI determine the current and expected OSPF area.

- Present a proposed configuration change.

- Ask the network engineer for approval.

- Apply the approved configuration to the Cisco router.

- Verify the network after the change.

The important design principle is:

AI recommends. NetPilot validates and controls the change. The engineer approves. NetPilot executes and verifies.

This is an important step toward making NetPilot a potential network automation product rather than simply an AI troubleshooting script.

1. Previous NetPilot Environment

The NetPilot project was being developed on a Windows laptop using a Python virtual environment.

Project location:

E:\IT\Cisco Labs\NetPilot

The Python virtual environment was:

.venv

The project already contained the Cisco connectivity and Ollama integration required for this lab.

The local AI model was:

qwen2.5:32b

Ollama was running on the separate desktop at:

192.168.1.217:11434

NetPilot communicates with Ollama over HTTP.

2. Cisco Lab Topology

The lab consisted of three Cisco routers:

R1

/    \

/      \

R2        R3

\      /

\    /

Management addresses used by NetPilot:

| Router | Management IP |
| --- | --- |
| R1 | 192.168.1.201 |
| R2 | 192.168.1.202 |
| R3 | 192.168.1.203 |

The OSPF links were:

R1 Gi1 ↔ R2 Gi1

10.0.12.0/30

R1 Gi2 ↔ R3 Gi1

10.0.13.0/30

R2 Gi2 ↔ R3 Gi2

10.0.23.0/30

The intended OSPF design was:

Area 0

for all three links.

3. The OSPF Problem

The lab intentionally contained an OSPF configuration error.

R2 was configured with:

GigabitEthernet2

10.0.23.1/30

OSPF Area 0

R3 was configured with:

GigabitEthernet2

10.0.23.2/30

OSPF Area 1

Therefore:

R2 Gi2 → Area 0

R3 Gi2 → Area 1

Because the two directly connected interfaces were in different OSPF areas, an OSPF adjacency could not form.

The expected result was:

R2 Gi2 ↔ R3 Gi2

OSPF adjacency: DOWN

4. Existing Cisco Client

NetPilot already had a Cisco client that used Netmiko.

File:

app/cisco_client.py

The client contained the Cisco router information and a function for running show commands.

It was then expanded to allow NetPilot to send configuration commands.

5. Complete cisco_client.py

The complete script used for this lab was:

from netmiko import ConnectHandler

# Imports Netmiko, which allows Python to connect to Cisco devices using SSH.

DEVICES = {

"R1": {

"device_type": "cisco_ios",

"host": "192.168.1.201",

"username": "netpilot",

"password": "NetPilotLab2026!",

},

"R2": {

"device_type": "cisco_ios",

"host": "192.168.1.202",

"username": "netpilot",

"password": "NetPilotLab2026!",

},

"R3": {

"device_type": "cisco_ios",

"host": "192.168.1.203",

"username": "netpilot",

"password": "NetPilotLab2026!",

},

}

# Stores the connection information for all three Cisco routers.

def run_command(device_name, command):

# Creates a function that connects to a router and runs a show command.

device = DEVICES[device_name]

# Looks up the connection information for the requested router.

connection = ConnectHandler(**device)

# Opens an SSH connection to the Cisco router.

try:

output = connection.send_command(command)

# Sends the requested show command to the router.

return output

# Sends the router's output back to the program.

finally:

connection.disconnect()

# Closes the SSH connection when finished.

def send_config(device_name, commands):

# Creates a function that connects to a router and sends configuration commands.

device = DEVICES[device_name]

# Looks up the connection information for the requested router.

connection = ConnectHandler(**device)

# Opens an SSH connection to the router.

try:

output = connection.send_config_set(commands)

# Sends the configuration commands to the Cisco router.

return output

# Sends the router's response back to the program.

finally:

connection.disconnect()

# Closes the SSH connection.

6. Understanding the New send_config() Function

The important addition was:

def send_config(device_name, commands):

This function allows NetPilot to do more than simply read the network.

Previously:

NetPilot → Cisco

↓

SHOW commands

↓

Information

Now NetPilot can also do:

NetPilot → Cisco

↓

Configuration

↓

Cisco

The actual configuration is sent using:

connection.send_config_set(commands)

Netmiko handles entering configuration mode and sending the commands.

7. Testing Configuration Changes

Before allowing AI to make changes, a simple test was created.

File:

app/test_config.py

The purpose was to prove that NetPilot could successfully modify a Cisco router.

Complete script:

from app.cisco_client import send_config

# Imports the function that allows NetPilot to send configuration commands.

commands = [

"interface Loopback0",

"description NETPILOT-TEST",

]

# Creates two Cisco configuration commands.

# The commands enter Loopback0 and add a description.

print("===== TESTING CONFIGURATION =====")

# Displays a heading so we know the test has started.

output = send_config("R3", commands)

# Connects to R3 and sends the configuration commands.

print(output)

# Displays the response returned by the Cisco router.

The script was executed with:

python -m app.test_config

The Cisco router responded:

configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#interface Loopback0

R3(config-if)#description NETPILOT-TEST

R3(config-if)#end

R3#

This proved that NetPilot could successfully send configuration commands to R3.

8. Creating the Human Approval Concept

The next goal was to prevent NetPilot from automatically changing a router.

Instead, the program needed to ask the engineer for approval.

The desired workflow was:

AI Recommendation

↓

Proposed Change

↓

Human Approval

↓

YES

↓

Apply Configuration

or:

AI Recommendation

↓

Proposed Change

↓

Human Approval

↓

NO

↓

Do Nothing

9. Testing Human Approval

A test script called:

app/test_approval.py

was created to demonstrate the concept.

The important section was:

answer = input("Apply this change? (yes/no): ")

The input() function pauses the Python program and waits for the engineer to type a response.

If the engineer types:

yes

the program can proceed.

If the engineer types:

no

the program stops without making a change.

This established the foundation for human-in-the-loop remediation.

10. Combining AI and Remediation

The next step was to create a new program:

app/ai_remediation.py

This became the primary script for the v0.5 AI remediation workflow.

The script combines:

- Cisco information gathering

- OSPF analysis

- Ollama/Qwen

- AI recommendation

- Recommendation parsing

- Human approval

- Cisco configuration

- Post-change verification

11. Complete ai_remediation.py

The complete script is shown below.

from app.cisco_client import run_command, send_config

# Imports the functions used to read from and configure Cisco routers.

from app.ollama_client import ask_ollama

# Imports the function that sends network information to Qwen.

ROUTERS = ["R1", "R2", "R3"]

# Creates a list containing all routers NetPilot will check.

def collect_ospf_evidence():

# Collects OSPF information from all routers.

evidence = ""

# Creates an empty piece of text where the router information will be stored.

for router in ROUTERS:

# Goes through R1, R2, and R3 one at a time.

print(f"Collecting OSPF information from {router}...")

# Displays the router currently being checked.

neighbors = run_command(router, "show ip ospf neighbor")

# Gets the OSPF neighbor information from the router.

interfaces = run_command(router, "show ip ospf interface brief")

# Gets the OSPF interface information from the router.

evidence += f"\n===== {router} =====\n"

# Adds the router name to the evidence.

evidence += "\nOSPF NEIGHBORS\n"

# Adds a heading for the neighbor information.

evidence += neighbors

# Adds the OSPF neighbor information to the evidence.

evidence += "\n\nOSPF INTERFACES\n"

# Adds a heading for the interface information.

evidence += interfaces

# Adds the OSPF interface information to the evidence.

return evidence

# Sends all collected information back to the program.

def get_ai_recommendation(evidence):

# Sends the network evidence to Qwen and asks for a diagnosis.

prompt = f"""

You are NetPilot, an AI network operations assistant.

Analyze the Cisco OSPF evidence below.

Identify the specific router and interface that requires correction.

Return ONLY these five lines:

ROUTER: <router>

INTERFACE: <interface>

CURRENT_AREA: <number>

EXPECTED_AREA: <number>

REASON: <short explanation>

Do not include configuration commands.

NETWORK EVIDENCE:

{evidence}

"""

# Creates the instructions that will be sent to Qwen.

# The {evidence} section contains the actual Cisco information.

print()

# Adds an empty line.

print("===== SENDING EVIDENCE TO AI =====")

# Shows that NetPilot is contacting the AI.

recommendation = ask_ollama(prompt)

# Sends the prompt and network evidence to Qwen.

return recommendation

# Returns the AI recommendation to NetPilot.

def parse_recommendation(recommendation):

# Reads Qwen's response and extracts the important information.

router = None

# Creates an empty variable for the router name.

interface = None

# Creates an empty variable for the interface.

current_area = None

# Creates an empty variable for the current OSPF area.

expected_area = None

# Creates an empty variable for the expected OSPF area.

reason = None

# Creates an empty variable for the explanation.

for line in recommendation.splitlines():

# Goes through the AI response one line at a time.

if line.startswith("ROUTER:"):

# Checks whether the current line contains the router.

router = line.replace("ROUTER:", "").strip()

# Removes "ROUTER:" and saves the router name.

elif line.startswith("INTERFACE:"):

# Checks whether the current line contains the interface.

interface = line.replace("INTERFACE:", "").strip()

# Removes "INTERFACE:" and saves the interface name.

elif line.startswith("CURRENT_AREA:"):

# Checks whether the current line contains the current area.

current_area = line.replace("CURRENT_AREA:", "").strip()

# Removes the label and saves the current area.

elif line.startswith("EXPECTED_AREA:"):

# Checks whether the current line contains the expected area.

expected_area = line.replace("EXPECTED_AREA:", "").strip()

# Removes the label and saves the expected area.

elif line.startswith("REASON:"):

# Checks whether the current line contains the reason.

reason = line.replace("REASON:", "").strip()

# Removes the label and saves the reason.

return router, interface, current_area, expected_area, reason

# Sends all five pieces of information back to the program.

def approve_change(router, interface, current_area, expected_area, reason):

# Displays the proposed change and asks the engineer for approval.

print()

# Adds an empty line.

print("===== PROPOSED REMEDIATION =====")

# Displays a heading for the proposed change.

print(f"Router: {router}")

# Displays the router that will be changed.

print(f"Interface: {interface}")

# Displays the interface that will be changed.

print(f"Current OSPF Area: {current_area}")

# Displays the current OSPF area.

print(f"Expected OSPF Area: {expected_area}")

# Displays the expected OSPF area.

print(f"Reason: {reason}")

# Displays why the change is recommended.

print()

# Adds an empty line.

print("Configuration that will be applied:")

# Displays a heading for the configuration.

print(f"interface {interface}")

# Displays the Cisco interface command.

print(f" ip ospf 1 area {expected_area}")

# Displays the OSPF configuration command.

print()

# Adds an empty line.

answer = input("Apply this change? (yes/no): ")

# Stops the program and asks the engineer for permission.

return answer.lower() == "yes"

# Returns True if the engineer typed yes.

# Returns False for any other answer.

def apply_change(router, interface, expected_area):

# Applies the approved configuration to the router.

commands = [

f"interface {interface}",

f"ip ospf 1 area {expected_area}",

]

# Creates the Cisco commands using the AI-selected interface

# and expected OSPF area.

print()

# Adds an empty line.

print("===== APPLYING CHANGE =====")

# Displays a message that configuration is being applied.

output = send_config(router, commands)

# Sends the configuration commands to the selected router.

print(output)

# Displays the router's response.

def verify_change(router):

# Checks the router after the configuration change.

print()

# Adds an empty line.

print("===== VERIFYING CHANGE =====")

# Displays a verification heading.

neighbors = run_command(router, "show ip ospf neighbor")

# Gets the current OSPF neighbors.

interfaces = run_command(router, "show ip ospf interface brief")

# Gets the current OSPF interface information.

print()

# Adds an empty line.

print("OSPF NEIGHBORS:")

# Displays a heading for neighbor information.

print(neighbors)

# Displays the current OSPF neighbors.

print()

# Adds an empty line.

print("OSPF INTERFACES:")

# Displays a heading for interface information.

print(interfaces)

# Displays the current OSPF interfaces.

if __name__ == "__main__":

# Checks whether this file is being run directly.

evidence = collect_ospf_evidence()

# Collects the current network information.

recommendation = get_ai_recommendation(evidence)

# Sends the network evidence to Qwen.

print()

# Adds an empty line.

print("===== NETPILOT AI DIAGNOSIS =====")

# Displays a heading for the AI diagnosis.

print(recommendation)

# Displays Qwen's recommendation.

router, interface, current_area, expected_area, reason = parse_recommendation(

recommendation

)

# Extracts the router, interface, areas, and reason from Qwen's response.

if not router or not interface or not expected_area:

# Checks whether Qwen provided enough information.

print()

# Adds an empty line.

print("AI recommendation could not be understood.")

# Reports that the AI response could not be used.

print("No configuration was changed.")

# Confirms that nothing was modified.

exit()

# Stops the program.

approved = approve_change(

router,

interface,

current_area,

expected_area,

reason,

)

# Shows the proposed change and asks the engineer for approval.

if approved:

# Runs this section if the engineer approved the change.

apply_change(router, interface, expected_area)

# Sends the approved configuration to the router.

verify_change(router)

# Checks the router after the change.

print()

# Adds an empty line.

print("===== REMEDIATION COMPLETE =====")

# Reports that the remediation process has finished.

else:

# Runs this section if the engineer rejected the change.

print()

# Adds an empty line.

print("Change rejected.")

# Reports that the engineer rejected the change.

print("No configuration was changed.")

# Confirms that NetPilot did not modify the router.

12. How the AI Prompt Was Improved

A major improvement was made to the AI instructions.

Instead of asking Qwen simply:

What is wrong?

NetPilot specifically asks it to identify:

ROUTER

INTERFACE

CURRENT_AREA

EXPECTED_AREA

REASON

This is important because earlier testing showed that Qwen could recognize that R2 and R3 were involved but could sometimes be unclear about which router actually needed to be changed.

The improved design forces the AI to identify the specific device and interface.

For this lab, the expected answer is:

ROUTER: R3

INTERFACE: GigabitEthernet2

CURRENT_AREA: 1

EXPECTED_AREA: 0

REASON: OSPF area mismatch

13. Why the AI Does Not Generate the Cisco Command

An important design decision was made.

Qwen identifies:

Router: R3

Interface: GigabitEthernet2

Expected Area: 0

NetPilot then creates the Cisco commands itself:

commands = [

f"interface {interface}",

f"ip ospf 1 area {expected_area}",

]

This produces:

interface GigabitEthernet2

ip ospf 1 area 0

This is safer than allowing the AI to return arbitrary commands and immediately executing them.

The architecture is therefore:

AI

↓

Identifies problem

↓

NetPilot interprets recommendation

↓

NetPilot creates controlled command

↓

Engineer approves

↓

NetPilot executes

14. AI Remediation Test

The complete program is started with:

python -m app.ai_remediation

NetPilot first collects information from:

R1

R2

R3

It collects:

show ip ospf neighbor

and:

show ip ospf interface brief

from each router.

15. Evidence Sent to AI

The evidence provided to Qwen includes information similar to:

===== R2 =====

OSPF INTERFACES

Gi2   1   0   10.0.23.1/30   1   DR   0/0

and:

===== R3 =====

OSPF INTERFACES

Gi2   1   1   10.0.23.2/30   1   DR   0/0

NetPilot therefore provides AI with enough information to recognize:

R2 Gi2 → Area 0

R3 Gi2 → Area 1

16. AI Diagnosis

Qwen correctly identified the problem:

R2 and R3 have an OSPF area mismatch.

More importantly, it identified the device requiring correction:

R3

and the specific interface:

GigabitEthernet2

The expected configuration was:

ip ospf 1 area 0

17. Human Approval

NetPilot then presents:

===== PROPOSED REMEDIATION =====

Router: R3

Interface: GigabitEthernet2

Current OSPF Area: 1

Expected OSPF Area: 0

Reason: OSPF area mismatch

Configuration that will be applied:

interface GigabitEthernet2

ip ospf 1 area 0

Apply this change? (yes/no):

At this point, the engineer is in control.

Typing:

yes

allows NetPilot to proceed.

Typing:

no

prevents the configuration from being changed.

18. Applying the Configuration

When approval is given, NetPilot creates:

interface GigabitEthernet2

ip ospf 1 area 0

and sends those commands to R3 using:

send_config(router, commands)

The router is therefore changed without the engineer having to manually SSH into R3.

19. Verification

After the change, NetPilot automatically runs:

show ip ospf neighbor

and:

show ip ospf interface brief

This allows NetPilot to determine whether the remediation actually worked.

The desired result is:

R2 Gi2 → Area 0

R3 Gi2 → Area 0

and:

R2 ↔ R3

FULL

The important concept is that NetPilot doesn't simply say:

"I applied the change."

It also checks the network afterward.

20. NetPilot v0.5 Architecture

The new architecture is:

Cisco Network

│

▼

NetPilot Evidence

Collection

│

▼

Qwen AI

│

▼

AI Diagnosis

│

▼

Router / Interface

Identification

│

▼

Recommended Change

│

▼

Human Approval

/           \

NO             YES

│               │

▼               ▼

STOP         Apply Change

│

▼

Cisco R3

│

▼

Verify

│

▼

Remediation Result

21. What Was Accomplished in v0.5

The major accomplishment of this lab was moving NetPilot from:

AI troubleshooting

toward:

AI-assisted network automation

NetPilot can now potentially:

- Collect network information.

- Analyze network conditions using local AI.

- Identify a specific device.

- Identify a specific interface.

- Identify the expected state.

- Recommend a remediation.

- Ask for human approval.

- Make the approved change.

- Verify the network afterward.

This is a significant architectural milestone.

22. Security Principle

NetPilot should not become a system where an AI model has unrestricted control of the network.

The preferred architecture is:

AI = Analyst

NetPilot = Control/Automation Engine

Engineer = Approval Authority

This separation will become increasingly important as the project becomes more advanced.

23. Future Development — Git

Git will eventually be introduced as the configuration and change-management system.

The future workflow could become:

AI Recommendation

↓

Proposed Configuration

↓

Git Change

↓

Validation

↓

Human Approval

↓

Deployment

Git will allow NetPilot to maintain a history of configuration changes.

24. Future Development — CI/CD

CI/CD will eventually be added to validate changes before they reach production devices.

The future architecture could look like:

Configuration Change

↓

Git

↓

CI/CD Pipeline

↓

Automated Validation

↓

Security Checks

↓

Configuration Checks

↓

Approval

↓

Deployment

This will allow NetPilot to evolve toward network DevOps / NetDevOps rather than simply being a collection of Python scripts.

25. Future Development — NetBox Source of Truth

NetBox will eventually become the Source of Truth for the network.

Instead of relying exclusively on information discovered from routers, NetPilot will eventually compare:

NetBox

↓

Intended Network State

against:

Cisco Routers

↓

Actual Network State

NetPilot can then identify:

Intent

↓

Actual

↓

Difference

↓

AI Analysis

↓

Remediation

This will significantly improve the reliability of the platform.

26. Future Development — Docker

Docker will eventually be introduced to package NetPilot.

The long-term architecture could contain separate components such as:

NetPilot

├── API

├── AI Engine

├── Network Collector

├── Remediation Engine

├── Database

├── NetBox

└── Web Interface

Docker will make it easier to deploy the platform consistently.

27. Future Business/Product Direction

NetPilot is being developed with the long-term goal of becoming a side business, not simply a personal networking lab.

The technical development should therefore eventually support a real product.

Potential product capabilities include:

AI Network Troubleshooting

+

Configuration Drift Detection

+

Network Intent

+

Source of Truth

+

Human-Approved Remediation

+

Configuration Versioning

+

CI/CD

+

Audit Logging

A potential product workflow could eventually be:

Customer Network

↓

NetPilot

↓

Discover

↓

Compare Against Intent

↓

Detect Problem

↓

AI Diagnosis

↓

Recommend Fix

↓

Customer Approval

↓

Automated Remediation

↓

Verification

↓

Audit Report

This creates a much stronger potential business proposition than simply selling an "AI chatbot for Cisco."

28. Future Business Development

As NetPilot becomes more mature, development will also include:

Branding

Create a professional NetPilot identity and visual design.

Website

Eventually create a website explaining:

- What NetPilot does.

- Who it is for.

- Problems it solves.

- How AI is used.

- Security model.

- Product demonstrations.

Marketing

Potential content can include:

- LinkedIn technical posts.

- Network automation demonstrations.

- AI troubleshooting videos.

- Cisco lab demonstrations.

- Configuration drift examples.

- Before/after remediation demonstrations.

Customer Discovery

Talk with network engineers, network administrators, MSPs, and small IT teams to determine which problems are worth solving commercially.

Product Packaging

Potential future versions could include:

NetPilot Community

NetPilot Professional

NetPilot Enterprise

The exact packaging and pricing should be determined later based on customer discovery rather than guessed now.

29. Current NetPilot Development Path

The project has now progressed through several concepts:

v0.1

Basic AI network troubleshooting

↓

v0.2

Network intent / improved AI diagnosis

↓

v0.3

Golden configuration

↓

v0.4

Configuration drift detection

↓

v0.5

AI-assisted remediation

↓

Future

Git

↓

CI/CD

↓

NetBox Source of Truth

↓

Docker

↓

Web UI / API

↓

Production Product

30. Key Lesson From v0.5

The biggest lesson from this lab is that AI should not simply be given access to the network and told to fix things.

Instead, NetPilot should create a controlled process:

COLLECT

↓

ANALYZE

↓

IDENTIFY

↓

RECOMMEND

↓

APPROVE

↓

EXECUTE

↓

VERIFY

↓

AUDIT

This is the foundation for turning NetPilot from an experimental AI networking project into a potentially useful AI-assisted network operations platform.

NetPilot v0.5 therefore represents the first major step toward closed-loop network automation with human approval.